3GPP TSG-RAN WG4 Meeting # 109	R4-2318587

# Chicago, US, November 13 – 17, 2023

Agenda item:	8.36.2

Source:	Apple

Title:	UE demodulation performance requirements for NR dynamic spectrum sharing

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108bis requirements for enhanced DSS were discussed and way forward [1] were agreed.  In this contribution we present our views on demodulation requirements for eDSS.

# 2. Discussion

In RAN4#108bis the demodulation requirements for eDSS were discussed and the follow agreements were reached [1]:

Whether to define PDCCH demodulation requirements for Rel-18 eDSS feature Agreement: - Define PDCCH demodulation requirements for eDSS Evaluation scenario Agreement: - 2-symbol CORESET - Basic assumptions - Rate matching pattern - Option 1: Single non-overlapping CRS rate matching patterns for PDCCH demodulation. - Other options not precluded - 4 CRS ports for LTE - Symbol #0: LTE PDCCH/PCFICH - Symbol #1: NR PDCCH overlapping with LTE CRS - Symbol #2: NR PDCCH gNB assumption for PDCCH symbols overlapped with LTE CRS Agreement: - PDCCH data and DMRS REs overlapped with LTE CRS are punctured. UE receiver assumption (e.g., channel estimation) Agreement: - PDCCH channel estimation is assumed to use only the clean PDCCH symbol. Whether to define the PDSCH demodulation requirements with two overlapping CRS rate matching patterns Agreement: - Not define PDSCH demodulation requirements.

The simulation assumptions for PDCCH demod requirements with eDSS are to be discussed this meeting. Since we have agreed on 4-port LTE CRS, we would need to configure 4TX. All requirements with PDCCH are limited to 1Tx and 2TX so far. Before finalizing the configuration for defining requirements, we propose to evaluate the performance with different configurations.

- Evaluate performance with different configurations before finalizing the parameters for defining PDCCH demod requirements with eDSS.

We propose the following common parameters:

- DCI format: 1_0

- CCE to REG mapping: Non-interleaved

- REG bundle size: 6 PRB

- CORESET duration: 2 symbols (symbol 1, 2)

- Precoder: Random precoder with precoder cycling per REG bundle

- LTE CRS: 4 ports

- TX assumption for overlapping REs: Puncture PDCCH and PDCCH DMRS REs overlapping with LTE CRS

- LTE PDCCH, PDSCH:  No transmission

- Slots with LTE PBCH/PSS/SSS: No PDCCH transmission

- Channel estimation: PDCCH CE only on clean symbols

- Common parameters for PDCCH performance evaluation in eDSS:
DCI format: 1_0

CCE to REG mapping: Non-interleaved

REG bundle size: 6 PRB

CORESET duration: 2 symbols (symbol 1, 2)

Precoder: Random precoder with precoder cycling per REG bundle

LTE CRS: 4 ports
TX assumption for overlapping REs: Puncture PDCCH and DMRS REs overlapping with LTE CRS
LTE PDCCH, PDSCH:  No transmission

Slots with LTE PBCH/PSS/SSS: No PDCCH transmission

Channel estimation: PDCCH CE only on clean symbols

The detailed parameters for evaluating with different AL and propagation condition are proposed as follows:

Table 1: Test Parameters for PDCCH performance evaluation in eDSS

| SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low |

- Test parameters for PDCCH performance evaluation in eDSS:

| SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low |

# 3. Conclusion

In this paper, we provide our views on the scope for UE demodulation requirements for DSS enhancements. Our observations and proposals are captured below:

- Evaluate performance with different configurations before finalizing the parameters for defining PDCCH demod requirements with eDSS.

- Common parameters for PDCCH performance evaluation in eDSS:
DCI format: 1_0

CCE to REG mapping: Non-interleaved

REG bundle size: 6 PRB

CORESET duration: 2 symbols (symbol 1, 2)

Precoder: Random precoder with precoder cycling per REG bundle

LTE CRS: 4 ports
TX assumption for overlapping REs: Puncture PDCCH and DMRS REs overlapping with LTE CRS
LTE PDCCH, PDSCH:  No transmission

Slots with LTE PBCH/PSS/SSS: No PDCCH transmission

Channel estimation: PDCCH CE only on clean symbols

- Test parameters for PDCCH performance evaluation in eDSS:

| SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low |

# Reference

- R4-2317000, “WF on NR_DSS_enh”, Ericsson.