3GPP TSG-RAN WG4 Meeting #109						                    R4-2318592

Chicago, USA, November 13 – 17, 2023

Agenda item:	8.9.2.3

Source:	Apple

Title:	Discussion on case 2 requirements of R18 gap enhancement

Document for:	Discussion

# Introduction

Case 2 requirements of R18 gap enhancement was discussed during the previous RAN4 meetings. The last agreements can be found in [1]. In this contribution, we continue discussing the open issue.

# Discussion

## UE capabilities for Concurrent gaps with NCSG and NCSG + NCSG

Issue 3-2-1: [Case 2] Whether to consider an additional capability for NCSG + NCSG in an FR

Moderator: Agreement from previous meeting that a new capability for Concurrent gaps with NCSG in an FR will be defined as a new UE capability in Rel-18. This issue is whether to consider an additional UE capability for NCSG + NCSG.

- Way forward

  - Option 1:

    - No, without additional UE capability.

  - Option 2:

    - Yes, with additional UE capability

  - Option 3:

    - Whether to consider an additional capability for NCSG + NCSG in an FR can be discussed once RRM requirements for NCSG + NCSG become stable

Conservatively, we prefer option 2. There is extra effort at UE side to support NCSG + concurrent gaps so RAN4 agreed a new capability to support this combination. When it comes to NCSG + NCSG, there might be some additional aspects that need to be considered. For example, there is one open issue regarding UE automatically change measurement on SCell from outside gap to NCSG when the SCell becomes deactivated. If there are two NCSG, then UE needs to know which NCSG to use for this measurement.

Proposal 1: consider an additional capability for NCSG + NCSG in an FR.

## Other Rel-17 rules to be revisited

Sub-topic description: This sub-topic covers NCSG upon SCell activation issue in concurrent gap with NCSG.

Open issues and candidate options before meeting:

- Agreement from previous meetings:

< Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? - New in Rel-18 - When Type-2 MG and NCSG are both configured, some serving cell MOs may associated to the NCSG and some are not. - Question 1: What is the expected UE behaviour (assume SMTC partially overlapped with NCSG) - Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Question 2: Whether additional UE capability indication is needed

Issue 3-3-1: [Case 2] Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? (Clarify Rel-17 understanding)

- Way forward

  - Option 1:

    - No,

      - The deactivated SCell MO(s) are measured within NCSG if the UE reports ‘intraFreq-needForNCSG’ on the band(s) where the deactivated SCell MO(s) located in.

      - Otherwise, the deactivated SCell MO(s) are measured outside of MG with interruption.

  - Option 2:

    - The Rel-17 UE behavior is that when the SMTC of deactivated SCell is fully or partially overlapped with NCSG, the deactivated SCell is measured via NCSG regardless the UE capability report of intraFreq-needForNCSG.

  - Option 3:

    - In Rel-17, if the UE supports NCSG (ncsg-MeasGapNR-Patterns-r17 or ncsg-MeasGapPatterns-r17) and the network configures an NCSG supported by the UE:

      - A deactivated SCell is measured within NCSG if at least some of the SCell’s SMTC overlaps with NCSG occasions; otherwise, the deactivated SCell is measured outside of NCSG.

      - An activated SCell is measured within NCSG only if either the SCell’s SSB is outside the active DL BWP or the SCell’s SMTC fully overlaps with NCSG, and the UE signaled that the SCell can be measured with NCSG via needForGapNCSG-InfoNR; otherwise, the activated SCell is measured outside of NCSG, if possible.

  - Option 4:

    - RAN4 shall wait for the outcome of the issue on whether all deactivated Scell  will be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG from Rel-17 maintenance on Rel-17 MGE

Issue 3-3-2: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18)

- Way forward

  - Option 1:

    - Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule)

      - Deactivated Scell MO associated with NCSG is measured within NCSG

      - Deactivated Scell MO not associated with NCSG is measured outside NCSG

  - Option 2:

    - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association.

  - Option 3:

    - The Rel-18 UE behavior (assume SMTC partially overlapped with NCSG) can follow the gap association, i.e., deactivated SCell MO associated with NCSG is measured within NCSG and the MO not associated with NCSG is measured outside NCSG.

Issue 3-3-3: [Case 2] Whether a new UE capability is needed for the support of NCSG for deactivated SCell?

- Way forward

  - Option 1:

    - No

  - Option 2:

    - A new indication shall be introduced enable support of NCSG for deactivated SCell only.

These several issues have been widely discussed in previous RAN4 meetings. One fundamental issue is that companies have different understanding of R17 UE behaviour, i.e., will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG?

In our understanding, RAN4 cannot simply assume “all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG”. At least it hasn’t been explicitly discussed in R17 when RAN4 introduced NCSG in NR.

Support of NCSG feature (nr-NeedForGapNCSG-Reporting-r17) is reported per UE. However, feedback of NeedForGapNCSG-InfoNR is per band per band combination. For instance, if UE ONLY supports NCSG in FR1, then UE shall report nr-NeedForGapNCSG-Reporting-r17, and report ‘gap’ for all FR2 bands. However, we cannot assume deactivated SCell in FR2 can be measured via NCSG, since this UE doesn’t even support NCSG in FR2.

Observation 1: NCSG is supposed to be a band dependent feature. However, nr-NeedForGapNCSG-Reporting-r17 is reported per UE.

Observation 2: NCSG can still be supported per band by indicating ‘ncsg’ in NeedForGapNCSG-InfoNR-r17 only for bands on which UE is willing to support NCSG.

Observation 3: “all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG” would make NCSG a per-UE feature.

Proposal 2: RAN4 shall NOT assume “all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG”.

Regarding whether a new UE capability is needed for the support of NCSG for deactivated SCell, we see some benefit of it. According to R17 NCSG reporting, UE indicates two entries respectively for intra-frequency and inter-frequency measurement:

One thing we would like to highlight is that the reporting is relatively static, based on the configured CA band combination and regardless of whether the SCell is activated or not. Therefore, if some MO cannot be measured within NCSG, e.g. SCell is activated and the MO is outside active BWP, UE shall not claim support of NCSG on this band or serving cell. For example:

Figure 1 NCSG for SCell measurement

Assuming there is a vacant RF chain when the SCell is deactivated, then UE can use it for measurement on the deactivated SCell. However, when the SCell is activated and BWP2 is the active BWP, UE may not support the measurement with NCSG since the MO is outside active BWP. Note that the feedback of NCSG is per RRC level while SCell activation/deactivation is done via MAC, which is more dynamic than RRC. Conservatively, the UE shall not claim support of NCSG on this SCell. Unless the UE is always operating with larger BW, e.g. as CBW, the UE may still support NCSG for this SCell. Therefore, some optimization is needed.

Observation 4: according to R17 NCSG reporting design, UE shall NOT indicate support of NCSG for the band unless UE can always perform measurement within NCSG regardless of which BWP is the active BWP. In other word, once UE indicates support of NCSG for the band, the MO within this band can always be measured within NCSG. This jeopardizes the benefit of using NCSG for measurement on SCC.

Proposal 3: a new indication shall be introduced enable support of NCSG for deactivated SCell only.

# Conclusion

In this contribution, we provide discussion on case 2 requirements. After discussion, the following conclusions are provided:

Proposal 1: consider an additional capability for NCSG + NCSG in an FR.

Observation 1: NCSG is supposed to be a band dependent feature. However, nr-NeedForGapNCSG-Reporting-r17 is reported per UE.

Observation 2: NCSG can still be supported per band by indicating ‘ncsg’ in NeedForGapNCSG-InfoNR-r17 only for bands on which UE is willing to support NCSG.

Observation 3: “all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG” would make NCSG a per-UE feature.

Proposal 2: RAN4 shall NOT assume “all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG”.

Observation 4: according to R17 NCSG reporting design, UE shall NOT indicate support of NCSG for the band unless UE can always perform measurement within NCSG regardless of which BWP is the active BWP. In other word, once UE indicates support of NCSG for the band, the MO within this band can always be measured within NCSG. This jeopardizes the benefit of using NCSG for measurement on SCC.

Proposal 3: a new indication shall be introduced enable support of NCSG for deactivated SCell only.

# References

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2314323, WF on NR MG enhancements (Part 1), MediaTek inc.

- R4-2310175, WF on NR MG enhancements, MediaTek inc.

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.

- R4-2220359, WF on NR_MG_enh2 Part 1, MediaTek inc.

- R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.

- R4-2202604, Further reply LS on R17 NR MG enhancements – Concurrent MG, MediaTek inc.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation