3GPP TSG-RAN WG4 Meeting #109						                    R4-2318596

Chicago, USA, November 13 – 17, 2023

Agenda item:	8.9.4

Source:	Apple

Title:	Discussion on performance requirement for case 1 and 2

Document for:	Discussion

# Introduction

Performance part 1 of R18 gap enhancement was discussed during the previous RAN4 meetings. The last agreements can be found in [1]. In this contribution, we provide some discussion on some of the open issues.

# Discussion

## Sub-topic 4-1: Performance principles for Case 1 and Case 2

Sub-topic description: This sub-topic provides general principles for performance part.

Issue 4-1-1: Which general configuration shall be defined for the test cases?

- Way forward

  - Option 1:

    - Only inter-frequency measurement

    - Only non-DRX

    - without SSB time index detection

  - Option 2:

    - Only define test case in NR SA in both FR1 and FR2

    - Do not introduce the test for L1 impact

    - Do not introduce test cases for intra-freq measurement without gap

    - Define a minimum set of test cases for SSB-based measurement

    - Only define test case under non-DRX

    - Define test case without SBI reporting.

    - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

    - Do not define test cases with simultaneously FR1 and FR2 gaps configured.

    - Test cases are limited to single serving carrier

    - Only use mandatory gap patterns to define test cases

    - Focus on only fully non-overlp and partially partial overlap in the test case design

    - Verify gap dropping behaviour without introducing additional test cases

In general, RAN4 only needs to define test cases to verify new functionalities/requirements when joint configuration of Pre-MG, NCSG and concurrent gaps are configured, since UE anyway needs to pass existing test cases for Pre-MG, NCSG and concurrent gaps which were introduced in R17. With this consideration, RAN4 can skip some of the scenarios when discussing testing scope. After reviewing proposals under option 1 and 2, we suggest the following:

Proposal 1: general configuration for test cases for case 1 and 2:

- Only inter-frequency measurement

- Only non-DRX

- without SSB time index detection

- Only define test case in NR SA in both FR1 and FR2

- Do not introduce the test for L1 impact

- Do not introduce test cases for intra-freq measurement without gap

- Define a minimum set of test cases for SSB-based measurement

- On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

- Do not define test cases with simultaneously FR1 and FR2 gaps configured.

- Test cases are limited to single serving carrier

- Only use mandatory gap patterns to define test cases

- Focus on only partially partial overlap in the test case design

# Conclusion

In this contribution, we provide discussion on performance part 1 requirements. After discussion, the following conclusions are provided:

Proposal 1: general configuration for test cases for case 1 and 2:

- Only inter-frequency measurement

- Only non-DRX

- without SSB time index detection

- Only define test case in NR SA in both FR1 and FR2

- Do not introduce the test for L1 impact

- Do not introduce test cases for intra-freq measurement without gap

- Define a minimum set of test cases for SSB-based measurement

- On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

- Do not define test cases with simultaneously FR1 and FR2 gaps configured.

- Test cases are limited to single serving carrier

- Only use mandatory gap patterns to define test cases

- Focus on only partially partial overlap in the test case design

# References

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2314323, WF on NR MG enhancements (Part 1), MediaTek inc.

- R4-2310175, WF on NR MG enhancements, MediaTek inc.

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.

- R4-2220359, WF on NR_MG_enh2 Part 1, MediaTek inc.

- R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.

- R4-2202604, Further reply LS on R17 NR MG enhancements – Concurrent MG, MediaTek inc.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation