3GPP TSG-RAN WG4 Meeting #109						                    R4-2318613

Chicago, USA, November 13 – 17, 2023

Agenda item:	8.25.2.4

Source:	Apple

Title:	Discussion on network B requirements of R18 MUSIM

Document for:	Discussion

# Introduction

Network B requirements of R18 MUSIM have been widely discussed in previous RAN4 meetings. However, there are still some open issues. In this contribution, we provide our view on these issues.

# Discussion

Issue 4-1-1: Network B requirements conditions

- Proposals

  - P1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only (Ericsson vivo CMCC China Telecom Huawei oppo Apple)

    - P1-1: The inactive state requirement should be the same as NW B’s Idle state (Ericsson vivo China Telecom Nokia)

  - P2: RAN4 only one set of requirements for NW-B requirements when UE is allocated with MUSIM gaps.  Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer. Continue discussion other conditions during or once NW B requirements are agreed. (Nokia)

  - P3: Postpone the discussion of additional conditions for defining Network B requirements until there is agreement on the framework for defining the requirements (issue 4-1-2). (Qualcomm)

For cell reselection in NW B, longer MGRP can be expected and it is not expected to be dropped due to collision with NW A gap. Otherwise all the MUSIM gaps would be dropped. Therefore, it makes sense to us that RAN4 can assume no MUSIM gap is dropped in NW B requirements. With this assumption, in general, we support P1, P1-1 and P2 with some modification.

Proposal 1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only.

Proposal 2: The inactive state requirement should be the same as NW B’s Idle state.

Proposal 3: Add the condition “MUSIM gaps will not be dropped due to collision with other MUSIM gaps” when defining NW B requirements.

Issue 4-1-2: Network B requirements framework

- Proposals

  - P1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (xiaomi Qualcomm oppo)

  - P2: The NW-B’s requirement should decouple with MUSIM gaps(mgrp) requested by UE; RAN4 to introduce a relaxed NW-B’s IDLE mode requirement as follow (Ericsson CMCC China Telecom)

    - P2-1: N = 4 (Qualcomm Huawei Apple)

  - P3: The measurement/cell reselection requirements in IDLE/inactive mode for NW B will reuse the existing idle/inactive requirements as the baseline. In requirements for a particular DRX cycle, it is replaced by N*DRX, where N is FFS and 1≤N≤16. N could use the format of P1 or P2 (vivo)

  - P4: Replace DRX cycle by max(DRX cycle, MGRP_max) and introduce a scaling factor of 2. (MTK)

  - P5: The UE measurement requirements for measurement/cell reselection requirements in IDLE/inactive mode for NW B, need to be based on a reasonable balance of the allocated MUSIM gap. The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements. current DRX cycle is replaced with Max(DRX cycle, Min(MUSIM gap MGRP)). Remove the M1 scaling factor. (Nokia)

Recommendations:

Online session outcome:

- New Option 1: the network B requirement is related to MGRP

  - Issues for further discussion: the impact given that network B does not know the MGRP configured by network A

- New Option 2: the network B requirements is not related to MGRP, and with a fixed scaling factor N based on the DRX cycle. Further discuss the N

  - Option 2A: N = 4, and other values are not precluded.

  - Issue for further discussion: UE behavior or network impact for the case that NxDRX cycle is shorter than MGRP.

We continue support option 1. We don’t think NW B needs to know MGRP configured by network A, since it is only for UE in idle mode from NW B point of view.

Proposal 4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline.

Proposal 5: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps.

Issue 4-1-3: Requirement when MGRP = 5.12s

- Proposals

  - P1: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s could be defined. (CMCC vivo Huawei Apple Nokia)

    - P1-1: The new requirements for 5.12s could reuse corresponding requirements (number of DRX cycles) when DRX = 2.56s. (vivo Huawei Apple)

  - P2: RAN4 not need to discuss the requirement for MGRP=5.12s if the NW-B’s requirement is only related to NW-B’s DRX. (Ericsson)

  - P3: NW B requirements does not apply when MUSIM gap’s MGRP=5.12s (MTK)

  - P4: For MUSIM gap with 5.12s MGPR, whether to define related requirements depending on the discussion of 4-1-2 (vivo Qualcomm China Telecom)

Recommendations: Continue discussion

For completion, we support P1 to introduce requirement for 5.12s MGRP. P1-1 can also be considered since this is the last meeting for core part design.

Proposal 6: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined.

Proposal 7: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s.

Issue 4-1-6: Network B requirements test case

- Proposals

  - P1: Do not define test cases to verify any new requirements in network B. (Qualcomm vivo Huawei MTK )

  - P2: Do not exclude defining test cases to verify any new requirements in network B (Nokia)

  - P3: RAN4 to postpone the test case discussion to performance part (Ericsson)

Recommendations: Continue discussion

We support P1. Given that UE anyway needs to pass same test case in network A and meet the cell reselection requirements, it seems no need to further test this in network B.

Proposal 8: Do not define test cases to verify any new requirements in network B.

# Conclusion

In this contribution, we provide discussion on network B requirements of R18 MUSIM. After discussion, the following conclusions are provided:

Proposal 1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only.

Proposal 2: The inactive state requirement should be the same as NW B’s Idle state.

Proposal 3: Add the condition “MUSIM gaps will not be dropped due to collision with other MUSIM gaps” when defining NW B requirements.

Proposal 4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline.

Proposal 5: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps.

Proposal 6: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined.

Proposal 7: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s.

Proposal 8: Do not define test cases to verify any new requirements in network B.

# References

- R4-2317425, WF on R18 MUSIM, vivo

- R4-2314364, WF on R18 MUSIM, vivo

- R4-2310165, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo