3GPP TSG-RAN WG4 Meeting #109	R4-2318638

# Chicago, US, 13th – 17th November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on output power dynamics and Tx signal qulity for ATG UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce output power dynamics and Tx signal qulity for ATG UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ATG UE output power dynamics and Tx signal qulity are introduce in new section 6.3J and 6.4J respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ATG UE requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.3J; 6.4J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3J	Output power dynamics for ATG

### 6.3J.1	Minimum output power for ATG

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3J.1-1for ATG UE with omni-directional antenna and in Table 6.3J.1-2 for ATG UE with phased array antenna.

Table 6.3J.1-1: Minimum output power for ATG UE with omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 6.3J.1-2: Minimum output power for ATG UE with phased array antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -19 | -19+10log10 (BWChannel /20) | -19+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

### 6.3J.2	Transmit OFF power for ATG

The transmit OFF power requirements as specified in clause 6.3.2 are applicable for ATG UE.

### 6.3J.3	Transmit ON/OFF time mask for ATG

The transmit ON/OFF time mask requirements as specified in clause 6.3.3 are applicable for ATG UE.

### 6.3J.4	Power control for ATG

The power control requirements specified in clause 6.3.4 are applicable for ATG UE.

## 6.4J	Transmit signal quality for ATG

### 6.4J.1	Frequency error for ATG

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

NOTE 1:	the ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

NOTE 2:	UE shall rely on the ATG BS location broadcasted by the [ATG specific SIB] in 38.331.

### 6.4J.2	Transmit modulation quality for ATG

The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the requirements for Pi/2 BPSK modulation in 6.4.2.4.1 and the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.