3GPP TSG-RAN WG4 Meeting # 109	R4-2318663

Chicago, America, November 13 – November 17, 2023

Agenda item:	8.3.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#108bis meeting, a WF related to 8Rx UE requirements was agreed [1]. In this contribution, we provide simulation results based on the WF.

- Discussion

8Rx single carrier requirements

In Table 1, we provide our simulation results for rank 2, 4 and 8 for Rx UE receiver.

Table 1. SNR point of 70% maximum throughput for rank 2, 4, 8

| Rank | MCS | Propagation Condition | Channel Correlation | Alignment results TDD SNR (dB) | Impairment results TDD SNR (dB) | Alignment results FDD SNR (dB) | Impairment results FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 12.5 | 15.0 | 12.6 | 15.1 |
| 4 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 12.5 | 15.0 | 12.7 | 15.2 |
| 8 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 21.2 | 23.7 | 21.4 | 23.9 |

According to simulation results collection in the last meeting [2], the spans for alignment results are quite large:

- TDD: “2.8dB for rank2” and “3.7dB for rank 4”

- FDD: “3.1dB for rank2”, “4.3dB for rank 4” and “3.1dB for rank 8”

Therefore, we propose to increase the current margin 0.8dB used in the impairment results.

Observation 1: The spans for the alignment results are quite large:

- TDD: “2.8dB for rank2” and “3.7dB for rank 4”

- FDD: “3.1dB for rank2”, “4.3dB for rank 4” and “3.1dB for rank 8”

Proposal 1: To deal the issue of large span in the simulation results alignment. We propose to increase the current margin 0.8dB used in the impairment results.

8Rx CA requirements

RAN4 made agreements for the rank of CA requirements in last meeting.

Issue 2-2-4: Rank - Agreement - Rank 8 for 8Rx+8Rx, Rank 2 for 2Rx+8Rx and 4Rx+8Rx - Rank 8 + Rank 8 for 8Rx+8Rx for UE supporting Rank 8 for 8Rx - Rank 2 + Rank 2 for 4Rx+8Rx - Rank 2 + Rank 2 for 2Rx+8Rx

We provide our simulation results for 8Rx with rank 2 and rank 8 in following tables.

Table 2. SNR point of 70% maximum throughput for each CBW in TDD, Rank2

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 12.28 |
|  |  | 10 |  |  |  |  | 12.32 |
|  |  | 15 |  |  |  |  | 12.29 |
|  |  | 20 |  |  |  |  | 12.33 |
|  |  | 25 |  |  |  |  | 12.26 |
|  |  | 30 |  |  |  |  | 12.38 |
|  |  | 40 |  |  |  |  | 12.45 |
|  |  | 50 |  |  |  |  | 12.46 |
|  |  | 60 |  |  |  |  | 12.37 |
|  |  | 80 |  |  |  |  | 12.42 |
|  |  | 90 |  |  |  |  | 12.40 |
|  |  | 100 |  |  |  |  | 12.52 |

Table 3. SNR point of 70% maximum throughput for each CBW in FDD, Rank 2

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 2 | 19 (Table 1) | TDLC300-100-10 | 2x8 ULA Medium B | 12.23 |
|  |  | 10 |  |  |  |  | 12.55 |
|  |  | 15 |  |  |  |  | 12.41 |
|  |  | 20 |  |  |  |  | 12.38 |
|  |  | 25 |  |  |  |  | 12.35 |
|  |  | 30 |  |  |  |  | 12.33 |
|  |  | 35 |  |  |  |  | 12.40 |
|  |  | 40 |  |  |  |  | 12.39 |
|  |  | 45 |  |  |  |  | 12.34 |
|  |  | 50 |  |  |  |  | 12.42 |

Table 2. SNR point of 70% maximum throughput for each CBW in TDD, Rank8

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 low | 21.62 |
|  |  | 10 |  |  |  |  | 21.16 |
|  |  | 15 |  |  |  |  | 21.26 |
|  |  | 20 |  |  |  |  | 21.04 |
|  |  | 25 |  |  |  |  | 21.27 |
|  |  | 30 |  |  |  |  | 21.43 |
|  |  | 40 |  |  |  |  | 21.22 |
|  |  | 50 |  |  |  |  | 21.24 |
|  |  | 60 |  |  |  |  | 21.48 |
|  |  | 80 |  |  |  |  | 21.61 |
|  |  | 90 |  |  |  |  | 21.73 |
|  |  | 100 |  |  |  |  | 21.99 |

Table 3. SNR point of 70% maximum throughput for each CBW in FDD, Rank 8

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 low | 21.39 |
|  |  | 10 |  |  |  |  | 21.38 |
|  |  | 15 |  |  |  |  | 21.45 |
|  |  | 20 |  |  |  |  | 21.39 |
|  |  | 25 |  |  |  |  | 21.56 |
|  |  | 30 |  |  |  |  | 21.43 |
|  |  | 35 |  |  |  |  | 21.62 |
|  |  | 40 |  |  |  |  | 21.82 |
|  |  | 45 |  |  |  |  | 21.79 |
|  |  | 50 |  |  |  |  | 21.65 |

- Conclusion

In this contribution, we provide our views and simulation results for 8Rx PDSCH single carrier and CA requirements.

Observation 1: The spans for the alignment results are quite large, which are “2.8dB for rank2” and “3.7dB for rank 4”.

Proposal 1: Remove the smallest and largest values in alignment results to make the span smaller than 2.5dB.

- Reference

- R4-2316914, “WF for 8Rx and 4Tx performance requirements”, Huawei, HiSilicon

- R4-2315859, “Simulation results collection for 8 Rx UE demodulation requirements”, Ericsson