3GPP TSG-RAN WG4 Meeting # 109	R4-2318664

Chicago, America, November 13 – November 17, 2023

Agenda item:	8.36.2

Source:	MediaTek Inc.

Title:	Discussion on PDCCH requirements for DSS enhancements

Document for:	Discussion

- Introduction

In RAN4#108bis meeting, a WF related to DSS enhancement requirements was agreed [1]. In this contribution, we present our views on the open issues.

- Discussion

There is a remaining issue about rate matting pattern for evaluation after RAN4#108bis.

Evaluation scenario Agreement: - 2-symbol CORESET - Basic assumptions - Rate matching pattern - Option 1: Single non-overlapping CRS rate matching patterns for PDCCH demodulation. - Other options not precluded - 4 CRS ports for LTE - Symbol #0: LTE PDCCH/PCFICH - Symbol #1: NR PDCCH overlapping with LTE CRS - Symbol #2: NR PDCCH

Three features for DSS enhancement were introduced in RAN1 [2], which are 52-1, 52-1a, and 52-1b. As shown in Table1, for both 52-1a and 52-1b, there are more than one prerequisite feature group. Therefore, we propose not to introduce requirements for UE supporting FG 52-1a or 52-1b.

Table 1. UE features for NR PDCCH reception in symbols with LTE CRS REs

|  | 52-1 | 52-1a | 52-1b |
| --- | --- | --- | --- |
| Feature group | Reception of NR PDCCH candidates overlapping with LTE CRS REs | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot |
| prerequisite feature groups | 5-28 | 52-1, at least one of {14-1, 52-2} | 52-1, 22-12 |

Proposal 1: Consider single non-overlapping CRS rate matching patterns for PDCCH demodulation requirements.

Regarding the simulation assumption for evaluation, we propose the following parameter assumptions and test cases. It is noted that RAN4 should consider the bandwidth of NR CORESET is within the bandwidth of LTE.

Proposal 2: Consider Table 1 for simulation assumptions and Table 2 for PDCCH requirements

Table 1. Simulation assumptions

| Parameters | Values |
| --- | --- |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLC300-100 |
| Antenna configuration | 2x2 and 2x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4 and 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

Table 2. Minimum requirements for PDCCH reception in symbols with LTE CRS REs

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 48 | 2 | 4 | TDLA30-10 | 2x2 Low | 1 | TBD |
| 10 | 48 | 2 | 8 | TDLC300-100 | 2x4 Low | 1 | TBD |

- Conclusion

In this contribution, we provide views about the PDCCH performance requirements for DSS enhancement WI. The proposals are summarized as below.

Proposal 1: Consider single non-overlapping CRS rate matching patterns for PDCCH demodulation requirements.

Proposal 2: Consider Table 1 for simulation assumptions and Table 2 for PDCCH requirements

- Reference

- R4-2317000, “WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements”, Ericsson

- R1-2308521, “Updated RAN1 UE features list for Rel-18 NR after RAN1#114”, AT&T, NTT DOCOMO, INC.