3GPP TSG-RAN WG4 Meeting # 109	R4-2318665

Chicago, America, November 13 – November 17, 2023

Agenda item:	8.14.6.1

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for less than 5MHz BW

Document for:	Discussion

- Introduction

In RAN4#108bis meeting, a WF related to UE demodulation requirements for less than 5MHz BW was agreed [1]. In this contribution, we provide our views for the remaining issues.

- Discussion

2.1	PDSCH and SDR requirements

Issue 1-1-1: Introduction of new requirements Way forward FFS, whether new PDSCH demodulation performance requirements needs to be introduced: - FFS, whether to introduce a set of requirements for PDSCH for UE supporting only less than 5 MHz: - Consider minimum requirements for RedCap (5.2.2.1.17) as a reference - Further evaluate a need for optional features and excluding advance features which is out of scope of the RAN1 WI - FFS, a need for SDR requirements - FFS, a need to introduce the limited set or no new requirements for PDSCH for UE supporting multiple CBWs (i.e., less than and more than 5 MHz) Issue 1-1-3: PDSCH parameters for further evaluation (non-HST scenario) Agreement: - Consider the following parameters for further evaluation of PDSCH performance in less than 5MHz: - Duplex: FDD, CBW: 3MHz, SCS: 15kHz, Number of PRBs: 15 - Parameters from RedCap PDSCH minimum requirements (5.2.2.1.17) can be used for reference - Use 2 RX as a starting point, FFS 4 RX Issue 1-1-4: PDSCH requirements in HST scenario Way forward - FFS, whether to introduce PDSCH requirements for less than 5MHz CBW in HST conditions with the speed up to 500 km/h: - Option 1: Use HST DPS propagation conditions (B3.3) and test 5.2.2.1.10 as a reference - Option 2: Use TS 38.101-4 Table 5.2.2.1.1-3 Test 1-6 [single-tap propagation conditions B.3.1] as a reference - 64QAM, 0.43, HST-972, Rank 1, 1Tx, 2Rx/4Rx. - Other options are not precluded - FFS, the paramters (Dmin, Ds, f_d) of high speed propogation conditions

For PDSCH requirements, considering that there might have UEs only supporting less than 5MHz. RAN4 can define limited new PDCSH requirements by considering minimum requirements for RedCap (5.2.2.1.17) as a starting point.

Proposal 1: For non-HST scenario, define PDSCH demodulation requirements with 15PRBs for UE supporting less than 5MHz. RAN4 can reuse minimum performance for RedCap in 5.2.2.1.17 and only consider 2Rx as below.

Table 1: Minimum performance for Rank 1 in less than 5MHz

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 3 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | Test 1-1 in Table 5.2.2.1.17-3 |
| 1-2 | 3 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | Test 1-2 in Table 5.2.2.1.17-3 |
| 1-3 | 3 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | Test 1-3 in Table 5.2.2.1.17-3 |

Table 2: Minimum performance for Rank 2 in less than 5MHz

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 2-1 | 3 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | Test 2-1 in Table 5.2.2.1.17-4 |

According to TS38.101-4, the purpose of the SDR test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The SDR requirements is not related to the number of PRBs. Considering that there might have UEs only supporting less than 5MHz, RAN4 can update TS 38.101-4 Tables 5.5A-1 and 5.5A-4 for less than 5MHz.

Proposal 2: Update TS 38.101-4 Tables 5.5A-1 and 5.5A-4 to support less than 5MHz:

- Add “2/AL2 for 15 kHz / 3 MHz” in the field of “Number of PDCCH candidates and aggregation levels” in Tables 5.5A-1

- Add number of PRBs for 15 kHz / 3 MHz as 15 in Tables 5.5A-4

2.2	PDCCH requirements

Issue 1-2-1: Introduction of new requirements Way forward: FFS, whether new PDCCH demodulation performance requirements needs to be introduced in less than 5 MHz CBW: - Consider only 15KHz SCS, FDD, 2Rx, FFS for 4Rx - Option1: Don’t define PDCCH requirements for channel bandwidth less than 5MHz - Non punctured PDCCH: - Option 2: Define (non-punctured) PDCCH demodulation requirements with 15PRBs, 3MHz CBW, for UE supporting NR_FR1_lessthan_5MHz_BW: - 12 PRB CORESET - 2 symbols, AL2, DCI 1_0 (35 bits for 15 PRBs) - Option 3: Define (non-punctured) PDCCH demodulation requirements with 24 PRB PDCCH, for 5 MHz CBW with AL 4. - Punctured PDCCH: - FFS, how to address the testability issue, i.e., no ACK/NACK for the SIB1 scheduled by PDCCH in CORESET#0. - Option 4: If the testability issue can be resolved than consider PDCCH demodulation requirements with punctured PDCCH for CORESET#0 (FFS, testing punctured PDCCH mapped to USS in CORESET#0) - with 3 symbols AL 8 PDCCH with 3 MHz CBW, interleaved - Other options are not precluded Issue 1-2-2: Propagation conditions Way forward: - Consider the following propagation conditions: - TDLA30-10 for 1Tx, - TDLC300-100 for 2Tx antenna, - HST propagation conditions for 500km/h speed - FFS, Single-tap or HST DPS propagation conditions - Other options are not preluded.

As we mentioned in the last meeting, there is some testability issue for PDCCH in CORESET#0. Therefore, we propose not to introduce PDCCH requirements for the punctured case. However, considering that there might have UEs only supporting less than 5MHz, RAN4 can introduce limited new requirements for non-punctured case.

Proposal 3: For UE supporting less than 5MHz, do not define PDCCH demodulation requirements for punctured case.

Proposal 4: For UE supporting less than 5MHz, define (non-punctured) PDCCH demodulation requirements with 15PRBs, 3MHz CBW:

Table 3: Minimum performance for PDCCH in less than 5MH

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 12 | 2 | 2 | TBD (DCI 1_0, 35 bits) | TDLA30-10 | 1x2 Low | Test 1 in Table 5.3.2.1.1-1 |

2.3	PBCH requirements

Issue 1-3-3: PBCH parameters Agreement - Use the following parameters as a starting point Duplex; Bandwidth (MHz) / Subcarrier spacing (kHz); SSB/PBCH index; Propagation condition; Antenna configuration and correlation matrix; Reference value. Pm-bch (%); SNR (dB). FDD; 3 / 15; Unknown; TDLC300-100; 1 x 2 Low, [FFS, 1x4]; 1; TBD. Issue 1-3-4: Whether to consider HST conditions for PBCH Way forward The issue requires further discussion: - Option 1: Not to consider HST conditions for PBCH - Option 2: Further check the impact of HST conditions on PBCH performance. - Interested companies are encouraged to bring simulations results at the next meeting.

In the last meeting, RAN4 agreed to define PBCH requirements with unknow SSB index case. We provide our initial simulation results in Figure 1. The Pm-bch (without SSB index knowledge) := (1-Pssb) + Pssb * PBCH_BLER, where Pssb is the SSB index detection success rate and PBCH_BLER is the PBCH BLER with the know SSB index.

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low | 1 | -4.6 |

Figure 1. PBCH performance in case SS/PBCH block index is not known

As there are no conformance tests in RAN5, we think RAN4 does not need to investigate further about PBCH requirements. RAN4 can only define requirements for 2Rx case.

Proposal 5: Consider only 2Rx case for PBCH requirements in less 5MHz.

2.4	CSI requirements

Issue 1-4-1: A need for new requirements Way forward: Further discussion is needed whether to introduce new CSI reporting requirements for channel bandwidth less than 5MHz: - Option1: Define CSI requirements for CBW=3MHz and SCS=15kHz: PMI, CQI and RI. - Option 2: Do not CSI requirements. FFS, requirements and applicability rules if UE supports only less then 5MHz CBW

Considering that there might have UEs only supporting less than 5MHz, RNA4 can define limited new test cases for CSI feedback by reusing the test framework in current specification.

Proposal 6: Define CQI definition test under static channel condition with 15PRBs for UE supporting less than 5MHz

- Use Table 6.2.2.1.1.1-1 as a starting point

- 15PRBs, CQI Table 2, 2T2R, Rank 2, 2 SNR test points

- Reuse the same metric as Rel-15 CQI definition test in static condition

Proposal 7: Define CQI reporting test under fading channel condition with 15PRBs for UE supporting less than 5MHz

- Use Table 6.2.2.1.2.1-1 as a starting point

- 15PRBs, CQI Table 2, 2T2R, Rank 1, TDLA30-5, 2 SNR test points

- Reuse the same metric as Rel-15 CQI reporting test in fading condition

Proposal 8: Define PMI reporting test with 15PRBs for UE supporting less than 5MHz

- Use Table 6.3.2.1.1-1 as a starting point

- 15PRB, Single PMI, TypeI-SinglePanel, MCS13 (16QAM, 0.48), Rank 1, 4 CSI-RS ports, High XP 4 x 2, TDLA30-5

- Reuse the same metric as Rel-15 4Tx PMI reporting requirements

Proposal 9: Define RI reporting test with 15PRBs for UE supporting less than 5MHz

- Use Table 6.4.2.1-1 Test 1 as a starting point

- 15PRBs, CQI Table 2, SNR=0dB, ULA Low 2x2, TDLA30-5, fixed RI=2 vs. follow RI

- Reuse the same metric as Rel-15 RI reporting requirements

2.5	General aspects

Issue 1-5-1: Common HST propagation conditions and parameters Way forward: Further discuss which HST propagation conditions (500 km/h) and parameters to use: - Option 1: Single-tap propagation conditions ( based on B.3.1) - Ds = [300] m, Dmin = [2] m, f_d =[972] Hz - Option 2: HST DPS propogation conditions (based on B.3.3.) - Ds = [700] m, Dmin = [150] m, f_d = [TBA] Issue 1-5-2: Applicability of requirements Way forward: FFS, how to introduce applicability rules for UE Demodulation and CSI reporting requirements in less than 5 MHz channel BW: - Option 1: Create requirements’ applicability table for UE supporting NR_FR1_lessthan_5MHz_BW. - Option 2: The new requirements are only applicable to the specific bands for this WI, instead of being band-agnostic. - Other options are not precluded.

RAN4 uses the applicability rules to indicate what UE features/capabilities should UE have for passing the requirements. The UE features/capabilities are related to RAN1 defined feature groups. RAN1 had introduced feature groups for UEs to operate in less than 5MHz bandwidth [2]. Therefore, we prefer to follow RAN4 legacy rule by using requirements applicability table.

Proposal 10: Follow RAN4 legacy rule by using requirements applicability table for UE supporting less than 5MHz.

Table 4: Requirements applicability for optional UE features (PDSCH and PDCCH)

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| TBD (UE capability related to less than 5MHz) | FR1 FDD | PDSCH | TBD |  |
|  | FR1 FDD | PDCCH | TBD |  |

Table 5: Requirements applicability for optional UE features (CSI)

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| TBD (UE capability related to less than 5MHz) | FR1 FDD | CQI | TBD |  |
|  | FR1 FDD | PMI | TBD |  |
|  | FR1 FDD | RI | TBD |  |

However, it is possible that UEs can support both less and larger than 5MHz. Therefore, we propose to introduce rules to skip the above proposed PDSCH, PDCCH and CSI related requirements as there are no new design for PDSCH, PDCCH (non-punctured) and reference signal for less than 5MHz.

- For PDSCH, we choose the minimum performance for Rank 1 in Table 5.2.2.1.1-3 and minimum performance for Rank 2 in Table 5.2.2.1.1-4, where the test conditions are similar to that in the proposed Table 1 and 2.

- For PDCCH, we choose minimum performance for PDCCH with 15 kHz SCS in Table 5.3.2.1.1-1, where the test conditions are similar to that in the proposed Table 3.

- For CSI, we choose tests in Table 6.2.2.1.1.1-1, 6.2.2.1.2.1-1, 6.3.2.1.1-1 and Test 1 in Table 6.4.2.1-1 as these tests are with similar test conditions as we proposed for CSI requirements in section 2.4.

Proposal 11: Introduce requirements’ applicability tables for PDSCH, PDCCH (non-punctured), CQI, RI and PMI requirements to indicate which test case can be skipped when UE supports both less and larger than 5MHz.

Table 6: Applicability of requirements for less than 5MHz (PDSCH)

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDSCH | Test 1-1 in Table 5.2.2.1.1-3 | FR1 FDD | PDSCH | Test 1-1 in Table 1 |  |
| FR1 FDD | PDSCH | Test 1-2 in Table 5.2.2.1.1-3 | FR1 FDD | PDSCH | Test 1-2 in Table 1 |  |
| FR1 FDD | PDSCH | Test 1-3 in Table 5.2.2.1.1-3 | FR1 FDD | PDSCH | Test 1-3 in Table 1 |  |
| FR1 FDD | PDSCH | Test 2-1 in Table 5.2.2.1.1-4 | FR1 FDD | PDSCH | Test 2-1 in Table 2 |  |

Table 6: Applicability of requirements for less than 5MHz (PDCCH)

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDCCH | Test 1 in Table 5.3.2.1.1-1 | FR1 FDD | PDCCH | Test 1 in Table 3 |  |

Table 6: Applicability of requirements for less than 5MHz (CQI, PMI, RI)

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | CQI | Tests in Table 6.2.2.1.1.1-1 | FR1 FDD | CQI | Test in Proposal x |  |
| FR1 FDD | CQI | Tests in Table 6.2.2.1.2.1-1 | FR1 FDD | CQI | Test in Proposal x |  |
| FR1 FDD | PMI | Tests in Table 6.3.2.1.1-1 | FR1 FDD | PMI | Test in Proposal x |  |
| FR1 FDD | RI | Test 1 in Table 6.4.2.1-1 | FR1 FDD | RI | Test in Proposal x |  |

- Conclusion

In this contribution, we provide our views for the remaining issues of the WI NR_FR1_lessthan_5MHz_BW. Proposals are summarized as follows.

Proposal 1: For non-HST scenario, define PDSCH demodulation requirements with 15PRBs for UE supporting less than 5MHz. RAN4 can reuse minimum performance for RedCap in 5.2.2.1.17 and only consider 2Rx as below.

Table 1: Minimum performance for Rank 1 in less than 5MHz

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 3 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | Test 1-1 in Table 5.2.2.1.17-3 |
| 1-2 | 3 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | Test 1-2 in Table 5.2.2.1.17-3 |
| 1-3 | 3 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | Test 1-3 in Table 5.2.2.1.17-3 |

Table 2: Minimum performance for Rank 2 in less than 5MHz

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 2-1 | 3 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | Test 2-1 in Table 5.2.2.1.17-4 |

Proposal 2: Update TS 38.101-4 Tables 5.5A-1 and 5.5A-4 to support less than 5MHz:

- Add “2/AL2 for 15 kHz / 3 MHz” in the field of “Number of PDCCH candidates and aggregation levels” in Tables 5.5A-1

- Add number of PRBs for 15 kHz / 3 MHz as 15 in Tables 5.5A-4

Proposal 3: For UE supporting less than 5MHz, do not define PDCCH demodulation requirements for punctured case.

Proposal 4: For UE supporting less than 5MHz, define (non-punctured) PDCCH demodulation requirements with 15PRBs, 3MHz CBW:

Table 3: Minimum performance for PDCCH in less than 5MH

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 12 | 2 | 2 | TBD (DCI 1_0, 35 bits) | TDLA30-10 | 1x2 Low | Test 1 in Table 5.3.2.1.1-1 |

Proposal 5: Consider only 2Rx case for PBCH requirements in less 5MHz.

Proposal 6: Define CQI definition test under static channel condition with 15PRBs for UE supporting less than 5MHz

- Use Table 6.2.2.1.1.1-1 as a starting point

- 15PRBs, CQI Table 2, 2T2R, Rank 2, 2 SNR test points

- Reuse the same metric as Rel-15 CQI definition test in static condition

Proposal 7: Define CQI reporting test under fading channel condition with 15PRBs for UE supporting less than 5MHz

- Use Table 6.2.2.1.2.1-1 as a starting point

- 15PRBs, CQI Table 2, 2T2R, Rank 1, TDLA30-5, 2 SNR test points

- Reuse the same metric as Rel-15 CQI reporting test in fading condition

Proposal 8: Define PMI reporting test with 15PRBs for UE supporting less than 5MHz

- Use Table 6.3.2.1.1-1 as a starting point

- 15PRB, Single PMI, TypeI-SinglePanel, MCS13 (16QAM, 0.48), Rank 1, 4 CSI-RS ports, High XP 4 x 2, TDLA30-5

- Reuse the same metric as Rel-15 4Tx PMI reporting requirements

Proposal 9: Define RI reporting test with 15PRBs for UE supporting less than 5MHz

- Use Table 6.4.2.1-1 Test 1 as a starting point

- 15PRBs, CQI Table 2, SNR=0dB, ULA Low 2x2, TDLA30-5, fixed RI=2 vs. follow RI

- Reuse the same metric as Rel-15 RI reporting requirements

Proposal 10: Follow RAN4 legacy rule by using requirements applicability table for UE supporting less than 5MHz.

Table 4: Requirements applicability for optional UE features (PDSCH and PDCCH)

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| TBD (UE capability related to less than 5MHz) | FR1 FDD | PDSCH | TBD |  |
|  | FR1 FDD | PDCCH | TBD |  |

Table 5: Requirements applicability for optional UE features (CSI)

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| TBD (UE capability related to less than 5MHz) | FR1 FDD | CQI | TBD |  |
|  | FR1 FDD | PMI | TBD |  |
|  | FR1 FDD | RI | TBD |  |

Proposal 11: Introduce requirements’ applicability tables for PDSCH, PDCCH (non-punctured), CQI, RI and PMI requirements to indicate which test case can be skipped when UE supports both less and larger than 5MHz.

Table 6: Applicability of requirements for less than 5MHz (PDSCH)

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDSCH | Test 1-1 in Table 5.2.2.1.1-3 | FR1 FDD | PDSCH | Test 1-1 in Table 1 |  |
| FR1 FDD | PDSCH | Test 1-2 in Table 5.2.2.1.1-3 | FR1 FDD | PDSCH | Test 1-2 in Table 1 |  |
| FR1 FDD | PDSCH | Test 1-3 in Table 5.2.2.1.1-3 | FR1 FDD | PDSCH | Test 1-3 in Table 1 |  |
| FR1 FDD | PDSCH | Test 2-1 in Table 5.2.2.1.1-4 | FR1 FDD | PDSCH | Test 2-1 in Table 2 |  |

Table 6: Applicability of requirements for less than 5MHz (PDCCH)

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDCCH | Test 1 in Table 5.3.2.1.1-1 | FR1 FDD | PDCCH | Test 1 in Table 3 |  |

Table 6: Applicability of requirements for less than 5MHz (CQI, PMI, RI)

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | CQI | Tests in Table 6.2.2.1.1.1-1 | FR1 FDD | CQI | Test in Proposal x |  |
| FR1 FDD | CQI | Tests in Table 6.2.2.1.2.1-1 | FR1 FDD | CQI | Test in Proposal x |  |
| FR1 FDD | PMI | Tests in Table 6.3.2.1.1-1 | FR1 FDD | PMI | Test in Proposal x |  |
| FR1 FDD | RI | Test 1 in Table 6.4.2.1-1 | FR1 FDD | RI | Test in Proposal x |  |

- Reference

- R4-2316924, “WF on NR_FR1_lessthan_5MHz_BW_demod”, Nokia, Nokia Shanghai Bell

- R1-2310635, “Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis”, AT&T, NTT DOCOMO, INC.