3GPP TSG-RAN WG4 Meeting # 109	R4-2318666

Chicago, America, November 13 – November 17, 2023

Agenda item:	9.6.6

Source:	MediaTek Inc.

Title:	Workplan on demodulation requirements for IoT-NTN enhancement

Document for:	Approval

- Introduction

According to the latest WID for IoT NTN enhancements (IoT_NTN_enh) [1], the objectives are as follows:

### 4.1 Objective of SI or Core part WI or Testing part WI The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks) according to the following assumptions: - GEO and NGSO (LEO and MEO). - Earth fixed Tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS capabilities The detailed objectives are to specify enhanced NB-IoT NTN and eMTC NTN radio interfaces and E-UTRAN/NG-RAN as follows: 4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements. 4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4]. 4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3]. ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify UE RRM performance requirements to support the agreed mobility enhancements for NB-IoT/eMTC [RAN4] - Specify UE and Base Station demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC [RAN4] NOTE: The need for Performance requirements for improved GNSS operations will be established once the need for specification work has been decided.

In this contribution, we provide the workplan for IoT-NTN enhancement demodulation requirements.

- Work Plan

According to the WID, RAN4 is tasked to specify UE and Base Station demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC.

2.1	UE Demodulation performance part (36.102)

- November, 2023 (RAN4#109)

  - Approve the work plan.

  - Discuss UE demodulation requirements for IoT-NTN enhancement.

  - Discuss simulation assumptions.

- February 2024 (RAN4#110)

  - Continue discussion on UE demodulation requirements for IoT-NTN enhancement.

  - Agree on simulation assumptions.

  - Discuss possible work split for the CR work, if needed.

- April 2024 (RAN4#110bis)

  - Collect the simulation results.

  - Provide CR/Draft CR based on work split and discuss CRs/Draft CRs.

- May 2024 (RAN4#111)

  - Update simulation assumptions if necessary.

  - Finalize CRs and close the performance part.

2.1	SAN Demodulation performance part (36.108)

- November, 2023 (RAN4#109)

  - Approve the work plan.

  - Discuss SAN demodulation requirements for IoT-NTN enhancement.

  - Discuss simulation assumptions.

- February 2024 (RAN4#110)

  - Continue discussion on SAN demodulation requirements for IoT-NTN enhancement.

  - Agree on simulation assumptions.

  - Discuss possible work split for the CR work, if needed.

- April 2024 (RAN4#110bis)

  - Collect the simulation results.

  - Provide CR/Draft CR based on work split and discuss CRs/Draft CRs.

- May 2024 (RAN4#111)

  - Update simulation assumptions if necessary.

  - Finalize CRs and close the performance part.

- Conclusion

In this contribution, we provide work plans on UE and SAN demodulation requirements for IoT-NTN enhancement.

- Reference

- RP-231407, Revised WID on IoT NTN enhancements, MediaTek Inc., RANP #100 meeting, June 2023.