3GPP TSG-RAN WG4 Meeting #109                                                                        R4-2318679

# Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.11.4

Source:	Apple Inc.

Title:	PDSCH Demodulation Requirements for Type-2 UEs in Intra-band Non-contiguous Non-collocated NR Carrier Aggregation

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

The Intra-band Non-collocated EN-DC/NR-CA WI was approved during RAN#95 meeting. The focus of this WI is to study the feasibility of supporting non-collocated scenarios, including the minimum performance requirements for PDSCH. The work has been delimited to TDD in bands n77/n78. This is, band combinations DC_42_n77/n78 for EN-DC, and CA_n77(2A) and CA_n78(2A) for NR-CA. About minimum PDSCH performance requirements, they should be obtained based on the agreed conditions of applicable maximum received time difference (MRTD) requirement of 33us and the maximum power imbalance between component carriers of 25dB. In this paper, we follow previous RAN4#108bis agreements and submit our simulation results to start the process of aligning the requirement.

# 2. Discussion

RAN4 RF core part has discussed a number of types of UE for this scenario. Details are included in the table below:

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA, ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA, ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |

During the remainder of this WI, we will analyze the need for additional UE demodulation requirements for Type-2 UEs. Here, Type-2 UEs are capable for independent RF component to receive two independent component carriers. In this way, the test can fall back to a typical CA test, but now with the applicability of MRTD and Power Imbalance values that have been decided before: 33us and 25dB.

In addition to this, we want to reiterate our position regarding the practical aspects of implementing this feature. More specifically, that we should assume that there is sufficient frequency separation to avoid excessive burden on low-pass filter rejection after LO in the proposed RF reference architecture. In this regard, we have always assumed that there’s little to no cross-carrier interference between CCs.

Observation 1: The scenario in question is Intra-band non-collocated non-contiguous NR CA, which special emphasis in the non-contiguous aspect, as very early decided by the RAN4 RF session.

As of now, this feature is intended to work on Japanese markets, where according to their spectrum allocation, a minimum frequency separation between component carriers (CCs) is expected to be of at least 80MHz +BWanother/2, defined as the center of  BWanother relative to edge of BWwanted. See figure below.

Since this WI has been started to introduce the scenario for Japanese operators, we believe that it is sufficient to confirm to a frequency separation of 80MHz+BWanother/2 between 2 CCs as a side condition (R4-2314750). Once this feature gains traction and gets adopted in other markets, we can revisit this condition in subsequent releases (R19 and beyond).

Observation 2: In RAN4-108 the RF session has been agreed to introduce a minimum frequency separation of 80MHz+BWanother/2 between 2 CCs as a side condition for the test cases using the requirements specified in this WI. An LS to RAN5 has been submitted, with tdoc number R4-2314750.

Observation 3: In RAN4-108bis the Demod session has been agreed to support this minimum frequency separation of 80MHz+BWanother/2 between 2 CCs as a side condition. A new LS to RAN5 has been submitted, this time with tdoc number R4-2314750.

Proposal 1: RAN4 follow up on discussing how to capture this side condition for Intra-band Non-contiguous Non-collocated NR-CA scenarios such that the requirement will only be applicable to bands that are separated by 80MHz+BWanother/2.

Hence, with the previous proposal, we can conclude and confirm that there will be no significant adjacent channel interference, and time and frequency offsets can be corrected independently for each CC, regardless of the UE target cost and complexity.

Finally, during RAN4-108bis there has been steady progress in defining the technical parameters for the requirement that will be set.

Observation 4: During RAN4-108bis it was agreed that the weaker carrier should use Rank1, and the stronger carrier should use Rank2, given the existing power imbalance requirement of <=25dB.

Observation 5: Since the SNR distance between the lowest MCS and the higher MCS in the 64QAM is not enough to cover the 25dB power imbalance range, the weaker carrier will consider the 64QAM table while the stronger carrier uses the 256QAM table.

Using these considerations and the MCS levels agreed during RAN4-108bis, our results are as follows:

| Table | MCS | Rank | SNR |
| --- | --- | --- | --- |
| 64QAM | 4 | 1 | -2.5 |
| 64QAM | 5 | 1 | -1.5 |
| 256QAM | 22 | 2 | 22.7 |
| 256QAM | 23 | 2 | 23.6 |
| 256QAM | 24 | 2 | 24.5 |
| 256QAM | 25 | 2 | 25.6 |

Proposal 2: During RAN4#109, align results submitted by interested companies and decide the best MCS pair to guarantee a SNR difference of no more than 25dB, considering the confidence interval given by the computed spans.

# 4. Conclusion

Our observations and proposals about the testing criteria for Type-2 UEs in Intra-band Non-collocated NR-CA scenarios are summarized below:

Observation 1: The scenario in question is Intra-band non-collocated non-contiguous NR CA, which special emphasis in the non-contiguous aspect, as very early decided by the RAN4 RF session.

Observation 2: In RAN4-108 the RF session has been agreed to introduce a minimum frequency separation of 80MHz+BWanother/2 between 2 CCs as a side condition for the test cases using the requirements specified in this WI. An LS to RAN5 has been submitted, with tdoc number R4-2314750.

Observation 3: In RAN4-108bis the Demod session has been agreed to support this minimum frequency separation of 80MHz+BWanother/2 between 2 CCs as a side condition. A new LS to RAN5 has been submitted, this time with tdoc number R4-2314750.

Proposal 1: RAN4 follow up on discussing how to capture this side condition for Intra-band Non-contiguous Non-collocated NR-CA scenarios such that the requirement will only be applicable to bands that are separated by 80MHz+BWanother/2.

Observation 4: During RAN4-108bis it was agreed that the weaker carrier should use Rank1, and the stronger carrier should use Rank2, given the existing power imbalance requirement of <=25dB.

Observation 5: Since the SNR distance between the lowest MCS and the higher MCS in the 64QAM is not enough to cover the 25dB power imbalance range, the weaker carrier will consider the 64QAM table while the stronger carrier uses the 256QAM table.

Proposal 2: During RAN4#109, align results submitted by interested companies and decide the best MCS pair to guarantee a SNR difference of no more than 25dB, considering the confidence interval given by the computed spans.

# References

- R4-2316917	, “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson.

- R4-2311306, “On frequency separation for intra-band non-collocated CA”, Apple Inc.