# 3GPP TSG-RAN WG4 Meeting #109	R4-2318719

# Chicago, US, 13– 17 Nov, 2023

Source: 	Huawei, HiSilicon

Title:	TP for TR 38.880: DC_7A_n78A with 3Tx

Agenda Item:	7.29.2

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE bands B7 (1Tx) with NR band n78 (2Tx) for TR 38.880.

# Text Proposal

----- Start of TP -----

## 6.1	DC_7_n78

### 6.1.1	Operating bands for EN-DC

Table 6.1.1-1:  EN-DC band combination of band 7+n78

| EN-DC Band combination | Bands |
| --- | --- |
| DC_7A_n78A | 7, n78 |

### 6.1.2	Maximum output power for inter-band EN-DC

Table 6.1.2-1: UE Power Class for 3Tx inter-band EN-DC

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_7A_n78A5,6 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: (Void in this TR) NOTE 3: (Void in this TR) NOTE 4: (Void in this TR) NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC2 with UL MIMO in NR band NOTE 6: The UE supports PC3 in E-UTRA band, and supports PC3 with UL MIMO in NR band |  |  |  |  |

### 6.1.3	MSD scenario studies

Table 6.1.3-1 summarizes frequency ranges where harmonics mixing occur for DC_7_n78

Table 6.1.3-1: MSD scenarios for DC_7_n78 with total power class 2

|  | Aggressor Tx band | Victim Rx band | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | 2nd harmonic of n78 | 3rd harmonic of n7 Rx | No, but can reuse from DC_41_n77 |

### 6.1.4	REFSENS requirements

The MSD due to harmonic mixing can be taken from a similar band combination such DC_41A_n77A for PC2 with 2Tx

Table 6.1.4-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 7 | 10 | 15 | 12 (RBstart=0) | 5 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 7 | 20 | 15 | 50 (RBstart=0) | 20 | 19.4 | NOTE 4 | UL2/DL3 |
| NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

----- End of TP -----