# 3GPP TSG-RAN WG4 Meeting #109	R4-2318720

# Chicago, US, 13– 17 Nov, 2023

Source: 	Huawei, HiSilicon

Title:	TP for TR 38.880 DC_8A_n78A

Agenda Item:	7.29.2

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE bands B8 (1Tx) with NR band n78 (2Tx) for TR 38.880.

# Text Proposal

----- Start of TP -----

## 6.2	DC_8_n78

### 6.2.1	Operating bands for EN-DC

Table 6.2.1-1:  EN-DC band combination of band 8+n78

| EN-DC Band combination | Bands |
| --- | --- |
| DC_8A_n78A | 8, n78 |

### 6.2.2	Maximum output power for inter-band EN-DC

Table 6.2.2-1: UE Power Class for 3Tx inter-band EN-DC

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_8A_n78A5,6 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: (Void in this TR) NOTE 3: (Void in this TR) NOTE 4: (Void in this TR) NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC2 with UL MIMO in NR band NOTE 6: The UE supports PC3 in E-UTRA band, and supports PC3 with UL MIMO in NR band |  |  |  |  |

### 6.2.3	MSD scenario studies

Table 6.2.3-1 summarizes frequency ranges where harmonics mixing occur for DC_8_n78

Table 6.2.3-1: MSD scenarios for DC_8_n78 with total power class 2

|  | Aggressor Tx band | Victim Rx band | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| IMD | IMD4 (3rd order of band 8 UL - fundamental of n78 UL ) | 8 Rx | yes |

### 6.2.4	REFSENS requirements

Reuse the existing 2Tx MSD requirements.

----- End of TP -----