3GPP TSG-RAN WG4 Meeting#109	R4-2318732

Chicago, USA 13th Nov – 17th Nov, 2023

Source: 	Qualcomm Incorporated

Title: 	Simulation Results on PDSCH Performance Requirements for FR2 Multi-Rx

Agenda item:	8.7.4.2

Document for:	Discussion

# 1. Introduction

Based on the agreed simulation assumptions during RAN4#108-bis [1], in this paper we share our simulation results.

2. Simulation Results for PDSCH Requirements

2.1 mDCI with Fully overlapping resources

The common simulation assumptions for our results are provided below. The PDSCH resources are scheduled as agreed during the last meeting for both fully overlapping and non-overlapping cases.

| Transmission Scheme | Multi-DCI fully overlapping and non-overlapping |
| --- | --- |
| Correlation model | FR2 spatial correlation model [2] |
| SCS/BW | 120KHz, 100MHz |
| MCS/Layer combination | Fully overlapping: 2L+2L (MCS11, MCS13) 1L+1L (MCS17) Non-overlapping: 2L+2L (MCS13) |
| Channel model | TDLA-30ns-75Hz (statistically independent) |
| Precoding | Random across TRPs |
| DMRS port index | 7 and 8 for TRP1 and TRP2, respectively |
| No. of PDSCH DMRS symbols | 2 (1+1) |
| No. of PDSCH symbols | 10 |
| Cross-polarization coefficient | 0.0625 |
| Carrier frequency | 28GHz |

2.1 Multi-DCI with Fully-overlapping Resources

Below we share our results at -12dB and -15dB cross-talk power values for both separate and joint processing in the context of mDCI fully overlapping case at 70% peak throughput.

Table 1: PDSCH performance results with joint decoding for mDCI fully overlapping case

| Joint Processing (2L+2L) |  |  |  |
| --- | --- | --- | --- |
| -12dB |  | -15dB |  |
| MCS13 | MCS11 | MCS13 | MCS11 |
| 12.4 | 10.3 | 12.3 | 10.2 |

| Joint Processing (1L+1L) |  |
| --- | --- |
| -12dB | -15dB |
| MCS17 | MCS17 |
| 10 | 9.9 |

Table 2: PDSCH performance results with separate processing for mDCI fully overlapping case

| Separate Processing (2L+2L) |  |  |  |
| --- | --- | --- | --- |
| -12dB |  | -15dB |  |
| MCS13 | MCS11 | MCS13 | MCS11 |
| N/A* | 12.4 | 14.3 | 11.5 |

*Peak throughput can’t be reached for MCS13 at -12 dB cross-talk

| Separate Processing (1L+1L) |  |
| --- | --- |
| -12dB | -15dB |
| MCS17 | MCS17 |
| 11.8 | 10.6 |

2.2 Multi-DCI with Non-overlapping Resources

Below we share our results at -12dB and -15dB cross-talk power values for both separate and joint processing in the context of mDCI non-overlapping case at 70% peak throughput.

Table 1: PDSCH performance results with joint decoding for mDCI non-overlapping case

Joint Processing (2L+2L)

-12dB

MCS13

11.9

Table 2: PDSCH performance results with separate processing for mDCI non-overlapping case

| Separate Processing (2L+2L) |  |
| --- | --- |
| -12dB | -15dB |
| MCS13 | MCS13 |
| 12.4 | 12.4 |

3. Conclusions

In this paper, we share our simulation results for PDSCH demodulation requirements in the context of FR2 multi-Rx.

4. References

[1] R4-2316963, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #108-bis, Xiamen, CN, Oct 9 – 13, 2023.

[2] R4-2309824, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #107, Incheon, KR, May 22 – 26, 2023.