3GPP TSG-RAN WG4 Meeting#109	R4-2318734

Chicago, USA 13th Nov – 17th Nov, 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on the performance requirements for IoT NTN enhancements

Agenda item:	9.6.6

Document for:	Discussion

# 1. Introduction

As part of Rel-18, a work item was proposed to consider performance enhancements for IoT NTN. According to WID [1], RAN4 needs to consider UE demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC devices.

2. Performance Requirement with Disabled HARQ

During Rel-17, there were no performance requirements defined for NB-IoT/eMTC devices with disabled HARQ feedback. Due to large RTT delay, HARQ-ACK feedback is not efficient. To this end, disabled HARQ feedback allows network to avoid stop-and-wait process during HARQ feedback so that new data can be transmitted.

We note that PDSCH performance requirement with disabled HARQ feedback was defined for NR NTN in Rel-17. We think that a similar approach can be followed to define performance requirement with disabled HARQ feedback for NB-IoT/eMTC devices as well.

Observation 1: Disabled HARQ feedback was considered in Rel-17 NR NTN and performance requirements were defined.

Observation 2: A similar approach can be followed to define performance requirements for NB-IoT/eMTC devices.

Proposal 1: Introduce PDSCH performance requirements with disabled HARQ feedback for NB-IoT/eMTC devices.

Similar to NR NTN, a test setup can assume reTx disabled for all HARQ process to replicate disabled HARQ feedback.

Proposal 2: For disabled HARQ feedback, consider reTx disabled for all HARQ processes.

We note that for Rel-17 IoT NTN performance requirements were defined with NTN-TDLA (NLOS) and NTN-TDLC (LOS) channel models. In our view, same channel models can be assumed to introduce performance requirements with disabled HARQ feedback.

Proposal 3: Consider NTN-TDLA (NLOS) and NTN-TDLC (LOS) channel models used during Rel-17 IoT NTN performance requirements with disabled HARQ feedback.

As for the antenna configuration, similar to Rel-17 IoT NTN, we would like to propose 1x1 configuration.

Proposal 4: Consider 1x1 antenna configuration for simulation assumptions.

3. Conclusions

In this paper, we share our views on the performance requirements for IoT NTN enhancements. The following have been proposed.

Observation 1: Disabled HARQ feedback was considered in Rel-17 NR NTN and performance requirements were defined.

Observation 2: A similar approach can be followed to define performance requirements for NB-IoT/eMTC devices.

Proposal 1: Introduce PDSCH performance requirements with disabled HARQ feedback for NB-IoT/eMTC devices.

Proposal 2: For disabled HARQ feedback, consider reTx disabled for all HARQ processes.

Proposal 3: Consider NTN-TDLA (NLOS) and NTN-TDLC (LOS) channel models used during Rel-17 IoT NTN performance requirements with disabled HARQ feedback.

Proposal 4: Consider 1x1 antenna configuration for simulation assumptions.

4. References

[1] R4-2316921, “WID on IoT NTN enhancements,” 3GPP TSG RAN WG Meeting #99-e, Electronic, Dec 12 – 16, 2022.