3GPP TSG-RAN WG4 Meeting #109	R4-2318738

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0426 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 10/31/2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CSI report quantity (cri-RI-PMI-CQI) was removed from the specification for 1Tx CQI tests and currently no CSI report quantity is defined for these tests. Each vendor may choose to implement their own preference, which may cause different test results across vendors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add back cri-RI-PMI-CQI report quantity and set codebook configuration to “Not configured” for 1Tx CQI tests based on LS response from RAN1 (R1-2310649). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test setup will remain incomplete for 1Tx CQI tests |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.1.2, 6.2.2.2.1.2, 6.2.3.1.1.2, 6.2.3.2.1.2, 6.2A.3.1.1, 8.2A.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

Table 6.2.2.1.1.2-1: CQI reporting test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Duplex Mode |  |  | FDD |  |
| Subcarrier spacing |  | kHz | 15 |  |
| SNR |  | dB | 1 | 2 |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 1 |  |
|  | CDM Type |  | No CDM |  |
|  | Density (ρ) |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |
|  | CSI-IM RE pattern |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 |  |
| ReportConfigType |  |  | Periodic |  |
| CQI-table |  |  | Table 3 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| Codebook configuration |  |  | Not configured |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| Csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  | Not configured |  |
| Physical channel for CSI report |  |  | PUCCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |
| Measurement channel |  |  | As specified in Table A.4-4, TBS.4-1 |  |

--- End of change 1 ---

--- Start of change 2 ---

Table 6.2.2.2.1.2-1: CQI reporting test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD UL-DL pattern |  |  | FR1.30-1 |  |
| SNR |  | dB | 1 | 2 |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 1 |  |
|  | CDM Type |  | No CDM |  |
|  | Density (ρ) |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |
|  | CSI-IM RE pattern |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |  |
| ReportConfigType |  |  | Periodic |  |
| CQI-table |  |  | Table 3 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| Codebook configuration |  |  | Not configured |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| Csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  | Not configured |  |
| Physical channel for CSI report |  |  | PUCCH |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |
| Measurement channel |  |  | As specified in Table A.4-4, TBS.4-2 |  |

--- End of change 2---

--- Start of change 3 ---

Table 6.2.3.1.1.2-1: CQI reporting test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| SNR |  | dB | -2 | -1 |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 1 |  |
|  | CDM Type |  | No CDM |  |
|  | Density (ρ) |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |
|  | CSI-IM RE pattern |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 |  |
| ReportConfigType |  |  | Periodic |  |
| CQI-table |  |  | Table 3 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| Codebook configuration |  |  | Not configured |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  | Not configured |  |
| Physical channel for CSI report |  |  | PUCCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |
| Measurement channel |  |  | As specified in Table A.4-4, TBS.4-1 |  |

--- End of change 3---

--- Start of change 4 ---

Table 6.2.3.2.1.2-1: CQI reporting test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD UL-DL pattern |  |  | FR1.30-1 |  |
| SNR |  | dB | -2 | -1 |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 1 |  |
|  | CDM Type |  | No CDM |  |
|  | Density (ρ) |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |
|  | CSI-IM RE pattern |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |  |
| ReportConfigType |  |  | Periodic |  |
| CQI-table |  |  | Table 3 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| Codebook configuration |  |  | Not configured |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  | Not configured |  |
| Physical channel for CSI report |  |  | PUCCH |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |
| Measurement channel |  |  | As specified in Table A.4-4, TBS.4-2 |  |

--- End of change 4---

--- Start of change 5 ---

Table 6.2A.3.1.1-1: CA CQI reporting test parameters for FDD and TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Propagation channel |  |  | AWGN |
| Antenna configuration |  |  | 1×2 with static channel specified in Annex B.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 1 |
|  | CDM Type |  | No CDM |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 2, 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |
| ReportConfigType |  |  | Periodic |
| CQI-table |  |  | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| Codebook configuration |  |  | Not configured |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Csi-ReportingBand |  |  | 1111111 |
| aperiodicTriggeringOffset |  |  | Not configured |
| Physical channel for CSI report |  |  | PUCCH |
| Maximum number of HARQ transmission |  |  | 1 |
| Measurement channel |  |  | Derived as per section 5.1.3.2 of TS 38.214 [12] |

--- End of change 5---

--- Start of change 6 ---

Table 8.2A.3.1.1-1: CA CQI reporting test parameters for each CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Subcarrier spacing |  | kHz | 120 |
| Duplex Mode |  |  | TDD |
| TDD Slot Configuration |  |  | FR2.120-2 Annex A.1.3 |
| Propagation channel |  |  | AWGN |
| Antenna configuration |  |  | 1×2 with static channel specified in Annex B.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | 13 |
|  | CSI-RS periodicity and offset | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 1 |
|  | CDM Type |  | No CDM |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 8/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 8/1 |
| ReportConfigType |  |  | Periodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| Codebook configuration |  |  | Not configured |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 for 50MHz, 100MHz, 16 for 200MHz, 32 for 400MHz |
| csi-ReportingBand |  |  | 111111111 |
| CSI-Report periodicity and offset |  | slot | 8/3 |
| aperiodicTriggeringOffset |  |  | Not configured |
| Physical channel for CSI report |  |  | PUCCH |
| CQI/RI/PMI delay |  | ms | 8.375 |
| Maximum number of HARQ transmission |  |  | 1 |
| Measurement channel |  |  | Derived as per section 5.1.3.2 of TS 38.214 [12] |

--- End of change---