3GPP TSG-RAN4 Meeting #109	R4-2318749

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Agenda item:	7.22.2

Source:	Apple

Title:	A-MPR discussion for FDD HPUE NS_06

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The WID [1] envisages improvements on uplink coverage by introducing PC2 to several FDD bands. This contribution focuses on the network signalling flag NS_06. This flag is used by the bands n12, n13 and n14. This contribution provides analysis on NS_06 and makes proposals on A-MPR.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are used:

- Power Class 2

- Fixed Bias

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

###### 2.2 Simulation results

The NS_06 flag defines an alternative SEM mask for channel bandwidth up to 15MHz. While the NS_06 mask shows similarities to general SEM the additional requirements are tighter and therefore require more power back-off.

In the following the NS_06 requirements are used and channel bandwidth of 5MHz, 10MHz and 15MHz are simulated.

The simulations results are provided for QPSK with DFT-s-OFDM and CP-OFDM in Table 1. For CP-OFDM the results indicate that the MPR is sufficient. However, there is a tendency for some outer allocations bordering the channel edge to require slightly more power back-off. For DFT-s-OFDM there exist certain outer allocation which need 1.5dB which is 0.5dB increased to MPR. Including the results and observations from [2] it might be considered to define 0.5dB relaxation for outer allocations.

Table 1: NS_06 Power Back-off results for PC2

Proposal: Discuss whether 0.5dB relaxation for outer allocations can be justified for PC2.

# 4	Conclusions

This contribution provides A-MPR simulation results. The following proposals are made:

Proposal: Discuss whether 0.5dB relaxation for outer allocations can be justified for PC2.

# 4	References

- RP‑231064, “Revised WID: High power UE (power class 2) for NR FR1 FDD single band“, China Unicom, 3GPP TSG RAN meeting #100

- R4- 2316474, “PC2 A-MPR for NS_06”, Huawei, HiSilicon, WG4 Meeting#108bis