3GPP TSG-RAN4 Meeting #109	R4-2318753

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Agenda item:	4.1

Source:	Apple

Title:	[NR_newRAT-Core] MSD analysis for CA_n28A-n78A

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

Receiver harmonic mixing causes self-desensitization in the case of a higher LO harmonic falls into the UL of the second band from the band combination. In case of CA_n28A-n78A the fifth LO harmonic can fall into the UL from band n78. For such an event relaxation is required for an UE to achieve compliance with requirements.

# 2  Discussion

This combination shall provide an MSD analysis for harmonic mixing with the band combination CA_n28A-n78A. MSD analysis for EN-DC with n77 has been provided previously by [1]. A UE supporting bands n77 and n78 typically uses the same filter for both bands and therefore has same filter performance. The analysis can therefore be re-used. The Tables 1 and 2 provide the values of the individual components and signal power.

Table 1: Assumptions for signal path performance

| Components | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 15 | dB |
| Front-end loss | 4 | dB |
| Triplexer isolation | 25 | dB |
| B28 Rx filter total selectivity - main | 70 | dB |
| B28 Rx filter total selectivity - diversity | 40 | dB |
| PCB isolation | 70 | dB |
| Rx 5th order harmonic rejection | 40 | dB |

Table 2: Degradation analysis for harmonic mixing

| Performance of Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity | Unit |
| Tx power at antenna port | 23 | 8 | dBm |
| Tx power at triplexer output | -2 | -17 | dBm |
| Tx power at LNA input | -72 | -57 | dBm |
| Tx power at antenna port | 23 | 8 | dBm |
| Performance of PCB Coupling Path |  |  |  |
| Tx power at HB PA output | 27 | 27 | dBm |
| Tx power at LNA input | -43 | -43 | dBm |
| Combined |  |  |  |
| Tx power at LNA input | -43.0 | -42.8 | dBm |
| Tx power after Rx mixing (referred to antenna) | -79.0 | -78.8 | dBm |
| MSD after uncorrelated MRC (5MHz carrier) | 18.6 |  | dB |

Proposal 1: Define harmonic mixing MSD of 18.6dB for a 5MHz channel and agree the companion CRs.

# 4	Conclusions

This combination provides an MSD analysis for harmonic mixing with the band combination CA_n28A-n78A and makes the following proposal:

Proposal 1: Define harmonic mixing MSD of 18.6dB for a 5MHz channel and agree the companion CRs.

# 4	References

- R4-1713154, “MSD analysis for B28 in DC_28A_n77A due to 5th order harmonic mixing “, MediaTek Inc., 3GPP TSG RAN meeting #85