3GPP TSG-RAN WG4 Meeting #109	R4-2318767

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to include the FR2 multi-Rx correlation model in the specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Perf |  |  |  |  |  | Date: |  |  | 10/31/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To capture agreed correlation model for FR2 multi-Rx |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include agreed correlation model for FR2 multi-rx in 38.101-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correlation model for FR2 multi-Rx will remian undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

B.2.5 Correlation model for FR2 multi-Rx

The corelation model for FR2 mutli-Rx in a two TRP scenario with 1 antenna elecment with the same polarization in one direction at the gNB and UE side is defined as follows,

Rfr2_multi_rx=Eh16×1h16×1H=11+ρ∙diag1  ρ  ρ  1⊗RMIMO

=11+ρRMIMO00ρRMIMO00ρRMIMO00RMIMO,

where

- h16×1is a vector of re-arranged channel values for 2 antenna elements at each TRP and at each UE panel

- ⊗ denotes the Kronecker product and diag denotes the diagonal matrix

- RMIMO is a 4×4 correlation matrix for each link between two TRPs to two UE panels at the UE side generated with parameters α, β, and γ, as stated in B2.3.2

- ρ is the cross-talk power ratio

--- End of change ---