3GPP TSG-RAN WG4 Meeting #109	R4-2318769

Chicago, USA, Nov. 2023

Title		: Intra-band CA MPR for FR2 UL 256QAM

Source	: Qualcomm Incorporated

Agenda item		: 8.6.2

Document for	: Approval

# Introduction

Single CC MPRs have been agreed [1], but for completion of the feature, intra-band CA MPR must also be specified. We propose to derive intra-CA MPR values from observing the same patterns established for 64QAM.

# Discussion

## Overview

The proposal is reproduced from [2]. Intra-band CA MPR values can be tentatively established by similarity arguments extended from existing MPR values, or from a thorough evaluation campaign. For the former method, barring some anomalies, there is a pattern: both, the contiguous and NC CA MPRs are derived from the 400M outer single CC CP-OFDM values with a further pattern across different CABWs. This pattern can be retained for 256QAM. Example below is shown for PC1, but similar values and a pattern exist for PC2/3/4/5/6.

| 400M outer, single CC MPR (dB) | DFT-s | CP-OFDM |
| --- | --- | --- |
| 64QAM | 6.5 | 9 |

| Contig. CA MPR (dB) |  | < 400 | < 800 | < 1400 | < 2400 |
| --- | --- | --- | --- | --- | --- |
| DFT-s | 64QAM | 9 | 10.7 | 11.2 | 11.7 |
| CP- | 64QAM | 9 | 10.7 | 11.2 | 11.7 |

+0.5

+1.7

+0.5

| NC CA MPR (dB) |  | < 400 | < 800 | < 1400 | < 2400 |
| --- | --- | --- | --- | --- | --- |
| DFT-s | 64QAM | 9 | 10.7 | 11.2 | 11.7 |
| CP- | 64QAM | 9 | 10.7 | 11.2 | 11.7 |

+0.5

+0.5

+1.7

Based on the observed patterns, the following proposal can be made:

Proposal 1: Intra-band CA MPRs for both, contig. and NC, and for both PC1 and PC5 in 256QAM operation are increased from their respective 64QAM values by 3 dB as shown in table below:

| Contig. and NC CA, PC1 and PC5, MPR (dB) |  | < 400 | < 800 | < 1400 | < 2400 |
| --- | --- | --- | --- | --- | --- |
| DFT-s | 64 QAM | 9 | 10.7 | 11.2 | 11.7 |
|  | 256 QAM | 9+3 | 10.7+3 | 11.2+3 | 11.7+3 |
| CP- | 64 QAM | 9 | 10.7 | 11.2 | 11.7 |
|  | 256 QAM | 9+3 | 10.7+3 | 11.2+3 | 11.7+3 |

# References

[1]	R4-2317595, ‘Draft CR for Rel-18 38.101-2 to introduce FR2-1 UL 256 QAM RF requirements’, Xiaomi et al, RAN#108Bis, Xiamen, CN, Oct 2023

[2]	R4-2315054, ‘MPR for FR2 UL256QAM’, Qualcomm, RAN#108Bis, Xiamen, CN, Oct 2023