3GPP TSG-RAN WG4 Meeting #109	R4-2318787

Chicago, US, Nov 13 – Nov 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Advanced Receivers - Test parameters - Simulations

Agenda item:	8.18.2.2

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenario. Updated agreements were made in RAN4#108-bis [1] for facilitating companies to provide simulation results.

Simulation assumptions

Table 1 provides an overview of the already agreed simulation assumptions taken from the WF [1] and proposed test parameters:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver for phase II simulation | R-ML |
| --- | --- |
| Rank allocation for the target and co-scheduled UE, with 1 co-scheduled UE | 2Tx2Rx UE: 1+1 2Tx4Rx UE: 1+1 4Tx4Rx UE: 2+2 |
| Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs | 2Tx 2Rx: 1+1 (Frequency multiplexed co-UEs) 4Tx4Rx: 2+1+1 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for - rank 1 (DMRS port 0) + 1 (DMR port 1) |
| Antenna configuration | For rank 1+1: 2T2R, 2T4R For rank 2+2: 4Tx 4Rx For rank2+1+1: 4Tx4Rx |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium (for 2T2R,2T4R), ULA Low (for 2T4R only) Rank 2+2: ULA Low Rank 2+1+1: ULA Low |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Random PMI selection For rank 2+2: Orthogonal PMI selection For rank2+1+1: Orthogonal PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Only consider 1 and 2 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | MCS 13 for rank1+1. MCS17 for rank2+2, rank2+1+1 |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth configuration. Full OFDM symbol allocation. |

Simulation results

Based on the information in Table 1, we provide the simulation results on the cases of interest for this meeting, listed in Table 2.

Table 2 Simulation Overview – study cases for genie-aided knowledge of all parameters + blind detection

| Case | Co- UE number | Rank target UE | Rank Co- UE | MCS target UE | Modulation order co- UE | MIMO | Channel model | Precoder Co- UE | FDRA Co-UE | R-ML genie | R-ML BD1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 13 | QPSK | 2Tx2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 15 | 15.1 |  |
| 2 |  |  |  |  | 16QAM |  |  |  |  | 18.8 | 19.2 |  |
| 3 |  | 2 | 2 | 17 | QPSK | 4Tx 4Rx ULA medium | TDLA30-10 | orthogonal |  | 13.6 | 13.6 |  |
| 4 |  |  |  |  | 16QAM |  |  |  |  | 16.4 | 16.4 |  |
| 5 |  | 1 | 1 | 13 | QPSK | 2Tx 4Rx ULA low | TDLA30-10 | random |  | 6.9 | 6.9 |  |
| 6 |  |  |  |  | 16QAM |  |  |  |  | 8.4 | 8.4 |  |
| 7 |  | 1 | 1 | 13 | QPSK | 2Tx 4Rx ULA medium | TDLC300-100 |  |  | 12.7 | 12.7 |  |
| 8 |  |  |  |  | 16QAM |  |  |  |  | 16.9 | 17.1 |  |
| 9 |  | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Partial CHBW allocation (0~25 PRBs) | 12.7 | 12.8 |  |
| 10 |  | 1 | 1 | 13 | 16QAM | 2Tx 2Rx ULA medium |  |  |  | 14.4 | 14.6 |  |
| 11 |  | 2 | 2 | 17 | QPSK | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | 12.9 | 12.9 |  |
| 12 |  | 2 | 2 | 17 | 16QAM | 4Tx 4Rx ULA Low |  |  |  | 15.3 | 15.3 |  |
| 13 |  | 1 | 1 | 13 | QPSK | 2Tx 4Rx ULA low | TDLA30-10 | random |  | 6.1 | 6.1 |  |
| 14 |  | 1 | 1 | 13 | 16QAM |  |  |  |  | 6.8 | 6.8 |  |
| 15 |  | 1 | 1 | 13 | QPSK | 2Tx 4Rx ULA medium | TDLC300-100 |  |  | 10.3 | 10.4 |  |
| 16 |  | 1 | 1 | 13 | 16QAM |  |  |  |  | 12.4 | 12.4 |  |
| 17 | 2 | 2 | 1 for each Co-UE | 17 | Co-UE1: QPSK Co-UE2: 16QAM | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal | Full CHBW allocation (52PRBs) | 17.6 | 17.6 |  |
| 18 |  | 1 | 1 for each Co-UE | 13 | Co-UE1: QPSK Co-UE2: 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 | random | Co-UE1: 0~25 PRBs Co-UE2: 38~51 PRBs | 15.1 | 15.2 |  |
| 19 |  | 2 | 1 for each Co-UE | 17 | Co-UE1: QPSK Co-UE2: QPSK | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal | Full CHBW allocation (52PRBs) | 15.4 | 15.5 |  |
| 20 |  | 1 | 1 for each Co-UE | 13 | Co-UE1: QPSK Co-UE2: QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Co-UE1: 0~25 PRBs Co-UE2: 38~51 PRBs | 14.2 | 14.3 |  |
| Note 1: R-ML BD refers to R-ML with blind detection of co-scheduled UEs DMRS ports and FDRA only. |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution we have provided Nokia's simulation results for Advanced receivers.

- References

- R4-2316915 - WF on advanced receiver for MU-MIMO