3GPP TSG-RAN WG4 Meeting #109	R4-2318789

Chicago, US, Nov 13 – Nov 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Lessthan5MHz UE demod perf and CSI requirements – simulation results

Agenda item:	8.14.6.1

Document for:	Discussion

Introduction

This contribution contains Nokia’s initial simulation results for the <5MHz WID [1].

Simulation results

PDSCH simulation results

In Table 1 and Table 2 we have provided our simulation results for PDSCH using the existing RedCap requirements from 38.101-4 as baseline.

Table 1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) 10MHz/3MHz filter |
| 1-1 | R.PDSCH.1-1.1 FDD | 3 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | -2.3/-2.2 |
|  |  |  |  |  | 2x4, ULA Low | 70 | -5.7/-5.6 |
| 1-2 | R.PDSCH.1-2.1 FDD | 3 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 5.0/5.2 |
|  |  |  |  |  | 2x4, ULA Low | 70 | TBA |
| 1-3 | R.PDSCH.1-4.1 FDD | 3 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 20.0/21.3 |
|  |  |  |  |  | 2x4, ULA Low | 70 | TBA |
| 1-4 | R.PDSCH.1-4.1 FDD | 3 / 15 | 64QAM 0.43 | HST417 | 1x2, ULA Low | 70 | TBA/6.6 |
|  |  |  |  |  | 1x4, ULA Low | 70 | TBA/3.6 |

Table 2: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) 10MHz/3MHz filter |
| 2-1 | R.PDSCH.1-3.1 FDD | 3 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | 18.4/18.7 |
|  |  |  |  |  | 2x4, ULA Low | 70 | TBA/12.0 |

PDCCH simulation results

Non-punctured

In Table 3 we have provided our simulation results for the two discussed options with 5MHz CBW and AL4 and 3MHz CBW and AL2.

Table 3 PDCCH simulation results with non-punctured configuration.

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) | SNR(dB) with RX Filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 5 | 24 | 2 | 4 | TDLC300-100 | 1x2 Low | no | 1 | -0.5 | -0.5 |
| 1-2 | 5 | 24 | 2 | 4 | TDLC300-100 | 2x2 Low | no | 1 | -2 | -2 |
| 1-3 | 5 | 24 | 2 | 4 | TDLA30-10 | 1x4 Low | no | 1 | -4 | -3.9 |
| 1-4 | 5 | 24 | 2 | 4 | TDLA30-10 | 2x4 Low | no | 1 | -5.7 | -5.6 |
| 1-5 | 3 | 15 | 2 | 2 | TDLC300-100 | 1x2 Low | no | 1 | 3.8 | 4 |
| 1-6 | 3 | 15 | 2 | 2 | TDLC300-100 | 2x2 Low | no | 1 | 2.3 | 2.6 |
| 1-7 | 3 | 15 | 2 | 2 | TDLA30-10 | 1x4 Low | no | 1 | -1 | -0.5 |
| 1-8 | 3 | 15 | 2 | 2 | TDLA30-10 | 2x4 Low | no | 1 | -2.3 | -1.7 |

Punctured

We have included Figure 1 which shows how CCE puncturing will occur for different configurations.

Figure 1: CCE to REG mapping with interleaved and non-interleaved options in 15 RB CORESET#0

As indicated in our discussion contribution we focus on the configuration with 3 symbols, interleaved, AL8, which will test puncturing of 2.5 CCEs.

Our simulation results are shown in Table 4:

Table 4: PDCCH simulation results for punctured CORESET#0

| Test# | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) | SNR(dB) with RX Filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 3 | 15 | 3 | 8 | TDLA30-10 | 1x2 Low | yes | 1 | 1.1 | 1.5 |
| 2-2 | 3 | 15 | 3 | 8 | TDLC300-100 | 2x2 Low | yes | 1 | -3.8 | -3.7 |
| 2-3 | 3 | 15 | 3 | 8 | TDLA30-10 | 1x4 Low | yes | 1 | -5.3 | -4.9 |
| 2-4 | 3 | 15 | 3 | 8 | TDLC300-100 | 2x4 Low | yes | 1 | -7.4 | -7.3 |

PBCH simulation results

The results in Table 5 are done based on the agreement “Define requirements for 12 PRB PBCH with 3 MHz CBW, 15kHz SCS, FDD, unknown SSB/PBCH index” (see [1]):

Table 5 PBCH simulation results for non-HST case (unknown index)

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1Tx/2Rx Low | 1 | -2.8 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1Tx/4Rx Low | 1 | -7 |

In addition, Table 6 contains our simulation results for PBCH using the HST417 channel model.

Table 6 PBCH simulation results for HST case (unknown index)

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | HST417 | 1Tx/2Rx Low | 1 | -6.4 |
| FDD | 3 / 15 | Unknown | HST417 | 1Tx/4Rx Low | 1 | -8.9 |

Conclusion

Within this contribution we have provided our simulation results PDSCH, PDCCH and PBCH related to <5MHz CBW.

- References

- R4-2316924 - WF on NR_FR1_lessthan_5MHz_BW_demod