3GPP TSG-RAN WG4 Meeting #109	R4-2318791

Chicago, US, Nov 13 – Nov 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - PDSCH

Agenda item:	8.7.4.2

Document for:	Discussion

Introduction

During the RAN4#108bis discussions, agreements were made for most of the discussion points regarding the topic of MultiRx demodulation performance and CSI requirements for PDSCH and related issues.

In the following, we would provide Nokia’s viewpoint on the topic and discuss on the remaining open issues related to the topic.

Discussion

Test cases and simulation parameters for mDCI fully-overlapping

In RAN4#108bis following agreements were made regarding the simulation parameters for mDCI fully-overlapping configuration [1].

Issue 2-1-4: Test cases and simulation parameters for mDCI fully overlapping. - Agreement: Parameter; Unit; Value. TRxP #1(Note 1); TRxP #2(Note 1). Transmit TRxP of SSB; TRxP #1. PDCCH configuration; TCI state; TCI State #1; TCI State #2. CORESETPoolIndex; 0,1. Duplex mode; TDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 1. Length (L); Specific to each Reference channel as defined in A.3.2.2. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 1. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; Antenna port indexes; 1+1: {1000} 2+2: {1000,1001}; 1+1: {1002} {1002,1003}. TCI state; TCI State #1; TCI State #2. DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. Resource allocation; Fully-overlapping. Precoding configuration; SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity.. Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1.

We have used the agreed parameter setup given above for Nokia’s simulations and our observations as per the simulation results are discussed below,

Figure 1: Simulation results for mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing

Figure 2: Simulation results for mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing

It can be observed from the simulation results that higher crosstalk interference significantly reduces the achievable maximum throughput when separate processing is used. This behavior can be observed for all the three combinations of time and frequency offsets used [4]. Additionally, as per the simulation results presented by other companies in RAN4#108bis, it can be observed that joint processing would be more effective in alleviating the effects of crosstalk interference. Therefore, joint processing would be essential to achieve reasonable throughput performance for mDCI with a fully-overlapping configuration in real propagation conditions where it would be likely to receive signals from the TRPs at lower AoA offsets accounting for higher values of crosstalk interference.

When separate processing is used, crosstalk interference can significantly reduce the demodulation performance for the mDCI fully-overlapping scenario for both 1+1 and 2+2 scenarios already from ρ = -12dB.

Additionally, from the simulation results, it can be observed that the use of the agreed time and frequency offset combinations does not degrade the demodulation performance [4].

The simulation results obtained without the added impairments of time and frequency offset and with the addition of corresponding time and frequency offset combinations of (-0.0625us, 600Hz) and (0.25us, 0Hz) are identical for the agreed simulation configurations of mDCI fully-overlapping Rank 1+1 with MCS17 and Rank 2+2 with MCS13 using separate processing.

Based on our observations for the mDCI fully-overlapping scenario we have the following proposals:

Use the time and frequency offset combination of (-0.0625us, 600Hz) for mDCI fully-overlapping simulations.

Define following test cases for mDCI fully-overlapping scenario with separate processing:

| Test num. | Layer combination |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Time offest and frequency offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | Rank 1+1 |  | 100 / 120 | 64QAM, 0.43 | TDLA30-75 | 2x2 XPL (ρ = -12dB) | (-0.0625us, 600Hz) | 70 | TBD |
| 1-2 | Rank 2+2 |  | 100 / 120 | 16QAM, 0.48 | TDLA30-75 | 2x2 XPL (ρ = -12dB) | (-0.0625us, 600Hz) | 70 | TBD |
|  |  | Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

Considering the considerably more effective demodulation performance, we propose following test cases for the mDCI-based full-overlapping scenario with joint processing.

Define the following test cases for mDCI fully-overlapping scenario with joint processing:

| Test num. | Layer combination |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Time offest and frequency offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-3 | Rank 1+1 |  | 100 / 120 | 64QAM, 0.43 | TDLA30-75 | 2x2 XPL (ρ = -6dB) | (-0.0625us, 600Hz) | 70 | TBD |
| 1-4 | Rank 2+2 |  | 100 / 120 | 16QAM, 0.48 | TDLA30-75 | 2x2 XPL (ρ = -6dB) | (-0.0625us, 600Hz) | 70 | TBD |
|  |  | Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

Test cases and simulation parameters for mDCI non-overlapping

In RAN4#108bis following agreements were made regarding the simulation parameters for mDCI non-overlapping configuration [1].

Issue 2-1-5: Test cases and simulation parameters for mDCI non-overlapping - Agreement: Parameter; Unit; Value. TRxP #1(Note 1); TRxP #2(Note 1). Transmit TRxP of SSB; TRxP #1. PDCCH configuration; TCI state; TCI State #1; TCI State #2. CORESETPoolIndex; 0,1. Duplex mode; TDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 1. Length (L); Specific to each Reference channel as defined in A.3.2.2. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 1. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; Antenna port indexes; 2+2: {1000,1001}; 2+2: {1002,1003}. TCI state; TCI State #1; TCI State #2. DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. Resource allocation; non-overlapping. Precoding configuration; SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity.. Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1.

We have used the agreed parameter setup given above for Nokia’s simulations and our observations as per the simulation results are discussed below,

Figure 3: mDCI non-overlapping Rank 2+2 with MCS13 – Separate processing (-0.0625us, 600Hz).

As per Nokia’s simulation results, the effect of crosstalk interference on the demodulation performance is significantly lower, though still measurable, for the mDCI non-overlapping scenario when compared with the mDCI fully-overlapping scenario.

The effect of crosstalk interference on the demodulation performance is still measurable for the mDCI non-overlapping scenario, but with much lower impact. To include the effect of the cross-talk for requirement definition a higher value of ρ can be selected.

Define following test cases for mDCI non-overlapping scenario with separate processing.

| Test num. | Layer combination |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Time offest and frequency offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-5 | Rank 2+2 |  | 100 / 120 (RB allocation: 32/32) | 16QAM, 0.48 | TDLA30-75 | 2x2 XPL (ρ = -6dB) | (-0.0625us, 600Hz) | 70 | TBD |
|  |  | Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

Conclusion

In this paper we present Nokia’s view on the PDSCH requirements of FR2 multi-RX downlink demodulation performance.

We have made following Observations and Proposals within this contribution:

Test cases and simulation parameters for mDCI fully-overlapping

- When separate processing is used, crosstalk interference can significantly reduce the demodulation performance for the mDCI fully-overlapping scenario for both 1+1 and 2+2 scenarios already from ρ = -12dB.

- The simulation results obtained without the added impairments of time and frequency offset and with the addition of corresponding time and frequency offset combinations of (-0.0625us, 600Hz) and (0.25us, 0Hz) are identical for the agreed simulation configurations of mDCI fully-overlapping Rank 1+1 with MCS17 and Rank 2+2 with MCS13 using separate processing.

- Use the time and frequency offset combination of (-0.0625us, 600Hz) for mDCI fully-overlapping simulations.

Define following test cases for mDCI fully-overlapping scenario with separate processing:

| Test num. | Layer combination |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Time offest and frequency offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | Rank 1+1 |  | 100 / 120 | 64QAM, 0.43 | TDLA30-75 | 2x2 XPL (ρ = -12dB) | (-0.0625us, 600Hz) | 70 | TBD |
| 1-2 | Rank 2+2 |  | 100 / 120 | 16QAM, 0.48 | TDLA30-75 | 2x2 XPL (ρ = -12dB) | (-0.0625us, 600Hz) | 70 | TBD |
|  |  | Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

Define the following test cases for mDCI fully-overlapping scenario with joint processing:

| Test num. | Layer combination |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Time offest and frequency offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-3 | Rank 1+1 |  | 100 / 120 | 64QAM, 0.43 | TDLA30-75 | 2x2 XPL (ρ = -6dB) | (-0.0625us, 600Hz) | 70 | TBD |
| 1-4 | Rank 2+2 |  | 100 / 120 | 16QAM, 0.48 | TDLA30-75 | 2x2 XPL (ρ = -6dB) | (-0.0625us, 600Hz) | 70 | TBD |
|  |  | Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

Test cases and simulation parameters for mDCI non-overlapping

The effect of crosstalk interference on the demodulation performance is still measurable for the mDCI non-overlapping scenario, but with much lower impact. To include the effect of the cross-talk for requirement definition a higher value of ρ can be selected.

Define following test cases for mDCI non-overlapping scenario with separate processing.

| Test num. | Layer combination |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Time offest and frequency offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-5 | Rank 2+2 |  | 100 / 120 (RB allocation: 32/32) | 16QAM, 0.48 | TDLA30-75 | 2x2 XPL (ρ = -6dB) | (-0.0625us, 600Hz) | 70 | TBD |
|  |  | Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

- References

- R4-2316963 - WF on NR_FR2_multiRX_DL_Demod

- R4-2317947 - Topic summary for [108bis][316] NR_FR2_multiRX_DL_Demod

- R4-2316929 - Ad-hoc meeting minutes on NR_FR2_multiRX_DL_Demod

- R4-2318792 - On MultiRx Demodulation performance and CSI requirements - Simulation Results

- R4-2315938 - Simulation results of PDSCH requirements of FR2 multiRX DL (MediaTek)