3GPP TSG-RAN WG4 Meeting #109	R4-2318792

Chicago, US, Nov 13 – Nov 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - Simulation Results

Agenda item:	8.7.4.2

Document for:	Discussion

Introduction

In the RAN4#107 [1], simulation configurations including the channel model was agreed. Further agreements were reached in the RAN#108 [2] and RAN#108bis [3] discussions regarding the simulation parameters and the PDSCH demodulation configurations for the simulations.

This paper contains Nokia’s simulation results for Multi Rx.

Simulation Configuration

Based on the agreements in RAN4#108bis, the following applies for simulation setup used for our simulations:

Table 1: The simulation parameters

|  |  | Notes |
| --- | --- | --- |
| sDCI Separate Processing | MIMO detector processing 2Rx and li layers per TRP |  |
| sDCI Joint Processing | MIMO detector processing 4Rx and l1 + l2 layers across both TRP |  |
| mDCI Separate Processing | MIMO detector processing 2Rx and li layers per TRP |  |
| mDCI Joint Processing | MIMO detector processing 4Rx and l1 + l2 layers across both TRP |  |
| Receiver (separate and joint) | MMSE-IRC |  |
| Channel model | TDLA30-75 + correlation/cross-talk model |  |
| Carrier frequency | FR2-1, 30 GHz |  |
| CBW/SCS | 100MHz/120kHz |  |
| Antenna configuration | 2x2, XPL per TRP |  |
| DMRS symbol allocation | (2,11) symbol indexes |  |
| Cross-correlation coefficient (Gamma) | 0.0625 |  |
| PDSCH resource allocation | 66 per TRP & 32 per TRP | 66 per TRP for 3.2.2 & 3.2.3 32 per TRP for 3.1.2 |
| MCS index (MCS table 1) | 64QAM, 0.43 (MCS17) & 16QAM, 0.48 (MCS13) | MCS13 for 3.1.2 & 3.2.3 MCS17 for 3.2.2 |
| Time offset / Frequency offset | Option 1: (0us, 0Hz) Option 2: (-0.0625us, 600Hz) Option 3: (0.25us, 0Hz) |  |

Simulation Results

mDCI non-overlapping

Agreed simulation configuration (RAN4#108bis)

In RAN4#108bis following simulation parameters were agreed for the mDCI non-overlapping configuration [3].

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 2+2: {1000,1001} | 2+2: {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation results for Rank 2+2 non-overlapping with MCS13 – Separate processing.

Simulation results given under the subsection 3.1.2 consider the multi TRP scheme with 2Tx2Rx antenna configuration and (2+2) layer combination for different values of crosstalk interference (rho) using MCS13. Cross-polarization correlation coefficient of 0.0625 was used for the simulations.

The simulation results with time and frequency offset of (-0.0625us, 600Hz) respectively are shown in Figure 1.

Figure 1: mDCI non-overlapping Rank 2+2 with MCS13 – Separate processing (-0.0625us, 600Hz).

Table 2: Results for mDCI non-overlapping Rank 2+2 with MCS13 – Separate processing (-0.0625us, 600Hz).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI non-overlapping Rank 2+2 (-0.0625us, 600Hz) | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 32/32 | No Crosstalk | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 32/32 | -15 | 70 | 10.1 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 32/32 | -12 | 70 | 10.2 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 32/32 | -9 | 70 | 10.5 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 32/32 | -6 | 70 | 11.0 |

It can be observed from the simulation results that the effect of crosstalk interference on the demodulation performance is significantly lower for the mDCI non-overlapping scenario.

The effect of crosstalk interference on the demodulation performance is still measurable for the mDCI non-overlapping scenario, but with much lower impact.

mDCI Fully-overlapping

Agreed simulation configuration (RAN4#108bis)

In RAN4#108bis following simulation parameters were agreed for the mDCI fully-overlapping configuration [3].

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1+1: {1000} 2+2: {1000,1001} | 1+1: {1002} {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Fully-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation results for Rank 1+1 fully-overlapping with MCS17 – Separate processing.

Simulation results given under the subsection 3.2.2 consider the multi TRP scheme with 2Tx2Rx antenna configuration and (1+1) layer combination for different values of crosstalk interference (rho) using MCS17. Cross-polarization correlation coefficient of 0.0625 was used for the simulations.

The simulation results without time or frequency offset are shown in Figure 2.

Figure 2: mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing (without time and frequency offsets).

Table 3: Results for mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing (without time and frequency offsets).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 1+1 (0us, 0Hz) | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | No Crosstalk | 70 | 9.1 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -15 | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -12 | 70 | 11.3 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -9 | 70 | 16.3 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -6 | 70 | N/A |

The simulation results with time and frequency offset of (-0.0625us, 600Hz) respectively are shown in Figure 3.

Figure 3: mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing (-0.0625us, 600Hz).

Table 4: Results for mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing (-0.0625us, 600Hz).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 1+1 (-0.0625us, 600Hz) | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | No Crosstalk | 70 | 9.1 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -15 | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -12 | 70 | 11.3 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -9 | 70 | 16.3 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -6 | 70 | N/A |

The simulation results with time and frequency offset of (0.25us, 0Hz) respectively are shown in Figure 4.

Figure 4: mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing (0.25us, 0Hz).

Table 5: Results for mDCI fully-overlapping Rank 1+1 with MCS17 – Separate processing (0.25us, 0Hz).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 1+1 (0.25us, 0Hz) | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | No Crosstalk | 70 | 9.1 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -15 | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -12 | 70 | 11.3 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -9 | 70 | 16.3 |
|  | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 2x2 XPL | 66 | -6 | 70 | N/A |

It can be observed from the simulation results that with higher crosstalk interference, it significantly reduces the achievable maximum throughput when separate processing is used. This behavior can be observed for all the three combinations of time and frequency offset used.

Additionally, it can be observed from the simulation results that the use of the agreed time and frequency offset combinations does not degrade the demodulation performance.

The simulation results obtained without the added impairments of time and frequency offset and with the addition of corresponding time and frequency offset combinations of (-0.0625us, 600Hz) and (0.25us, 0Hz) are identical for the scenario of mDCI fully-overlapping Rank 1+1 with MCS17 using separate processing.

Simulation results for Rank 2+2 fully-overlapping with MCS13 – Separate processing.

Simulation results given under the subsection 3.2.3 consider the multi TRP scheme with 2Tx2Rx antenna configuration and (2+2) layer combination for different values of crosstalk interference (rho) using MCS13. Cross-polarization correlation coefficient of 0.0625 was used for the simulations.

The simulation results without time or frequency offset are shown in Figure 5.

Figure 5: mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing (without time and frequency offsets).

Table 6: Results for mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing (without time and frequency offsets).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 2+2 (0us, 0Hz) | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | No Crosstalk | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -15 | 70 | 11.5 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -12 | 70 | 13.7 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -9 | 70 | N/A |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -6 | 70 | N/A |

The simulation results with time and frequency offset of (-0.0625us, 600Hz) respectively are shown in Figure 6

Figure 6: mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing (-0.0625us, 600Hz).

Table 7: Results for mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing (-0.0625us, 600Hz).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 2+2 (-0.0625us, 600Hz) | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | No Crosstalk | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -15 | 70 | 11.5 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -12 | 70 | 13.7 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -9 | 70 | N/A |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -6 | 70 | N/A |

The simulation results with time and frequency offset of (0.25us, 0Hz) respectively are shown in Figure 7.

Figure 7: mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing (0.25us, 0Hz).

Table 8: Results for mDCI fully-overlapping Rank 2+2 with MCS13 – Separate processing (0.25us, 0Hz).

| Simulation Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration (2Tx 2Rx) (α=β=0, γ=0.0625) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mDCI fully-overlapping Rank 2+2 (0.25us, 0Hz) | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | No Crosstalk | 70 | 10.0 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -15 | 70 | 11.5 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -12 | 70 | 13.7 |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -9 | 70 | N/A |
|  | 100/120 | FR2.120-1 | MCS13 | 2+2 | TDLA30-75 | 2x2 XPL | 66 | -6 | 70 | N/A |

It can be observed from the simulation results that with higher crosstalk interference, it significantly reduces the achievable maximum throughput when separate processing is used. This behavior can be observed for all the three combinations of time and frequency offset used.

Additionally, it can be observed from the simulation results that the use of the agreed time and frequency offset combinations does not degrade the demodulation performance.

The simulation results obtained without the added impairments of time and frequency offset and with the addition of corresponding time and frequency offset combinations of (-0.0625us, 600Hz) and (0.25us, 0Hz) are identical for the scenario of mDCI fully-overlapping Rank 2+2 with MCS13 using separate processing.

Conclusion

Within this contribution we have presented our simulation results for the performance alignment of PDSCH requirements of FR2 multi-RX downlink demodulation requirements.

We have made following observations within the contribution,

- The effect of crosstalk interference on the demodulation performance is still measurable for the mDCI non-overlapping scenario, but with much lower impact.

The simulation results obtained without the added impairments of time and frequency offset and with the addition of corresponding time and frequency offset combinations of (-0.0625us, 600Hz) and (0.25us, 0Hz) are identical for the scenario of mDCI fully-overlapping Rank 1+1 with MCS17 using separate processing.

The simulation results obtained without the added impairments of time and frequency offset and with the addition of corresponding time and frequency offset combinations of (-0.0625us, 600Hz) and (0.25us, 0Hz) are identical for the scenario of mDCI fully-overlapping Rank 2+2 with MCS13 using separate processing.

- References

- R4-2309824 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception (RAN4#107)

- R4-2313977 - WF for Rel-18 Multi-Rx performance requirements (RAN4#108)

- R4-2316963 - WF on NR_FR2_multiRX_DL_Demod (RAN4#108bis)