3GPP TSG-RAN WG4 Meeting #109	R4-2318793

Chicago, US, Nov 13 – Nov 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - PMI

Agenda item:	8.7.4.3

Document for:	Discussion

Introduction

During RAN4#108bis meeting, discussions on open issues regarding CSI requirements and simulation assumptions for proper PMI reporting continued. Particularly, simulation assumptions on TDD pattern, channel modeling and UE processing for PMI reporting test cases were addressed in detail. Next, we provide Nokia view on such open issues while providing new proposals whenever applicable.

Discussion

Simulation parameters for PMI reporting requirement for sDCI SDM scheme

In RAN4#108bis meeting, the initial set of parameters first proposed in RAN4#108 (see [1]) were further discussed and few simulation parameters (for example, CQI/RI/PMI delay) were updated accordingly.

Issue 3-1-6: Simulation parameters for PMI reporting requirement for sDCI SDM scheme - Agreement: Parameter; Unit; Value. TRxP #1 (Note 1); TRxP #2 (Note 1). Transmit TRxP of SSB; TRxP #1. PDCCH configuration; TCI state; TCI State #1. CORESETPoolIndex; 0. Duplex mode; TDD. Bandwidth; MHz; 100. Subcarrier spacing; kHz; 120. TDD DL-UL configurations; FR2.120-1 as specified in Annex A of TS 38.101-4. Active DL BWP index; 1. Propagation channel; TDLA30-35. Antenna configuration per TRxP; XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 2 panels). Beamforming Model; As specified in Annex B.4.1 of TS 38.101-4. Physical channel for CSI report; PUSCH. CQI/RI/PMI delay; ms; 1.75. Maximum number of HARQ transmission; 4.

In the simulation parameters table above, the antenna configuration per TRxP proposes a configuration XP 2X4 (N1,N2)= (2,1) with 4 antennas at UE across 2 panels; however, we see that a configuration with 4 antennas at UE across 1 panel is more flexible and thus achieves better performance.

Considering the codebook alternatives for PMI reporting, type I uses predefined matrices which are selected based on UE feedback and higher layer RRC configuration. Then, the supported number (N1,N2) of antennas on the horizontal and vertical directions for distinct number of CSI-RS ports and corresponding DFT oversampling (O1,O2) values are provided in the specification [2].

We see “XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 1 panel)” to have better performance compared to “XP 2x4 (Ng,N1,N2) = (2,1,1) (4 antenna at UE across 2 panels)” as it has more flexibility.

RAN4 to adapt XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 1 panel) for antenna configuration when defining PMI requirements.

Reference Channel

The need to introduce a new reference channel for PMI reporting in sDCI SDM scenarios was discussed during the RAN4#108bis meeting. While an agreement favouring the introduction of such reference channel was reached, the exact details of its implementation are still open for further investigation.

Issue 3-1-8: Reference Channel - Agreement: - Define new reference channel for PMI reporting requirement for sDCI SDM. - FFS on the exact details.

In the existing 38.101-4 specification we see that the reference channel used in section 6.3.2.1.7 with adaptation to 4 ports can be used as a starting point. In addition, we see two ways to configure the codebook (2 panel and 1 panel). Both will in our understanding work, however we slightly prefer the configure the codebook for 1 panel as such configuration will have more choices compared to the 2 panel option.

Reuse the reference channel from TS38.101-4, section 6.3.2.1.7 with adaptation to 4 ports as starting point. Configure the codebook for 1 panel option.

Conclusion

Within this contribution have presented Nokia's view on the open issues with relation definition of PMI requirements for MultiRx Demodulation performance.

We have the following observations and proposals:

Simulation parameters for PMI reporting requirement for sDCI SDM scheme

- We see “XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 1 panel)” to have better performance compared to “XP 2x4 (Ng,N1,N2) = (2,1,1) (4 antenna at UE across 2 panels)” as it has more flexibility.

- RAN4 to adapt XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 1 panel) for antenna configuration when defining PMI requirements.

Reference Channel

Reuse the reference channel from TS38.101-4, section 6.3.2.1.7 with adaptation to 4 ports as starting point. Configure the codebook for 1 panel option.

- References

- R4-2315916 On MultiRx Demodulation performance and CSI requirements – PMI.

- TS 38.214 Physical layer procedures for data v17.7.0.