3GPP TSG-RAN WG4 Meeting #109 	R4-2318794

# Chicago, US, 13th - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-4: Minimum requirements and Measurement Channel for mDCI non-overlapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL_Demod |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of minimum requirements for mDCI in non-overlapping case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of two additional sections: Definition of new requirements and definition of new reference measurement channels |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements will be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 7.2.2.2.6, A3.2.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1>>

##### 7.2.2.2.6	Minimum requirements for mDCI non-overlapping

The performance requirements are specified in Table 7.2.2.2.6-3 with the addition of the parameters in Table 7.2.2.2.6-2 and the downlink physical channel setup according to Annex C.5.1.

The test purposes are specified in Table 7.2.2.2.6-1.

Table 7.2.2.2.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and when UE receives multiple PDCCHs scheduling PDSCHs | 1-1 |

Table 7.2.2.2.6-2: Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Table 7.2.2.2.6-3: Minimum performance

| Test num. |  |  | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) |  |  |  |  | Modulation format and code rate |  |  | Propagation condition(Note 1) |  |  | Correlation matrix and antenna configuration(Note 2) |  |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  |  | SNR (dB)(Note 3) |
| 1-1 | R.PDSCH.10-1.1 TDD |  |  | R.PDSCH.10-1.2 TDD |  | 100 / 120 |  | 16QAM, 0.48 |  |  | TDLA30-75 |  |  | TBD [ρ = -12dB] |  |  | 70 |  |  | TBD |  |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply as defined in B.2.5 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |

## << End of change 1 >>

## << Start of change 2>>

#### A.3.2.2.10	Reference measurement channels for mDCI non-overlapping

Table A.3.2.2.10-1: PDSCH Reference Channel for TDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.10-1.1 TDD | R.PDSCH.10-1.2 TDD |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | [32] (Note 3) | [32] (Note 4) |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 10 | 10 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 |  |  |  |
| Allocated slots per 2 frames |  | 119 | 119 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 12552 | 12552 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 16896 | 16896 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 2 | 2 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 | 3 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slots i = 80, 81 | Bits | 33920 | 33920 |  |  |  |
| For Slots i = 82 | Bits | 26624 | 26624 |  |  |  |
| For Slots i = 83 | Bits | 35456 | 35456 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 91.843 | 91.843 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,84,…,159} | Bits | 73128 | 73128 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 201.434 | 201.434 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 31. Note 4: PDSCH is scheduled in PRB numbers from 34 to 65. |  |  |  |  |  |  |

## << End of change 2 >>