3GPP TSG-RAN WG4 Meeting # 109	R4-2318831

# Chicago, US, 13 – 17 November, 2023

Source:	LG Electronics

Title:	Discussion on NR sidelink CA operation

Agenda item:	8.30.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on remaining issues for NR SL CA based on approved WF[1].

Discussion

For sidelink CA, the constraints on interruption to WAN at SL carrier addition/release during WAN paging or SIB reception was discussed.

Issue 1-1: Constraints on interruptions to WAN at SL carrier addition/release due to paging and SIB reception Agreement: FFS if interruptions to WAN at SL carrier addition/release is allowed during WAN paging or system information reception FFS whether the scenario is valid that SL carrier addition/release can collide with WAN paging or SIB reception in RRC_IDLE/RRC_INACTIVE mode and whether LS is needed. FFS if RLF and CBD should be protected.

Network can configure the carrier set for SL CA operation in SIB12 based on RAN2 running CR [2].

However, it is up to SL UE when SL carrier is added or released for SL CA operation even during IDLE/INACTVE mode, so network cannot control SL carrier addition and release directly. Similar to existing interruption constraint, the constraints on interruptions to WAN at SL carrier addition/release can be applied.

- Proposal 1: Add the constraints on interruptions to WAN due to SL carrier addition/release

Conclusion

In this contribution, we provide views on NR sidelink CA operation, and, and propose

- Proposal 1: Add the constraints on interruptions to WAN at SL carrier addition/release

Reference

[1] R4-2317375, “WF on R18 NR SL RRM requirements (part 2),” OPPO.

[2] R2-2311495, “Running CR of TS 38.331 for SL Evolution,” OPPO