3GPP TSG-RAN WG4 Meeting # 109	R4-2318839

# Chicago, US, 13 – 17 November, 2023

Source:	LG Electronics

Title:	Discussion on RRM performance for NR sidelink evolution

Agenda item:	8.30.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on test cases for RRM performance part on NR sidelink evolution based on WF[1].

Discussion

In NR sidelink evoluation, following requirements for SL-U and SL CA were introduced.

  - For SL-U

    - UE transmit timing: existing requirements will be reused and add condition to apply the requirements

    - Initiation/Cease of S-SSB transmission: introduce new requirements

    - Synchronization reference selection/reselection: introduce new requirements

  - For SL CA

    - Selection / Reselection of Synchronization Reference Source under NR SL CA operation

    - Interruption to WAN due to SL carrier addition/release

For SL-U, new added requirements are related LBT failure impact, and the requirements and UE behavior are not big difference from exsiting one. To minimize test cases for SL-U RRM requirements, verification of SL-U operation in RRM seems to be sufficient. Therefore, one additional test case for initiation/cease of SLSS transmission could considered for SL-U RRM performance.

  - Proposal 1: Consider only one additional test cae for initiation/cease of SLSS transmission for SL-U RRM performance to verify SL-U operation.

SL CA is new feature for NR SL operation, so test case for SL CA operatio should be introduced. However, further discussion is needed whether to introduce test case for interruption to WAN due to SL carrier addition/release, since there is no band combination WAN+SL CA in RF session in this release even if RRM requirement is introduced. Therefore, one additional test case for selection/reselection of synchronization reference source under NR SL CA operation could be considered.

  - Proposal 2: Consider one additional test case for selection/reselection of synchronization reference source under NR SL CA operation.

Based on above proposals, following test case list could be considered in Rel-18 RRM performance part.

Table 1 Proposed list of test caes for NR SL RRM performance

| RRM core part | Test case list |
| --- | --- |
| SL-U |  |
| Iinitiation/Cease of S-SSB transmission | Test for SyncRef UE as synchronization reference source under CCA |
| SL CA |  |
| Selection/Reselection of synchronization reference source under NR SL CA operation | Test for GNSS configured as the highest priority for NR SL CA operation |

  - Proposal 3: Consider test cases for NR SL RRM performance in Table 1.

Conclusion

In this contribution, we provide views on the test cases for RRM performance part on NR sidelink evolution, and we propose

  - Proposal 1: Consider only one additional test cae for initiation/cease of SLSS transmission for SL-U RRM performance to verify SL-U operation.

  - Proposal 2: Consider one additional test case for selection/reselection of synchronization reference source under NR SL CA operation.

  - Proposal 3: Consider test cases for NR SL RRM performance in Table 1.

- Table 1 Proposed list of test caes for NR SL RRM performance

| RRM core part | Test case list |
| --- | --- |
| SL-U |  |
| Iinitiation/Cease of S-SSB transmission | Test for SyncRef UE as synchronization reference source under CCA |
| SL CA |  |
| Selection/Reselection of synchronization reference source under NR SL CA operation | Test for GNSS configured as the highest priority for NR SL CA operation |

Reference

[1] R4-2317375, “WF on R18 NR SL RRM requirements (part 2).” OPPO.