3GPP TSG-RAN4 Meeting #109		R4-2318845

Chicago, United States, 13th Nov. – 17th Nov., 2023

Title:	Discussion on RRM requirements for NTN deployment in above 10 GHz bands

Source:	Xiaomi

Agenda Item:	8.26.6.1

Document for:	Discussion

- Introduction

In RAN4#108bis meeting, the scope of RRM requirements for Rel-18 NTN deployment in above 10GHz bands was discussed, and the related WF was approved in [1]. In this contribution, we discuss the UE UL timing requirements for Rel-18 NTN deployment in above 10GHz bands and provide our proposals.

- Discussion

2.2	UE UL timing requirement

### Issue 1-6: Te_NTN for 60kHz and 120kHz Agreement: Companies should provide ‘the exact value of Te_NTN and values assumed for X and Y’ and ‘the analysis result based on the following criterion.’ Otherwise, the values/proposals won’t be captured in the list of options. Tg = 0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm): an effective guard period in CP - Tcp: a length of CP for the given SCS of UL channel/signal - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs) - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error - Tr: TAC resolution error (from TS38.213) - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133) - Tf: an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm - Tm: a margin needed at gNB receiver to accommodate any additional impairments if needed. - If a non-zero value is assumed in the proposal for Tm, the source of the impairments shall be provided too. Technical analysis is required if any number will be provided for each of the components in the next meeting. Whether the same or different values for different channels is contribution driven.

According to the equation agreed in last meeting, the maximum error budget for GNSS position error and satellite position error can be calculated in following table.

| SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | CP length (Tcp) | Tr (TAC resolution error) | Ta (TA adjustment accuracy error) | Td (UE downlink synchronization error) | Tp = Te_GNSS + Te_SAT | GNSS position error + serving-satellite position error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 60 | 36*64*Tc | 2*64*Tc | 2*64*Tc | 3.5*64*Tc | 10.5*64*Tc | 56m |
|  | 120 | 18*64*Tc | 1*64*Tc | 0.5*64*Tc | 3.5*64*Tc | 4*64*Tc | 21m |
| 240 | 60 | 36*64*Tc | 2*64*Tc | 2*64*Tc | 3*64*Tc | 11*64*Tc | 58m |
|  | 120 | 18*64*Tc | 1*64*Tc | 0.5*64*Tc | 3*64*Tc | 4*64*Tc | 21m |

Observation 1: The max error budget for GNSS position error (X) and serving-satellite position error (Y) is around 56m for 60 KHz UL SCS case and around 20m for 120 KHz UL SCS case.

Proposal 1: For 60 KHz UL SCS case, the GNSS position error is assumed as 15m and the serving-satellite position error is assumed as 30m, respectively.

Observation 2: It is too challenge for UE if the GNSS position error (X) is set as 10m and serving-satellite position error (Y) is set as 10m for 120 KHz UL SCS case.

Proposal 2: RAN4 not to define UL timing requirements for 120 KHz UL SCS case.

- Conclusion

In this contribution, we discussed the UL timing requirements for NTN bans above 10GHz and proposals are provided as follows:

Observation 1: The max error budget for GNSS position error (X) and serving-satellite position error (Y) is around 56m for 60 KHz UL SCS case and around 20m for 120 KHz UL SCS case.

Proposal 1: For 60 KHz UL SCS case, the GNSS position error is assumed as 15m and the serving-satellite position error is assumed as 30m, respectively.

Observation 2: It is too challenge for UE if the GNSS position error (X) is set as 10m and serving-satellite position error (Y) is set as 10m for 120 KHz UL SCS case.

Proposal 2: RAN4 not to define UL timing requirements for 120 KHz UL SCS case.

- Reference

[1]	R4-2317374	RRM WF and CR work split for NR_NTN_enh,	Qualcomm