3GPP TSG-RAN WG4 Meeting # 109	R4-2318849

# Chicago, USA, Nov. 13 – Nov. 17, 2023

Source: 	Xiaomi

Title: 	Discussion on RRM requirement for FeMIMO

Agenda Item:	8.29.2.1

Document for:	Discussion

# 1 Introduction

In RAN4#108bis meeting, it’s agreed to provide simulation results regarding to feasibility of TDCP test.

In this contribution, we provide simulation results and discuss how to design test case if feasible.

- Simulation assumption and results - Analysis of ideal correlation value - Test metric based on instant delta between ideal and estimated value - Test metric based on mean value delta between ideal and estimated value - Test metric based on CDF difference between ideal and estimated value - Test metric based only on estimated value

- Discussion

- 2.1 Simulation assumption and simulation results

The agreed simulation assumptions are as below:

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1slot |
| Channel model | TDL-A, delay spread=30ns |
| Doppler Spread | 10, 30, 75, 100, 200, 300 |
| SNR | 5:5:20 |
| Number of averaging samples: | one shot as baseline, 4 samples |
| Channel BW | 10MHz |
| SCS | 30KHz as baseline, 15KHz 2nd priority |
| Reference Channel estimation | LS CE for TRS as baseline, MMSE CE as 2nd priority |
| Correlation matrix and antenna configuration | 1x2 Low |

CDF curves of (including the 90% and 50% and 10% CDF points) for estimated TDCP. TDCP calculation reference:

ct,∆t=n=0N-1hnt+∆t∙hn*tnhnt+∆t2∙nhnt2

The simulation results for estimated correlation value are as below:

Fig.1 CDF of estimated correlation amplitude

Table 1: Estimated correlation value for SNR = 5dB, SCS=30KHz

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.3883 | 0.7243 | 0.8753 |
| Doppler = 30 Hz | 0.4105 | 0.7166 | 0.8706 |
| Doppler = 75 Hz | 0.3572 | 0.6910 | 0.8661 |
| Doppler = 100 Hz | 0.3264 | 0.6759 | 0.8617 |
| Doppler = 200 Hz | 0.2549 | 0.5908 | 0.8179 |
| Doppler = 300 Hz | 0.2283 | 0.5510 | 0.7754 |

Table 2: Estimated correlation value for SNR = 10dB, SCS=30KHz

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.6518 | 0.8757 | 0.9522 |
| Doppler = 30 Hz | 0.6604 | 0.8811 | 0.9580 |
| Doppler = 75 Hz | 0.5923 | 0.8598 | 0.9490 |
| Doppler = 100 Hz | 0.5410 | 0.8484 | 0.9470 |
| Doppler = 200 Hz | 0.4139 | 0.7646 | 0.9196 |
| Doppler = 300 Hz | 0.3651 | 0.7138 | 0.8905 |

Table 3: Estimated correlation value for SNR = 15dB, SCS=30KHz

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.8673 | 0.9587 | 0.9860 |
| Doppler = 30 Hz | 0.8395 | 0.9552 | 0.9843 |
| Doppler = 75 Hz | 0.7793 | 0.9437 | 0.9818 |
| Doppler = 100 Hz | 0.6944 | 0.9280 | 0.9798 |
| Doppler = 200 Hz | 0.5092 | 0.8532 | 0.9620 |
| Doppler = 300 Hz | 0.4386 | 0.7892 | 0.9411 |

Table 4: Estimated correlation value for SNR = 20dB, SCS=30KHz

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.9478 | 0.9862 | 0.9959 |
| Doppler = 30 Hz | 0.9381 | 0.9842 | 0.9948 |
| Doppler = 75 Hz | 0.8502 | 0.9715 | 0.9918 |
| Doppler = 100 Hz | 0.7602 | 0.9589 | 0.9901 |
| Doppler = 200 Hz | 0.5298 | 0.8935 | 0.9764 |
| Doppler = 300 Hz | 0.4620 | 0.8216 | 0.9582 |

- 2.2 Test metric

In legacy, accuracy requirement is defined based on the delta between ideal value and estimated value. In last meeting, some company mention that if ideal value is hard to derive, it’s also possible to define test metric based on only estimated value. Therefore, there are several possible Test metrics:

      - Test metric based on instant value delta between estimated and ideal value

      - Test metric based on mean value delta between estimated and ideal value

      - Test metric based on CDF delta between estimated and ideal value

      - Test metric based only on estimated value

For the first three metrics, it needs to discuss the feasibility of deriving ideal value. Then next, we will analyze ideal value first.

- 2.3 Analysis of ideal correlation value

For normal accuracy test, it verifies whether the estimated value can be within a threshold compared with ideal value for a fixed doppler.

In last meeting, it’s agreed that TDL-A channel will be used for verification. For TDCP test, the difficulty is that fading channel will be adopted, then ideal correlation value will vary time.

If the ideal correlation value will change within a smaller range, it’s still possible to test TDCP accuracy similar as before. Bessel function only provide the mean value. It’s still not clear about the distribution of ideal value corresponding to a specific doppler with fixed time delta. Besides, it’s also not clear what’s the difference between ideal valued derived from TDL-A channel and Bessel function.

Therefore, in the following, we analyse the two issues:

- Analyse difference of ideal value derived from TDL-A channel and ideal value derived from Bessel function

- Analyse Distribution of ideal correlation value based on TDL-A

2.3.1 Difference of ideal value between TDL-A channel and Bessel function

The ideal correlation value can be derived from TDL-A channel without noise. The CDF of ideal correlation value for different dopplers based on TDL-A channel is provided as below:

Fig.2 CDF of ideal correlation value based on TDL-A channel with 1 slot delay

We compare the ideal value derived from TDL-A model and the ideal value derived from Bessel function in the following table.

Table 5: Compare ideal value from TDL-A channel and from Bessel function

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Mean value derived from channel model | 0.9991 | 0.9910 | 0.9549 | 0.9295 | 0.8331 | 0.7685 |
| Median value derived from channel model | 0.9998 | 0.9978 | 0.9860 | 0.9742 | 0.9104 | 0.8405 |
| Value from Bessel function | 0.9998 | 0.9978 | 0.9862 | 0.9755 | 0.9037 | 0.79 |

From the table, it’s shown that ideal value from Bessel function is quite close to Median value derived from TDL-A channel since the distribution is long tail. The mean value is close either, e.g. difference smaller than 0.08 for all cases.

Observation 1: The ideal mean value or median value derived from TDL-A model is close to ideal value derived from Bessel function.

Proposal 1: Ideal mean/median value can be derived from TDL-A channel without noise. The ideal mean value or median value derived from TDL-A model is close to ideal value derived from Bessel function.

2.3.2 Distribution of ideal correlation value based on TDL-A channel

The PDF of ideal correlation value for different doppler is plotted as below (doppler = 10Hz,30Hz,200Hz, 300Hz):

Fig.3 PDF of ideal correlation value

It is shown that for doppler = 10Hz and 30Hz, the ideal correlation value distribution will concentrate to close to 1. it’s possible to use a constant value over time as ideal value.

While for doppler = 200Hz and 300Hz, the ideal value distribution will more diverse. Then it’s hard to use a constant value as ideal value over time.

Furthermore, we also provide the detail value in Table 2 based on CDF curve based on Fig.2.

Table: Ideal correlation value for different doppler based on TDL-A channel for SCS = 30KHz

|  | CDF = 5% | CDF = 10% | CDF = 50% | CDF = 90% | CDF = 95% |
| --- | --- | --- | --- | --- | --- |
| Doppler = 10 Hz | 0.9958 | 0.9979 | 0.9998 | 1.0000 | 1 |
| Doppler = 30 Hz | 0.9677 | 0.9811 | 0.9978 | 0.9997 | 0.9998 |
| Doppler = 75 Hz | 0.8023 | 0.8862 | 0.9860 | 0.9979 | 0.9987 |
| Doppler = 100 Hz | 0.6954 | 0.8018 | 0.9742 | 0.9964 | 0.9981 |
| Doppler = 200 Hz | 0.3849 | 0.5533 | 0.9104 | 0.9856 | 0.9921 |
| Doppler = 300 Hz | 0.3172 | 0.4749 | 0.8405 | 0.9699 | 0.9755 |

From the CDF distribution of ideal channel correlation amplitude, it can be observed that:

- For doppler = 10Hz, ideal correlation value span from 0.9958 to 1 with range = 0.0042 in 95% cases,

- For doppler = 30Hz, ideal correlation value span from 0.9677 to 0.9998 with range = 0.0321 in 90% cases

- For doppler = 75Hz, ideal correlation value span from 0.8023 to 0.9987 with range = 0.1964in 90% cases

- For doppler = 100Hz, ideal correlation value span from 0.7808 to 0.9981 with range = 0.2173 in 90% cases

- For doppler = 200Hz, ideal correlation value span from 0.5533 to 0.9921 with range = 0.4388 in 90% cases

- For doppler = 300Hz, ideal correlation value span from 0.4749 to 0.9755 with range = 0.5006 in 90% cases

It also shows that if doppler is larger, the ideal correlation value will span a big range. Therefore, if test the TDCP accuracy with ideal value for each instance, it’s expected that the ideal value will not change a lot during the whole time.

Therefore, low doppler is preferred for accuracy test. For example, if doppler = 10Hz is applied, the ideal doppler will within a smaller range, e.g. from 0.99 to 1 for 95% cases. Then it’s possible to set a fixed value as ideal value, Then the estimated value can be compared with the fixed value.

Observation 2: For low doppler case, e.g. smaller than 30Hz, the ideal correlation value distribution will concentrate to 1 with small variation. it’s possible to use a constant value over time as ideal value.

- For doppler = 10Hz, ideal correlation value span from 0.9958 to 1 with range = 0.0042 in 95% cases,

- For doppler = 30Hz, ideal correlation value span from 0.9677 to 0.9998 with range = 0.0321 in 90% cases

Observation 3: For high doppler case, large than 75Hz, the ideal value distribution will be more diverse. Then it’s hard to use a constant value as ideal value over time.

- For doppler = 75Hz, ideal correlation value span from 0.8023 to 0.9987 with range = 0.1964 in 90% cases

- For doppler = 100Hz, ideal correlation value span from 0.7808 to 0.9981 with range = 0.2173 in 90% cases

- For doppler = 200Hz, ideal correlation value span from 0.5533 to 0.9921 with range = 0.4388 in 90% cases

- For doppler = 300Hz, ideal correlation value span from 0.4749 to 0.9755 with range = 0.5006 in 90% cases

Based on the above observation, we propose that:

Proposal 2: It’s possible to use a constant value over time as ideal value for low doppler case, e.g. smaller than 30Hz. It’s hard to use a constant value as ideal value for high doppler case, e.g. larger than 75Hz.

- 2.4 Test metric based on instant delta between ideal and estimated value

Based on analysis in section 2.3, it’s possible to define test metric based on instant delta between ideal and estimated value for low doppler cases. The test metric is that:

Delta = instant Estimated value – Ideal value (fixed value)

    - Impact of SNR

In this section, we provide the CDF for estimation error. Here, we plot the estimation error for all dopplers including both low and high doppler. For high doppler, the ideal value is not known in real test but it can be known in simulation. We just would like to understand the impact of SNR to estimation error.

Fig.4 CDF of estimated correlation amplitude Error

Table: Estimated Error for different doppler based on TDL-A channel for SNR = 20dB

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.0040 | 0.0135 | 0.0494 |
| Doppler = 30 Hz | 0.0046 | 0.0134 | 0.0436 |
| Doppler = 75 Hz | 0.0045 | 0.0129 | 0.0398 |
| Doppler = 100 Hz | 0.0045 | 0.0126 | 0.0387 |
| Doppler = 200 Hz | 0.0041 | 0.0115 | 0.0317 |
| Doppler = 300 Hz | 0.0033 | 0.0116 | 0.0299 |

Table: Estimated Error for different doppler based on TDL-A channel for SNR = 15dB

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.0139 | 0.0408 | 0.1292 |
| Doppler = 30 Hz | 0.0152 | 0.0416 | 0.1336 |
| Doppler = 75 Hz | 0.0149 | 0.0388 | 0.1142 |
| Doppler = 100 Hz | 0.0145 | 0.0397 | 0.1076 |
| Doppler = 200 Hz | 0.0155 | 0.0374 | 0.0913 |
| Doppler = 300 Hz | 0.0141 | 0.0350 | 0.0840 |

Table: Estimated Error for different doppler based on TDL-A channel for SNR = 10dB

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.0480 | 0.1250 | 0.3456 |
| Doppler = 30 Hz | 0.0414 | 0.1158 | 0.3211 |
| Doppler = 75 Hz | 0.0473 | 0.1172 | 0.2780 |
| Doppler = 100 Hz | 0.0465 | 0.1161 | 0.2608 |
| Doppler = 200 Hz | 0.0464 | 0.1065 | 0.2204 |
| Doppler = 300 Hz | 0.0459 | 0.1018 | 0.2071 |

Table: Estimated Error for different doppler based on TDL-A channel for SNR = 5dB

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Doppler = 10 Hz | 0.1261 | 0.2767 | 0.6126 |
| Doppler = 30 Hz | 0.1300 | 0.2810 | 0.5711 |
| Doppler = 75 Hz | 0.1324 | 0.2862 | 0.5291 |
| Doppler = 100 Hz | 0.1295 | 0.2796 | 0.4937 |
| Doppler = 200 Hz | 0.1342 | 0.2587 | 0.4323 |
| Doppler = 300 Hz | 0.1247 | 0.2340 | 0.3887 |

Observation 4: With higher SNR level, instant estimated correlation error will decrease.

- At SNR = 20dB, estimated error can be less than 0.05 at CDF = 90% for all dopplers

- At SNR = 15dB, estimated error can be less than 0.14 at CDF = 90% for all dopplers

- At SNR = 10dB, estimated error can be less than 0.35 at CDF = 90% for all dopplers

- At SNR = 5dB, estimated error can be less than 0.62 at CDF = 90% for all dopplers

Considering the quantization step defined in RAN1, if TDCP test will be defined, SNR should be higher than 15dB.

Observation 5: if TDCP instant accuracy test will be defined, SNR should be higher than 15dB.

    - Threshold

In the next, we will take doppler = 10Hz as example. There are two possible ways to define the threshold:

- Based on correlation value delta without quantization

- Based on index delta after quantization

Based on correlation value delta without quantization

For doppler = 10Hz, ideal correlation value spans from 0.995 to 1 with range = 0.005 in 90% cases, then ideal value can be set for 0.995. From our simulation, the delta is 0.05 and 0.14 corresponding to CDF=90% for 20dB and 15dB respectively.

Based on index delta after quantization

the Quantization of amplitude value for TDCP reporting is as below:

| Value of q | Value of amplitude 1-2-16-q/2 |
| --- | --- |
| 0 | 0.99609375 |
| 1 | 0.994475728 |
| 2 | 0.9921875 |
| 3 | 0.988951457 |
| 4 | 0.984375 |
| 5 | 0.977903 |
| 6 | 0.96875 |
| 7 | 0.955806 |
| 8 | 0.9375 |
| 9 | 0.911612 |
| 10 | 0.875 |
| 11 | 0.823223 |
| 12 | 0.75 |
| 13 | 0.646446609 |
| 14 | 0.5 |
| 15 | 0.292893219 |

Suppose the ideal value is 0.995, corresponding to index 1.

For SNR = 15dB, the delta is 0.14 for CDF=90%. Then UE needs to report value larger than 0.86 for 90%. The corresponding index is 10. The index delta will be 9.

When SNR = 20dB, the estimation error will be smaller than 0.05 for 90%, the threshold should be larger than 0.94. The corresponding index is 8. the delta in terms of index will be 7.

It seems that the index delta is still relatively large even at SNR=20dB. We are not sure whether such big index delta is reasonable for accuracy test. We suggest RAN4 to further study the feasibility.

Observation 6: For TDCP instant accuracy test for doppler = 10Hz:

- For SNR = 20dB, if the threshold is defined in terms of value without quantization, delta=0.05 corresponding to CDF=90%. if the threshold is defined in terms of index, index delta will be 7.

- For SNR = 15dB, if the threshold is defined in terms of value without quantization, delta=0.14 corresponding to CDF=90%. if the threshold is defined in terms of index, index delta will be 9.

Proposal 3: For TDCP instant accuracy test, e.g. doppler = 10Hz, the accuracy delta with high SNR is still large compared with quantization step.

- 2.5 Test metric based on mean value delta between ideal and estimated value

    - Mean value difference

The test metric can be defined as:

Delta = estimated Mean value – ideal Mean value

In section 2.3.1, we already provide the ideal mean value derived from TDL-A channel. In the next, we will compare the mean value delta. The ideal mean value can be further aligned based on companies’ results. Or for simplicity, the value derived from Bessel function can be used.

Table: Mean value for ideal and estimated value:

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Ideal Mean value derived from channel model | 0.9991 | 0.9910 | 0.9549 | 0.9295 | 0.8331 | 0.7685 |
| Estimated mean value for SNR = 20dB | 0.9772 | 0.9707 | 0.9359 | 0.9100 | 0.8151 | 0.7522 |
| Estimated mean value for SNR = 15dB | 0.9393 | 0.9277 | 0.9036 | 0.8749 | 0.7860 | 0.7255 |
| Estimated mean value for SNR = 10dB | 0.8330 | 0.8397 | 0.8067 | 0.7888 | 0.7080 | 0.6549 |
| Estimated value for SNR = 5dB | 0.6753 | 0.6741 | 0.6467 | 0.6333 | 0.5554 | 0.5177 |

According to the above table, the value difference between ideal mean value with estimated mean value is as below:

Table: Mean value difference between ideal and estimated:

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Mean value delta for SNR = 20dB | 0.0218 | 0.0210 | 0.0190 | 0.0184 | 0.0156 | 0.0148 |
| Mean value delta for SNR = 15dB | 0.0596 | 0.0629 | 0.0537 | 0.0534 | 0.0473 | 0.0435 |
| Mean value delta for SNR = 10dB | 0.1660 | 0.1527 | 0.1450 | 0.1381 | 0.1235 | 0.1167 |
| Mean value delta for SNR = 5dB | 0.3237 | 0.3178 | 0.3097 | 0.2960 | 0.2718 | 0.2470 |

Observation 7: The mean value deltas for all dopplers are:

- For SNR = 20dB, without quantization, delta is smaller than 0.0218.

- For SNR = 15dB, without quantization, delta is smaller than 0.0629.

- For SNR = 10dB, without quantization, delta is smaller than 0.1527.

- For SNR = 5dB, without quantization, delta is smaller than 0.3237.

    - Impact of quantization

UE will report the index after quantization. We also calculated the mean value after quantization.

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Ideal Mean value derived from channel model | 0.9991 | 0.9910 | 0.9549 | 0.9295 | 0.8331 | 0.7685 |
| Quantized mean value for SNR = 20dB | 0.9727 | 0.9652 | 0.9256 | 0.8962 | 0.7953 | 0.7266 |
| Quantized mean value for SNR = 15dB | 0.9274 | 0.9143 | 0.8874 | 0.8558 | 0.7613 | 0.6985 |
| Quantized mean value for SNR = 10dB | 0.8053 | 0.8131 | 0.7798 | 0.7620 | 0.6790 | 0.6268 |
| Quantized value for SNR = 5dB | 0.6438 | 0.6395 | 0.6172 | 0.6042 | 0.5367 | 0.5032 |

We also calculate the mean value delta after quantization.

Table: Mean value difference after quantization when compared with ideal value

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Mean value delta after quantization for SNR = 20dB | 0.0262 | 0.0265 | 0.0294 | 0.0322 | 0.0354 | 0.0403 |
| Mean value delta after quantization for SNR = 15dB | 0.0715 | 0.0763 | 0.0699 | 0.0725 | 0.0721 | 0.0705 |
| Mean value delta after quantization for SNR = 10dB | 0.1937 | 0.1793 | 0.1719 | 0.1650 | 0.1526 | 0.1448 |
| Mean value delta after quantization for SNR = 5dB | 0.3552 | 0.3523 | 0.3392 | 0.3252 | 0.2905 | 0.2614 |

Observation 8: The mean value delta after quantization may increase 0.01~0.04:

- For SNR = 20dB, after quantization, delta is smaller than 0.0404

- For SNR = 15dB, after quantization, delta is smaller than 0.0725

- For SNR = 10dB, after quantization, delta is smaller than 0.1937

- For SNR = 5dB, after quantization, delta is smaller than 0.3552

- 2.6 Test metric based on CDF difference between ideal and estimated value

For example, after collecting many estimated values, it’s possible for TE to plot the CDF curve of estimated values. Then compare the estimated CDF curve with ideal CDF curve. Calculate the delta corresponding to CDF = 10%, CDF = 50% or CDF = 90% point.

For example, we plot the ideal and estimated correlation for 10Hz and 300Hz.

Fig.5 ideal and estimated CDF

Table: Doppler = 300Hz

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Ideal | 0.4749 index 14 | 0.8405 | 0.9699 index 5 |
| Estimated with SNR = 20dB | 0.4620 index 14 | 0.8216 | 0.9582 index 6 |
| Estimated with SNR = 15dB | 0.4386 index 14 | 0.7892 | 0.9411 index 7 |
| Estimated with SNR = 10dB | 0.3651 index 14 | 0.7138 | 0.8905 index 9 |
| Estimated with SNR = 5dB | 0.2283 index 14 | 0.5510 | 0.7754 index 11 |

Table: Doppler = 10Hz

|  | CDF = 10% | CDF = 50% | CDF = 90% |
| --- | --- | --- | --- |
| Ideal | 0.9979 index 0 | 0.9998 | 1.0000 index 0 |
| Estimated with SNR = 20dB | 0.9478 index 7 | 0.9862 | 0.9959 index 0 |
| Estimated with SNR = 15dB | 0.8673 index 10 | 0.9587 | 0.9860 index 3 |
| Estimated with SNR = 10dB | 0.6518 index 12 | 0.8757 | 0.9522 index 7 |
| Estimated with SNR = 5dB | 0.3883 index 14 | 0.7243 | 0.8753 index 10 |

For doppler = 300Hz, for CDF = 10% point, all the value will be mapping to index 14. There is no difference. For CDF = 90% point, there will be some index difference. For example, there will be 1,2,4,7 index delta from SNR=20dB to SNR=5dB. For CDF = 50% point, it’s median value difference.

For doppler = 10Hz, there will some delta for both low and high bound.

However, this method is a little complex since it needs to calculate the CDF distribution of estimated values. We didn’t prefer to adopt this kind of comparison.

Observation 9: it’s possible to plot the CDF curve of estimated values. Then compare the estimated CDF curve with ideal CDF curve for CDF = 10%,50% and 90% points. However, this kind of method is a little complex.

- 2.7 Test metric based only on estimated value

In last meeting, some company also mention that if ideal-value based test is not feasible, the test metric can be defined only based on estimated value.

For example, for doppler =300Hz, the estimated value will change from 0.4286 to 0.9411 with 80% probability. then it’s possible to set two thresholds corresponding to CDF=10% and CDF=90% respectively. UE need to report value within the two thresholds with 80%. When SNR decrease, two other thresholds will be derived. However, we didn’t think such threshold is meaningful. It’s hard to justify whether the estimated results are accurate or not if the threshold is defined only based on the estimated value.

Observation 10: It’s hard to justify whether the estimated results are accurate or not if the threshold is defined only based on the estimated value.

- 2.8 Summary

In this contribution, we provide analysis about ideal value and possible types of test metric.

From our point of view, it’s more meaningful that the estimated value is compared with ideal value. The test method is possible. Our concern is that no matter which metric is taken, the delta is large when compared with quantization step. We are not sure whether the test is necessary.

Proposal 4: It’s possible to compare estimated value with ideal value by different types of metrics. RAN4 to discuss the necessity to define accuracy test with large delta compared with quantization step.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: The ideal mean value or median value derived from TDL-A model is close to ideal value derived from Bessel function.

Proposal 1: Ideal mean/median value can be derived from TDL-A channel without noise. The ideal mean value or median value derived from TDL-A model is close to ideal value derived from Bessel function.

Observation 2: For low doppler case, e.g. smaller than 30Hz, the ideal correlation value distribution will concentrate to 1 with small variation. it’s possible to use a constant value over time as ideal value.

- For doppler = 10Hz, ideal correlation value span from 0.9958 to 1 with range = 0.0042 in 95% cases,

- For doppler = 30Hz, ideal correlation value span from 0.9677 to 0.9998 with range = 0.0321 in 90% cases

Observation 3: For high doppler case, large than 75Hz, the ideal value distribution will be more diverse. Then it’s hard to use a constant value as ideal value over time.

- For doppler = 75Hz, ideal correlation value span from 0.8023 to 0.9987 with range = 0.1964in 90% cases

- For doppler = 100Hz, ideal correlation value span from 0.7808 to 0.9981 with range = 0.2173 in 90% cases

- For doppler = 200Hz, ideal correlation value span from 0.5533 to 0.9921 with range = 0.4388 in 90% cases

- For doppler = 300Hz, ideal correlation value span from 0.4749 to 0.9755 with range = 0.5006 in 90% cases

Proposal 2: It’s possible to use a constant value over time as ideal value for low doppler case, e.g. smaller than 30Hz. It’s hard to use a constant value as ideal value for high doppler case, e.g. larger than 75Hz.

Observation 4: With higher SNR level, instant estimated correlation error will decrease.

- At SNR = 20dB, estimated error can be less than 0.05 at CDF = 90% for all dopplers

- At SNR = 15dB, estimated error can be less than 0.14 at CDF = 90% for all dopplers

- At SNR = 10dB, estimated error can be less than 0.35 at CDF = 90% for all dopplers

- At SNR = 5dB, estimated error can be less than 0.62 at CDF = 90% for all dopplers

Observation 5: if TDCP instant accuracy test will be defined, SNR should be higher than 15dB.

Observation 6: For TDCP instant accuracy test for doppler = 10Hz:

- For SNR = 20dB, if the threshold is defined in terms of value without quantization, delta=0.05 corresponding to CDF=90%. if the threshold is defined in terms of index, index delta will be 7.

- For SNR = 15dB, if the threshold is defined in terms of value without quantization, delta=0.14 corresponding to CDF=90%. if the threshold is defined in terms of index, index delta will be 9.

Proposal 3: For TDCP instant accuracy test, e.g. doppler = 10Hz, the accuracy delta with high SNR is still large compared with quantization step.

Observation 7: The mean value delta for all dopplers are:

- For SNR = 20dB, without quantization, delta is smaller than 0.0218.

- For SNR = 15dB, without quantization, delta is smaller than 0.0629.

- For SNR = 10dB, without quantization, delta is smaller than 0.1527.

- For SNR = 5dB, without quantization, delta is smaller than 0.3237.

Observation 8: The mean value delta after quantization may increase 0.01~0.04:

- For SNR = 20dB, after quantization, delta is smaller than 0.0404

- For SNR = 15dB, after quantization, delta is smaller than 0.0725

- For SNR = 10dB, after quantization, delta is smaller than 0.1937

- For SNR = 5dB, after quantization, delta is smaller than 0.3552

Observation 9: It’s possible to plot the CDF curve of estimated values. Then compare the estimated CDF curve with ideal CDF curve for CDF = 10%,50% and 90% points. However, this kind of method is a little complex.

Observation 10: It’s hard to justify whether the estimated results are accurate or not if the threshold is defined only based on the estimated value.

Proposal 4: It’s possible to compare estimated value with ideal value by different types of metrics. RAN4 to discuss the necessity to define accuracy test with large delta compared with quantization step.