3GPP TSG-RAN WG4 Meeting # 109				                                               R4-2318894

Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.124 | CR | 0049 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.124: EMC requirements for CA and DC combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, [Huawei] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LTE_EMC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE EMC enhancement as one of the BS/UE EMC enhancements WID objectives has been discussed. Based on the approve WF R4-2220252 and R4-230914, it could be concluded that the CA and DC is down selected has high priority features to be finished in this UE EMC WID. In RAN4#107 meeting, the test simiplification on CA and DC was discussed and A WF (R4-2309868) was approved. And the latest draft CR (R4-2316849) has been endorsed in RAN4#108bis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define test configurations for UE supporting multiple NR CA or DC band combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configurations on EMC for UE supporting multiple NR CA or DC band combinations is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

8.1	Test configurations

This clause defines the configurations for emission tests as follows:

-	the equipment shall be tested under normal test conditions;

-	the test configuration shall be as close to normal intended use as possible;

-	if the equipment is part of a system, or can be connected to ancillary equipment, then it shall be acceptable to test the equipment while connected to the minimum configuration of ancillary equipment necessary to exercise the ports;

-	if the equipment has a large number of ports, then a sufficient number shall be selected to simulate actual operation conditions and to ensure that all the different types of termination are tested;

-	the test conditions, test configuration and mode of operation shall be recorded in the test report;

-	ports which in normal operation are connected shall be connected to an ancillary equipment or to a representative piece of cable correctly terminated to simulate the input/output characteristics of the ancillary equipment, Radio Frequency (RF) input/output ports shall be correctly terminated;

-	ports that are not connected to cables during normal operation, e.g. service connectors, programming connectors; temporary connectors etc. shall not be connected to any cables for the purpose of EMC testing. Where cables have to be connected to these ports, or interconnecting cables have to be extended in length in order to exercise the EUT, precautions shall be taken to ensure that the evaluation of the EUT is not affected by the addition or extension of these cables;

-	emission tests shall be performed in two modes of operation:

-	with a communication link established (traffic mode); and

-	without a communication link established  (idle mode).

-	For UE supporting multiple NR CA or DC band combinations (including EN-DC, as well as NE DC), only one band combination with the largest frequency span (i.e. lowest frequency to highest frequency for the supported band combination) is selected for testing for each supported frequency range (i.e. FR1 only, FR1+FR2, FR2 only) for emission tests, besides radiated emission.

8.2	Radiated emission

# <End of change 1 >

# <Start of change 2>

9.1	Test configurations

This clause defines the configurations for immunity tests as follows:

-	the equipment shall be tested under normal test conditions as specified in the core specification;

-	the test configuration shall be as close to normal intended use as possible;

-	if the equipment is part of a system, or can be connected to ancillary equipment, then it shall be acceptable to test the equipment while connected to the minimum configuration of ancillary equipment necessary to exercise the ports;

-	if the equipment has a large number of ports, then a sufficient number shall be selected to simulate actual operation conditions and to ensure that all the different types of termination are tested;

-	the test conditions, test configuration and mode of operation shall be recorded in the test report;

-	ports which in normal operation are connected shall be connected to an ancillary equipment or to a representative piece of cable correctly terminated to simulate the input/output characteristics of the ancillary equipment, Radio Frequency (RF) input/output ports shall be correctly terminated;

-	ports, which are not, connected to cables during normal operation, e.g. service connectors, programming connectors, temporary connectors etc. shall not be connected to any cables for the purpose of EMC testing. Where cables have to be connected to these ports, or interconnecting cables have to be extended in length in order to exercise the EUT, precautions shall be taken to ensure that the evaluation of the EUT is not affected by the addition or extension of these cables;

-	the test arrangements for transmitter and receiver clauses of the transceiver are described separately for the sake of clarity. However, where possible the test of the transmitter clause and receiver clause of the EUT may be carried out simultaneously to reduce test time;

-	immunity tests shall be performed in two modes of operation:

-	with a communication link established (traffic mode); and

-	without a communication link established  (idle mode).

-	For UE supporting multiple NR CA or DC band combinations (including EN-DC, as well as NE DC), only one band combination with the largest frequency span (i.e. lowest frequency to highest frequency for the supported band combination) is selected for testing for each supported frequency range (i.e. FR1 only, FR1+FR2, FR2 only) for immunity tests, besides RF electromagnetic field and Electrostatic discharge.

9.2	RF electromagnetic field (80 MHz to 6000 MHz)

The test shall be performed on a representative configuration of the equipment or a representative configuration of the combination of UE and ancillary equipment.

# <End of change 2 >