3GPP TSG-RAN WG4 Meeting # 109		R4-2318909

Chicago, US, November 13 - 17, 2023

Agenda Item:	8.34.3.1

Source: 	CMCC

Title: 			 Discussion on RRM impact of cell DTX/DRX and CHO for network energy saving

Document for:	Discussion

- Introduction

In last meeting, RAN4’s achieved initial agreements for the RRM impact of cell DTX/DRX, power/spatial domain mechanism and CHO, the WF has been approved [1]. In this contribution, we keep discussing the RRM impact based on further output from RAN1 and RAN2.

- Discussion

Issue 2-1: RRM impacts of Cell DTX/DRX – general

Agreement:

- There is no RRM impacts for cell DTX assuming that following RS are not impacted by Cell DTX

  - PRS

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking)

  - Periodic/Semi-persistent CSI-RS (for BM)

- FFS whether there is impact on SCell activation requirements of Cell DTX.

Based on our understanding, it is not typical that network configure SSB-less SCell activation and SSB-less SCell DTX simultaneously. Therefore, there is no need to further consider the impact on SCell activation requirements of cell DTX.

Observation 1: It is not typical that network configure SSB-less SCell activation and SSB-less SCell DTX simultaneously.

Proposal 1: No need to further consider the impact on SCell activation requirements of cell DTX.

Issue 2-3: RRM impacts of CHO

- Proposals

  - Option 1: RAN4 to wait RAN2’s further progress on CHO enhancement in NES. (Ericsson, Intel)

    - Option 1a: RAN4 is to discuss new triggering of CHO in core requirements and test cases according to further RAN2 conclusions. (Intel)

    - Option 1b: RAN4 to study the impact of CHO procedure due to cell switch off. (Ericson)

RAN2 discussed the NES mode notification and NES CHO configuration details, following agreements has been achieved so far [2]:

Agreements Group common DCI format 2-X is reused to notify the UE that source cell is entering NES mode. - add one bit of DCI 2-X to trigger both use cases of Cell DTX/DRX activation and cell turning off. RAN2 send LS to RAN1 to request this signaling change. RAN2 will not specify anything related to target cell NES mode for CHO Open issue

Based on our understanding, the NES CHO procedure after UE beginning to evaluate the CHO conditions is basically same as legacy CHO. However, it is not yet clear when UE will start the evaluation of NES CHO condition, which may depends on the network configuration order of the NES mode notification (DCI format 2-X) and NES CHO configuration (RRC signaling).

Therefore, we propose to use the legacy CHO requirement as the starting point and wait more output from RAN2.

Proposal 2: Use the legacy CHO requirement as the starting point and wait more output from RAN2.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: It is not typical that network configure SSB-less SCell activation and SSB-less SCell DTX simultaneously.

Proposal 1: No need to further consider the impact on SCell activation requirements of cell DTX.

Proposal 2: Use the legacy CHO requirement as the starting point and wait more output from RAN2.

- Reference

[1] R4-2317405, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.

[2] Chair notes for RAN2#123bis, RAN2.