# 3GPP TSG-RAN WG4 Meeting # 109	R4-2318915			R4-23xxxxx

# Chicago, US, November 13 – 17, 2023

Agenda item:	8.28.4

Source: 	CMCC

Title: 	Discussion on NCR conformance testing

Document for:	Discussion

# 1. Introduction

In last meeting, A WF on NCR conformance testing requirement has been approved[1]. Previous approved WF are also listed for information. In this contribution, we continue discuss remaining issues.

Issue 1-1: Mixed type introduction - Agreement: - The Mixed type of NCR will not be introduced in Rel-18. Issue 3-1: Test configuration - Option 1: - Place an NCR-MT carrier at the lower end of each passband Generate an NR carrier using test equipment at the upper edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband. - Place an NCR-MT carrier at the upper end of each passband. Generate an NR carrier using test equipment at the lower edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband. - Other Options are not precluded.

- Discussion

- We focus on 2.1 test mode and 2.2 test configuration for simultaneous Tx between MT and forwarding UL.

- 2.1 Test mode

Previous agreements are listed as below for information :

- Issue 3-1: NCR-MT test model - For test model for NCR-MT, propose to use the Rel-16 IAB-MT test model as starting point for further study.

Following table list the IAB-MT testing mode.

Table 1: IAB-MT test mode

| Test mode | Applicable RF requirements | Output power | Configured RB number |
| --- | --- | --- | --- |
| IAB-MT-FR1-TM1.1 | IAB output power Transmit ON/OFF power Unwanted emissions - Occupied bandwidth - ACLR - Operating band unwanted emissions - Transmitter spurious emissions Transmitter intermodulation Receiver spurious emissions | - | NRB with QPSK |
| IAB-MT-FR1-TM2 | Total power dynamic range (at lower PSD TX power limit at min power) Transmitted signal quality - EVM of single 64QAM PRB allocation (at lower PSD TX power limit at min power) - Frequency error (at min power) | At min power At lower PSD Tx power limit at min power | 1 PRB with 64QAM |
| IAB-MT-FR1-TM2a | - EVM of single 256QAM PRB allocation (at min power) - Frequency error (at min power) | At min power | 1 PRB with 256QAM |
| IAB-MT-FR1-TM3.1 | Output power dynamics - Total power dynamic range (upper TX PSD power limit at max power with all 64QAM PRBs allocated) Transmitted signal quality - Frequency error (at max power) - EVM for modulation (at max power) | At max power Upper Tx PSD power limit at max power | NRB |
| IAB-MT-FR1-TM3.1a | Output power dynamics - Total power dynamic range (upper TX PSD power limit at max power with all 256QAM PRBs allocated) Transmitted signal quality - Frequency error (at max power) - EVM for 256QAM modulation (at max power) | at max power upper TX PSD power limit at max power | NRB |

According to above table, NCR-MT Operating band unwanted emissions requirement will only be tested when all PRB are configured. For LA NCR-MT, it will reuse UE SEM requirements and according to 38.521-1, UE SEM requirements will test Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocations. It’s worth noting that usually the worst SEM case is when RB is configured at edge_1PB with upper PSD limit. Besides for NCR-MT, usually the allocated PRB is less due to less data load. Therefore, edge_1PRB allocation should also be considered for LA NCR-MT SEM testing.

Proposal 1: at least LA NCR-MT SEM requirements should also be tested under edge_1PRB_left and edge_1PRB_right RB allocations with max Tx power. Additional test mode beside IAB testing modes should be added.

- 2.2 Test configuration when NCR-MT and fwd-link transmit simultaneously

- For the test modes when NCR-MT is configured within all RBs per carrier, option 1 as show below is OK to show the aggregate emission from MT and fwd-link UL.

  - Place an NCR-MT carrier at the lower end of each passband Generate an NR carrier using test equipment at the upper edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

  - Place an NCR-MT carrier at the upper end of each passband. Generate an NR carrier using test equipment at the lower edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

- Proposal 2: for the test modes when NCR-MT is configured within all RBs per carrier, option 1 in last meeting WF is OK to show the aggregate emission from MT and fwd-link UL.

- For the test mode as proposed in section 2.2 when only edge_1RB_left or edge_1RB_right RB is allocated, we also need to consider whether we can put the NCR-MT and NCR-fwd-link into the same carrier. According to RAN1 spec, FDMed operation between MT and fwd-link is supported, i.e. MT occupies some PRB and fwd-link occupies the remaining PRB within the same carrier. This same-carrier test configuration is the worst case for emission related requirements testing.

- Following fig show the detailed test configuration.

- Proposal 3: for the test modes when NCR-MT is configured at edge 1PRB(if specified), it’s suggested to further discuss the test configuration when MT occupies edge 1PRB and fwd-link occupies remaining PRB within the same carrier. Besides, another fwd-link carrier configured at the other side of pass-band edge. Detailed show as in above fig.

# 3. Conclusions

In this contribution, NCR are discussed with following observations and proposals.

Proposal 1: at least LA NCR-MT SEM requirements should also be tested under edge_1PRB_left and edge_1PRB_right RB allocations with max Tx power. Additional test mode beside IAB testing modes should be added.

- Proposal 2: for the test modes when NCR-MT is configured within all RBs per carrier, option 1 in last meeting WF is OK to show the aggregate emission from MT and fwd-link UL.

  - Place an NCR-MT carrier at the lower end of each passband Generate an NR carrier using test equipment at the upper edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

  - Place an NCR-MT carrier at the upper end of each passband. Generate an NR carrier using test equipment at the lower edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

- Proposal 3: for the test modes when NCR-MT is configured at edge 1PRB(if specified), it’s suggested to further discuss the test configuration when MT occupies edge 1PRB and fwd-link occupies remaining PRB within the same carrier. Besides, another fwd-link carrier configured at the other side of pass-band edge. Detailed shown as below.

# 4. Reference

- R4-2305933,WF on NR NCR RF conformance testing, CATT, approved.

- R4-2309864, WF for NCR RF conformance, CATT, approved

- R4-2313906,WF for NCR conformance testing, CATT, Approved.

- R4-2316908, WF on NCR conformance, CATT, approved.