# 3GPP TSG-RAN WG4 Meeting # 109				R4-2318918

# Chicago, US, November 13 – 17, 2023

Agenda item:	10

Source: 	CMCC

Title: 	Rel-18 RAN4 ATG UE feature list for NR

Document for:	Approval

- Introduction

This contribution includes the RAN4 ATG UE feature list for Rel-18 NR.

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-1 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Indicate the support of enhanced inter-frequency cell re-selection requirements for ATG (as specific in TS 38.133 Table 4.2D.2.4-2) |  | no | N/A | If UE does not support the capability, legacy measurement requirements (as specified in TS 38.133, Table 4.2D.2.4-1) are applied. | - Per UE | No | FR1 only | N/A |  | Optional without capability signaling |
|  | 35-2 | Omni-directional antenna type | Indicate the support of RF and RRM requirements with omni-directional antenna as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. |  | yes | N/A | If UE does not support omni-directional antenna type, the corresponding requirements cannot be guaranteed. | - Per Band | No | FR1 only | N/A | For one band, if UE does not report support of omni-directional antenna type, UE should report support of antenna array type. | Optional with capability signaling |
|  | 35-3 | Antenna array type | Indicate the support of RF and RRM requirements with antenna array as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. |  | yes | N/A | If UE does not support antenna array type, the corresponding requirements cannot be guaranteed. . | 2)Per Band | No | FR1 only | N/A | For one band, if UE does not report support of antenna array type, UE support of omni-directional antenna type. | Optional with capability signaling |
|  | 35-4 | Rated max output power | Indicate the support of rated maximum output power at maximum modulation order and full PRB configurations |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s maximum output power. | 2)Per band | No | FR1 only | N/A | Value range from 23dBm to 40dBm with 1dB as granularity | Mandatory with capability signaling |
|  | 35-5 | ATG specific P-max | Indicate the support of ATG specific P-max configured by network. |  | no | N/A | If UE does not support ATG specific P-max value, ATG UE can’t identify configured maximum output power PCMAX,f,c | - Per UE | No | FR1 only | N/A | Value range from -21dBm to 42dBm | Mandatory without capability signaling |

For RRM feature 35-1: the functionality of feature has been agreed, but the detail of the UE feature still requires discussion in ATG WI.

For RF and RRM feature 35-2 and 35-3: these features have been approved in previous meeting and have sent the LS to RAN2.

For RF feature 35-4: the feature has been approved in previous meeting and have sent the LS to RAN2. But no discussion of the feature type (9th column), please further check whether this capability is per band basis or not.

For RF feature 35-5, the feature have been approved in previous meeting and sent the LS to RAN2. but value range granularity has not been determined by RAN2.