3GPP TSG-RAN WG4 Meeting # 109	R4-2318936

# Chicago, US, November 13 – 17, 2023

Agenda item:	8.21.3

Source: 	Qualcomm Incorporated

Title: 	On AI/ML RAN4 Interoperability

Document for:	Approval

# Introduction

We present our view on the scope of AI/ML work item in this contribution.

# Discussion

## Test Model/Decoder in Two Sided Model Test: Test decoder options clarification

Since collaborate training procedure is offline for all the options discussed, we propose to defer the supported training type discussion to WI stage.

Proposal 1: Supported training type discussion is deferred to WI stage.

For decoder verification procedure, we can clarify it under the context of test repeatability agreement made in the previous meeting.

Proposal 2: For option 3, no decoder verification procedure is needed since the test decoder is standardized. For option 4, decoder verification procedure can be purposed to verify the test repeatability (variation among TE vendor implementations are bounded). If the test repeatability is applicable to option 2 (and 1), the same clarification applies.

We propose the clarification of “Number of test per test configuration/setup (propagation condition, CSI configuration etc excluding decoder/network side model configuration)” for the test decoder options, however, this item becomes irrelevant when the test repeatability can be ensured.

Proposal 3: Defer the discussion of “Number of test per test configuration/setup” until RAN4 confirmed that test repeatability in the selected option can’t be fully guaranteed.

## Test Model/Decoder in Two Sided Model Test: Test decoder pros& cons analysis

For “reflection on the real deployment (knowledge of model, training type, etc.)”, we propose to analyse from the following perspectives:

Proposal 4: “reflection on the real deployment” can be discussed in following perspectives:

- Knowledge of the test decoder

- Whether the decoder vendor can implement the test decoder

- Do not discuss the training type perspective if the clarification of training types is deferred to WI stage

For “TE requirements to deploy the decoder”, we observed following:

Proposal 5: “TE requirements to deploy the decoder” for each option are analyzed in the following:

- Option 1: highest requirements for TE since TE needs to support multiple test decoders own by other entities for each test configuration.

- Option 2: higher requirements for TE since TE needs to support one or multiple test decoders own by other entities for each test configuration.

- Option 3: lower requirements for TE since TE only needs to support one standardized test decoder for each test configuration

- Option 4: lowest requirements for TE since TE only needs to support one test decoder designed by itself for each test configuration

For “Specification Effort”, RAN4 needs to first identify what types of potential specification efforts are needed, then analyze which options require what effort instead of subjectively judging high or low efforts. Therefore, we propose the following methodology:

Proposal 6: “Specification Effort” for each option are analyzed based on the necessity of the following different types of specification efforts:

- Effort to specify high level parameters

- Effort for designing specifications to ensure test repeatability

- Effort for designing specifications to ensure ability to implement decoders with similar performance by other vendors

- Effort to specify decoder performance verification procedure

- Effort to fully specify the decoder

To better analyze the specification efforts in each category, we provide examples/options for how to derive the specification in some of the categories.

Proposal 7: We provide examples/potential options for derivation of the specification in some of the above categories for test decoder options:

- Effort for designing specifications to ensure test repeatability and ability to implement decoders with similar performance by other vendors

  - E.g. (or option A): Capture dataset with (nominal encoder input, latent message as encoder output) in the specification for TE vendors to train the decoder, to ensure test repeatability and ability for other vendors to train the decoder with similar performance.

    - FFS the definition of nominal encoder input, e.g., eigen vectors

    - FFS how to derive the dataset

- Effort to fully specify the decoder

  - E.g. (or option A): Decide one reference encoder structure, and each company perform joint training of its encoder/decoder pair based on the reference structures and the agreed high-level parameters. RAN4 then pick an encoder/decoder pair based on the agreed evaluation and selection criterion

    - FFS evaluation and selection criterion

| Specification effort category | Option 1: DUT provided decoder | Option 2: NW provided decoder | Option 3: Fully specified decoder | Option 4: TE provided decoder |
| --- | --- | --- | --- | --- |
| High level parameters | Yes | Yes | Yes | Yes |
| Ensure test repeatability | (To be discussed) | (To be discussed) | No | Yes, E.g. (or option A): Capture dataset with (nominal encoder input, latent message as encoder output) in the specification for TE vendors to train the decoder, to ensure test repeatability and ability for other vendors to train the decoder with similar performance. FFS the definition of nominal encoder input, e.g., eigen vectors FFS how to derive the dataset |
| Ensure ability to implement decoders with similar performance by other vendors | (To be discussed) | (To be discussed) | No |  |
| Decoder performance verification procedure | (To be discussed) | (To be discussed) | No | (To be discussed) |
| Full specification of the test decoder | No | No | Yes, E.g. (or option A): Decide one reference encoder structure, and each company perform joint training of its encoder/decoder pair based on the reference structures and the agreed high-level parameters. RAN4 then pick an encoder/decoder pair based on the agreed evaluation and selection criterion FFS evaluation and selection criterion | No |

Given that the RAN4 specification efforts are captured in the previous item, “Complexity of actual testing procedure for the ecosystem” should cover the procedures after RAN4 specified the test decoder, and focus on the “additional” efforts that don’t exist in the legacy test procedures. Therefore, we have the following proposal:

Proposal 8: “Complexity of actual testing procedure for the ecosystem”  for each option are analyzed based on the following principle:

Comparing the options with legacy test procedure, evaluate additional works/steps on other entities after RAN4 finish specification until finish certification

      - RAN5 specification

      - TE vendor implementation considerations

      - DUT vendor test execution

      - Network (other vendor) test execution

For the reference encoder/decoder specification, based on the understanding that it is for simulation alignment purpose, we have the following proposal of principles on the specification:

Proposal 9: The reference encoder (w.r.t. the test decoder) and the reference decoder (w.r.t. the test encoder) specification discussion for simulation alignment purpose follows the principle:

During the simulation assumption discussion in WI stage, RAN4 can choose one of the following options of reference encoder/decoder based on the alignment across collected preliminary simulation results

      - Option 1: Apply agreed common assumptions

      - Option 2: Introduce additional simulation assumptions in addition to agreed common assumptions

      - Option 3: (fully) specify the reference encoder/decoder

We summarize our proposal in the following table:

|  | Option 1: DUT provides decoder | Option 2: Decoder not from DUT and Spec | Option 3: Full decoder specification in standard | Option 4: TE specified decoder |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) |  |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | No need to discuss |  |  |  |
| Test decoder verification procedure at TE and/or DUT | [To verify the test repeatability (variation among TE vendor implementations are bounded) of the test decoder] | [To verify the test repeatability (variation among TE vendor implementations are bounded) of the test decoder] | No need | To verify the test repeatability (variation among TE vendor implementations are bounded) of the test decoder |
| Feasibility of test decoder verification procedure |  |  |  |  |
| Number of test per test configuration/setup (propagation condition, CSI configuration etc excluding decoder/network side model configuration) | No need to discuss if test repeatability can be ensured as agreed in the previous meeting |  |  |  |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment - Knowledge of the test decoder - Whether the decoder vendor can implement the test decoder | - Not reflected - Depends on what is specified | - Is reflected - Can implement within the same vendor | - Not reflected - Can implement | - Is reflected - Depends on what is specified |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | Highest requirements for TE since TE needs to support multiple test decoders own by other entities for each test configuration. | Higher requirements for TE since TE needs to support one or multiple test decoders own by other entities for each test configuration. | Lower requirements for TE since TE only needs to support one standardized test decoder for each test configuration | Lowest requirements for TE since TE only needs to support one test decoder designed by itself for each test configuration |
| Specification Effort (e.g. test decoder) | See proposal 6,7 |  |  |  |
| Confidentiality/ IP issues | Yes | Yes | No | No |
| Applicability to different scenarios/conditions/ configurations | No difference across scenarios |  |  |  |
| Complexity of actual testing procedure for the ecosystem | Comparing the options with legacy test procedure, evaluate additional works/steps on other entities after RAN4 finish specification until finish certification - RAN5 specification - TE vendor implementation considerations - DUT vendor test execution - Network (other vendor) test execution |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented | Depends on agreed high level parameters and whether/how to ensure test repeatability and the ability to implement decoders with similar performance by other vendors |  | May not have forward compatibility | Depends on agreed high level parameters and how to ensure test repeatability and the ability to implement decoders with similar performance by other vendors |
| Relationship with reference decoder/encoder for defining requirement | See proposal 9 |  |  |  |

## Test Model/Decoder in Two Sided Model Test: Test decoder feasibility analysis

We analyze the feasibility of the test decoder options. For option 1 and 2, since the test decoder is provided by UE and the network vendor, and the options are feasible if the loaded file representing test decoder isn’t subject to IP constraint and TE is able to load them correctly.

Proposal 10: Option 1 and 2 are feasible if the loaded file representing test decoder isn’t subject to IP constraint and TE is able to load them correctly.

Option 3 is feasible if RAN4 can achieve the agreements of the fully specified test decoders since there is no IP issue and TE requirement is less of a concern.

Proposal 11: Option 3 is feasible since there is a proper decoder derivation procedure as described in proposal 7 for pro & cons analysis, without IP issue and low TE requirements.

Option 4 doesn’t have IP and TE loading other decoder issue. Therefore, option 4 is feasible if the agreed principle of test repeatability and the ability to implement decoders with similar performance by other vendors can be ensured. We provide an option achieving the principle in the following proposal.

Proposal 12: Option 4 is feasible since it doesn’t have IP issue and with lowest TE requirement. In additional, the two additional properties, test repeatability and implement ability of other vendors, can be achieved at least by the example described in proposal 7 for pro & cons analysis.

# Conclusion

Since most of the proposals are covered in the table, we paste the table and proposals not covered in the table in the conclusion.

|  | Option 1: DUT provides decoder | Option 2: Decoder not from DUT and Spec | Option 3: Full decoder specification in standard | Option 4: TE specified decoder |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) |  |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | No need to discuss |  |  |  |
| Test decoder verification procedure at TE and/or DUT | [To verify the test repeatability (variation among TE vendor implementations are bounded) of the test decoder] | [To verify the test repeatability (variation among TE vendor implementations are bounded) of the test decoder] | No need | To verify the test repeatability (variation among TE vendor implementations are bounded) of the test decoder |
| Feasibility of test decoder verification procedure |  |  |  |  |
| Number of test per test configuration/setup (propagation condition, CSI configuration etc excluding decoder/network side model configuration) | No need to discuss if test repeatability can be ensured as agreed in the previous meeting |  |  |  |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment - Knowledge of the test decoder - Whether the decoder vendor can implement the test decoder | - Not reflected - Depends on what is specified | - Is reflected - Can implement within the same vendor | - Not reflected - Can implement | - Is reflected - Depends on what is specified |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | Highest requirements for TE since TE needs to support multiple test decoders own by other entities for each test configuration. | Higher requirements for TE since TE needs to support one or multiple test decoders own by other entities for each test configuration. | Lower requirements for TE since TE only needs to support one standardized test decoder for each test configuration | Lowest requirements for TE since TE only needs to support one test decoder designed by itself for each test configuration |
| Specification Effort (e.g. test decoder) | See proposal 6,7 |  |  |  |
| Confidentiality/ IP issues | Yes | Yes | No | No |
| Applicability to different scenarios/conditions/ configurations | No difference across scenarios |  |  |  |
| Complexity of actual testing procedure for the ecosystem | Comparing the options with legacy test procedure, evaluate additional works/steps on other entities after RAN4 finish specification until finish certification - RAN5 specification - TE vendor implementation considerations - DUT vendor test execution - Network (other vendor) test execution |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented | Depends on agreed high level parameters and whether/how to ensure test repeatability and the ability to implement decoders with similar performance by other vendors |  | May not have forward compatibility | Depends on agreed high level parameters and how to ensure test repeatability and the ability to implement decoders with similar performance by other vendors |
| Relationship with reference decoder/encoder for defining requirement | See proposal 9 |  |  |  |

Proposal 6: “Specification Effort” for each option are analyzed based on the necessity of the following different types of specification efforts:

- Effort to specify high level parameters

- Effort for designing specifications to ensure test repeatability

- Effort for designing specifications to ensure ability to implement decoders with similar performance by other vendors

- Effort to specify decoder performance verification procedure

- Effort to fully specify the decoder

To better analyze the specification efforts in each category, we provide examples/options for how to derive the specification in some of the categories.

Proposal 7: We provide examples/potential options for derivation of the specification in some of the above categories for test decoder options:

- Effort for designing specifications to ensure test repeatability and ability to implement decoders with similar performance by other vendors

  - E.g. (or option A): Capture dataset with (nominal encoder input, latent message as encoder output) in the specification for TE vendors to train the decoder, to ensure test repeatability and ability for other vendors to train the decoder with similar performance.

    - FFS the definition of nominal encoder input, e.g., eigen vectors

    - FFS how to derive the dataset

- Effort to fully specify the decoder

  - E.g. (or option A): Decide one reference encoder structure, and each company perform joint training of its encoder/decoder pair based on the reference structures and the agreed high-level parameters. RAN4 then pick an encoder/decoder pair based on the agreed evaluation and selection criterion

    - FFS evaluation and selection criterion

| Specification effort category | Option 1: DUT provided decoder | Option 2: NW provided decoder | Option 3: Fully specified decoder | Option 4: TE provided decoder |
| --- | --- | --- | --- | --- |
| High level parameters | Yes | Yes | Yes | Yes |
| Ensure test repeatability | (To be discussed) | (To be discussed) | No | Yes, E.g. (or option A): Capture dataset with (nominal encoder input, latent message as encoder output) in the specification for TE vendors to train the decoder, to ensure test repeatability and ability for other vendors to train the decoder with similar performance. FFS the definition of nominal encoder input, e.g., eigen vectors FFS how to derive the dataset |
| Ensure ability to implement decoders with similar performance by other vendors | (To be discussed) | (To be discussed) | No |  |
| Decoder performance verification procedure | (To be discussed) | (To be discussed) | No | (To be discussed) |
| Full specification of the test decoder | No | No | Yes, E.g. (or option A): Decide one reference encoder structure, and each company perform joint training of its encoder/decoder pair based on the reference structures and the agreed high-level parameters. RAN4 then pick an encoder/decoder pair based on the agreed evaluation and selection criterion FFS evaluation and selection criterion | No |

Proposal 9: The reference encoder (w.r.t. the test decoder) and the reference decoder (w.r.t. the test encoder) specification discussion for simulation alignment purpose follows the principle:

During the simulation assumption discussion in WI stage, RAN4 can choose one of the following options of reference encoder/decoder based on the alignment across collected preliminary simulation results

      - Option 1: Apply agreed common assumptions

      - Option 2: Introduce additional simulation assumptions in addition to agreed common assumptions

      - Option 3: (fully) specify the reference encoder/decoder

Proposal 10: Option 1 and 2 are feasible if the loaded file representing test decoder isn’t subject to IP constraint and TE is able to load them correctly.

Proposal 11: Option 3 is feasible since there is a proper decoder derivation procedure as described in proposal 7 for pro & cons analysis, without IP issue and low TE requirements.

Proposal 12: Option 4 is feasible since it doesn’t have IP issue and with lowest TE requirement. In additional, the two additional properties, test repeatability and implement ability of other vendors, can be achieved at least by the example described in proposal 7 for pro & cons analysis.