3GPP TSG-RAN WG4 Meeting # 109	R4-2318951

# Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	Views on draft CR for new TxD capability adaptation

Agenda Item:	8.5.2.1

Document for:	Approval

Introduction

In RAN4#108bis, a LS regarding a new TxD capability signalling were approved in [1] with the target to improve the support of inter-band CA + TxD. Another similar LS for 4Tx was also approved in [2].

This paper discusses how this would impact spec and how to introduce it.

Discussion

As a new capability for 2Tx TxD is introduced, theoretically all the parts that using legacy TxD capability would have an impact. A table of all the current TxD capability use cases has been provided here, which is the same to [3].

Table 1. Current txDiversity-r16 use cases in Rel-18 38.101-1

| Clauses | Contents | Note |
| --- | --- | --- |
| Clause 4.2 Applicability of minimum requirements | e) The requirements for Tx diversity in this release are applied for UE which indicates IE [txDiversity-r16] |  |
| Table 6.2.3.1-1: Additional maximum power reduction (A-MPR) | NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. |  |
| Clause 6.2.4 Configured transmitted power | ΔPPowerClass =3dB is applied when PC2 capable UE with txDiversity-r16 capability or PC1.5 capable UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’; |  |
|  | The value of ∆TRxSRS is 4.5dB for bands …, or when UE indicating txDiversity-r16 |  |
|  | The value of ∆TRxSRS is 7.5dB for bands … and not indicating txDiversity-r16 |  |
| Clause 6.2D.1 UE maximum output power for UL MIMO | If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or……. with the following exception: for UEs indicating txDiversity-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass. |  |
| 6.2G.1 UE maximum output power for Tx Diversity | When a UE indicates PC1.5 for a given band it achieves maximum power by means of Tx Diversity in the current version of the spec. Therefore, Tx Diversity is implied for PC1.5 even if the UE does not indicate txDiversity-r16 in the UE capabilities |  |
| 6.5G.2 Out of band emission for Tx Diversity | If UE indicates IE txDiversity-r16, Adjacent Channel Leakage power Ratio (ACLR) is defined as |  |
| F.8 EVM measurement for dual Tx | For UE with dual transmission antennas, if UE indicates IE [txDiversity-r16], |  |

However, since this issue is also being discussed in FR1 enh WI for 4Tx. In order to avoid duplicate analysis and possible overlapping, it is preferred to treat all the need revisions in one WI, since the capabilities are closely together.

Observation: TxD capabilities related revision can be treated in FR1 enh WI with 4Tx TxD capability to avoid duplicate analysis and possible overlapping.

Proposal: Not to consider TxD capabilities related revision in the CR of this WI.

Conclusion

In this paper, CR impact of new TxD capability signalling is discussed.

Observation: TxD capabilities related revision can be treated in FR1 enh WI with 4Tx TxD capability to avoid duplicate analysis and possible overlapping.

Proposal: Not to consider TxD capabilities related revision in the CR of this WI.

- References

[1] R4-2317762, LS on new per band per BC TxD capability, Huawei, RAN4#108bis

[2] R4-2317617, LS on signalling for 4Tx TxD, vivo, RAN4#108bis

[3] R4-2318946, Remaining issues of 4Tx requirements, vivo, RAN4#109