# 3GPP TSG-RAN WG4 Meeting # 109				R4-2318976

# Chicago, US, November 13 – 17, 2023

Agenda item:	10

Source: 	vivo

Title: 	Discussions on Rel-18 RAN4 UE feature list

Document for:	Discussion

- Introduction

In this contribution, we share our views on RAN4 UE feature list for Rel-18 NR.

- NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_ENDC_RF_FR1_enh2 | 27-1 | TxD capability | TxD support capability for 4Tx |  | YES | NO | 4Tx TxD can not be supported and verified | TBD | No | FR1 only |  |  | Optional with capability signalling |

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_RF_FR2_req_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_RF_FR2_req_Ph3 | 29-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Simultaneous reception of NR PDCCH/PDSCH overlapping with layer 1 RS with different QCL Type-D | Supports simultaneous reception of PDCCH/PDSCH with different QCL Type-D layer 1 RS for measurement on overlapping OFDM symbols. | 16-2c | Yes | N/A |  | Per band | N/A | FR2 only |  |  | Optional with capability signalling |
| 30. NR_FR2_multiRX_DL | 30-2 | Simultaneous measurement of layer 1 RS overlapping with another layer 1 RS with different QCL Type-D | Supports Simultaneous measurement of layer 1 RS overlapping with another layer 1 RS with different QCL Type-D on overlapping OFDM symbol(s). | 16-2c | Yes | N/A |  | Per band | N/A | FR2 only |  |  | Optional with capability signalling |
| 30. NR_FR2_multiRX_DL | 30-3 | Fast beam sweeping | Supports beam sweeping factor reduction for SSB-based layer 1 measurement. |  | Yes | N/A |  | Per band | N/A | FR2 only |  | Candidate values for Component 2: {2,4,6} for FR2-1 | Optional with capability signalling |

- NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | L3 measurement report for unknown SCell activation | Support of reporting valid L3 measurement results for the target being-activated SCell after receiving the SCell activation command |  | Yes | N/A | UE does not support reporting valid L3 measurement results after receiving the SCell activation command | Per UE | No | Yes | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signalling |
| 31. NR_RRM_enh3 | 31-2 | Beam sweeping factor reduction for FR2 unknown SCell activation | Support of reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure Support of reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure |  | Yes | N/A | UE does not support beam sweeping factor reduction for cell detection during FR2-1 unknown SCell activation. UE does not support beam sweeping factor reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation. | Per Band per BC | N/A | FR2 only | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. Candidate values for beam sweeping reduction for cell detection during FR2-1 unknown SCell activation are 1,2,4, or 6. [Agreed in WF R4-2310081] Candidate values for beam sweeping reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation are 0,1,2,3,4,5,6, or 7. [Agreed in WF R4-2310081] | Optional with capability signalling |
| 31. NR_RRM_enh3 | 31-3 | Other measurement enhancement for unknown SCell activation | Support of using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement. Support of performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation |  | Yes | N/A | UE does not use SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in MO for enhanced unknown Scell activation requirement. UE does not support performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation | Per UE | No | Yes | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signalling |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_HST_FR2_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_HST_FR2_enh | 34-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | 36-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | Switching period reporting enhancement | Additional switching period reporting for unaffected band involved case |  | YES | NO |  | Per BC | No | FR1 only | NO |  | Optional with capability signalling |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 39. NR_Mob_enh2 MeasIdleFR2-CarrierNR-r18 | 39-1 | Support of FR2 measurements during RRC connection setup/resume | - Support of FR2 measurements during RRC connection setup/resume Support of continuing FR2 measurements after RRC connection setup/resume |  | YES | NO | 1.UE doesn’t support of FR2 measurements during RRC connection setup/resume | Per UE | No | FR2 only | NO |  | Optional with capability signalling |
| 39. NR_Mob_enh2 fr2-MeasIdleAvailable-r18 | 39-2 | Support availability indication of IDLE/INACTIVE mode FR2 measurements | Support availability indication of IDLE/INACTIVE mode FR2 measurements for UE which is not capable of idleInactiveNR-MeasBeamReport-r16 |  | YES | NO | 1. UE doesn’t support availability indication of IDLE/INACTIVE mode FR2 measurements if UE is not capable of idleInactiveNR-MeasBeamReport-r16 | Per UE | No | FR2 only | NO |  | Optional with capability signalling |
| 39. NR_Mob_enh2 | 39-3 | Support of reporting L3 measurement results in L1 reporting | 1. Support reporting L3 measurement results in L1 reporting. |  | YES | No | 1. UE can not report L3 measurement results in L1 reporting | Per UE | No | No | NO |  | Optional with capability signalling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh parallelMeasurementWithoutRestriction-fr2-r18 | 40-1 | Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e., data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. [The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-18).] |  | YES | NO | UE doesn’t support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation of serving cell without scheduling restrictions. | Per Band | FDD only | FR2 only | NO |  | Optional with capability signalling |
| 40. NR_NTN_enh maxNumber-NGSO-SatellitesWithinOneSMTC-fr2-r18 | 40-2 | Parallel measurements on multiple NGSO satellites within a SMTC | Support of simultaneously measurements on target cells [belonging to different NGSO satellites within a SMTC] |  | YES | NO | UE doesn’t support of simultaneously measurements on target cells | Per Band | FDD only | FR2 only | NO |  | Optional with capability signalling |
| 40. NR_NTN_enh UEType MechanicalSteering-r18 | 40-3 | Support NTN UE capable of VSAT communicating with mechanical steering antenna | Support NTN UE capable of VSAT communicating with mechanical steering antenna |  | YES | NO | UE is not capable of VSAT communicating with mechanical steering antenna | Per UE | FDD only | FR2 only | NO |  | Optional with capability signalling |

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. # NR_cov_enh2 | 41-1 | Power boosting for QPSK for power class 3 UE | Power boosting for QPSK for power class 3 UE in TDD bands with duty cycle less than a certain value. |  | YES | NO | Power class 3 UE does not support power boosting for QPSK in TDD bands with duty cycle less than a certain value. | Per Band | TDD only | FR1 only | NO |  | Optional with capability signalling |
|  | 41-2 | Delta_PowerClass reporting | Reporting ΔPPowerClass , ΔPPowerClass, CA parameter valuesto network |  | YES | NO | Network can’t know ΔPPowerClass related parameters | TBD | TDD only | FR1 only | NO |  | Optional with capability signalling |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. # Netw_Energy_NR | 42-1 | SSB-less SCell operation for inter-band or intra-band non-contiguous CA | Support of SSB-less SCell operation for inter-band or intra-band non-contiguous CA under co-located deployment |  | Yes | No | UE does not support SSB-less operation | Per-FS | No | FR1 only | No |  | Optional with capability signalling |

- NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. # NR_DualTxRx_MUSIM | 43-1 | MUSIM gap and “keep solution” | Support UE indicates preferred MUSIM gap priority and MUSIM gap priority configuration; support UE indicates “keep solution” and “keep solution” configuration | musim-GapPreference-r17 | YES |  | UE is not capable to support MUSIM feature | Per UE | No | No | N/A |  | Optional |

- 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. XX | 44-1 | TxD capability | TxD support capability for 2Tx |  | YES | NO | Per band per band combination of 2Tx TxD capability can not be supported and verified | TBD | No | FR1 only |  |  | Optional with capability signalling |
| 44. XX |  |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 |  |  |  |  |  |  |  |  |  |  |  |  |