3GPP TSG-RAN WG4 Meeting # 109												R4-2318985

# Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	TP to 38.751 on RF requirement construction

Agenda Item:	8.7.1.2

Document for:	Approval

- Introduction

In previous meeting, many companies submit the simulation results for overall probability and some agreements on RF requirement construction was agreed. This TP capture the progress into the TR.

- Text proposal

## <<Start of Change>>

6.3.7	Consideration of AoA separation

#### 6.3.7.1	Candidate AoA separation

To reveal whether, in real networks, a multi-Rx UE has a clear tendency for the angle between TRPs that can be accessed, system level simulation is performed and the results are recorded in Annex A.2. The conclusion is that UE can access to TRP pairs only if the channel conditions are good enough, and there are no obvious preferred AoA separation. For simplicity while taking into account the constraints of the test system, 30°, 60°, 90°, 120°, 150° are agreed as the candidate AoA separations for requirement design.

#### 6.3.7.1	1AoA vs 2 AoA

How many AoA separations need to meet the requirement at the same time is the first problem to be solved. A popular option is to verify at least two AoA separations, one from [30°, 60°, 90°] and another one from [120°, 150°], and the intention is to get a full picture of UE performance.

As the simulation results shown in Annex A.8, when AoA separation changes, different UE implementation will show different trends, e.g., For the case that panels in opposite side, UE performance become better when the AoA separation is larger, but when panels in same side, UE performance will be worse with the increase of AoA separation. Due to the different trends, if two different AoA separation need to meet the requirements, to accommodate different UE implementation, the requirement for each AoA separation will always be gated by the implementation that has the worst performance. To avoid such restriction, RAN4 agree that only 1AoA separation from all candidates need to be verified.

#### 6.3.7.1	Specified vs declared

<Editor’s note: this clause will be added when there is a stable conclusion>

Annex <A>:
Simulation results

A.8 Simulation campaign for requirement design

Background

This section summarizes the simulation results from companies for final RF requirement, and the results for 3 typical UE implementations and 2 candidate combining methods are provided.

Table A.8-1 Overall probability for OR combining

| OR combining | Panels in adjacent faces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 60 | 90 | 120 | 150 | 180 |
| Qualcomm Incorporated |  | 23.7 | 33.6 | 35.4 | 26.6 |  |
| Apple | 15.2 | 25.6 | 37.5 | 36.7 | 23.0 | 9.1 |
| LG Electronics |  |  |  |  |  |  |
| Samsung1 | 8.0 | 17.0 | 21.0 | 15.0 | 7.0 | 1.0 |
| Samsung2 | 13.0 | 21.0 | 23.0 | 22.0 | 16.0 | 10.0 |
| Sony | 16.3 | 22.2 | 32.4 | 32.9 | 33.7 | 24.4 |
| Ericsson | 16.3 | 22.2 | 32.4 | 32.9 | 33.7 | 24.4 |
| Sony2 | 11.6 | 23.2 | 33.0 | 32.0 | 27.0 | 13.6 |
| Eircsson2 | 11.6 | 23.2 | 33.0 | 32.0 | 27.0 | 13.6 |
| vivo - plastic | 11.1 | 20.2 | 23.0 | 19.7 | 15.5 | 7.7 |
| vivo -metal | 12.3 | 17.6 | 18.5 | 17.2 | 16.2 | 8.8 |
| OPPO | 13.5 | 26.6 | 36.1 | 28.0 | 17.0 | 5.9 |
| Huawei, HiSilicon | 21.5 | 34.0 | 38.8 | 35.4 | 17.2 | 1.5 |
| Huawei, HiSilicon2 | 18.0 | 30.6 | 35.7 | 30.9 | 17.3 | 6.3 |
| Average | 14.0 | 23.6 | 30.6 | 28.5 | 21.3 | 10.5 |

| OR combining | Panels in opposite faces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 60 | 90 | 120 | 150 | 180 |
| Qualcomm Incorporated |  | 7 | 22 | 40.9 | 48.4 |  |
| Apple | 0.0 | 5.5 | 15.8 | 39.8 | 50.3 | 48.6 |
| Samsung1 | 0.0 | 4.0 | 19.0 | 35.0 | 37.0 | 29.0 |
| Samsung2 | 0.0 | 4.0 | 22.0 | 41.0 | 44.0 | 44.0 |
| Sony | 1.5 | 7.6 | 24.9 | 47.1 | 47.1 | 38.1 |
| Ericsson | 1.5 | 7.6 | 24.9 | 47.1 | 47.1 | 38.1 |
| vivo - plastic | 3.6 | 9.8 | 23.0 | 35.0 | 38.2 | 28.3 |
| vivo -metal | 17.8 | 20.6 | 19.6 | 16.7 | 21.7 | 17.2 |
| OPPO | 14.2 | 13.4 | 20.5 | 36.3 | 46.1 | 39.4 |
| Huawei, HiSilicon | 7.8 | 18.4 | 34.4 | 45.2 | 42.5 | 22.6 |
| Average | 5.2 | 9.8 | 22.6 | 38.4 | 42.2 | 33.9 |

| OR combining | Panels in same faces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 60 | 90 | 120 | 150 | 180 |
| Qualcomm Incorporated |  | 20.6 | 37.5 | 31.4 | 17.3 |  |
| Samsung1 | 23.0 | 23.0 | 9.0 | 1.0 | 0.0 | 0.0 |
| Samsung2 | 28.0 | 26.0 | 18.0 | 8.0 | 6.0 | 2.0 |
| Sony | 41.5 | 37.1 | 32.1 | 29.5 | 27.2 | 18.6 |
| Ericsson | 41.5 | 37.1 | 32.1 | 29.5 | 27.2 | 18.6 |
| vivo - plastic | 32.6 | 27.1 | 25.2 | 20.6 | 16.1 | 10.7 |
| vivo -metal | 23.5 | 17.0 | 17.8 | 20.1 | 19.3 | 10.0 |
| OPPO | 41.2 | 39.4 | 31.2 | 21.5 | 16.2 | 11.8 |
| Huawei, HiSilicon | 20.5 | 29.2 | 35.4 | 31.0 | 16.6 | 4.3 |
| Average | 31.5 | 28.5 | 26.5 | 21.4 | 16.2 | 9.5 |

Table A.8-2 Overall probability for arithmetic mean combining

| Arithmetic mean | Panels in adjacent faces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 60 | 90 | 120 | 150 | 180 |
| Qualcomm Incorporated |  | 12.8 | 17.1 | 17.8 | 15.9 |  |
| Apple | 9.5 | 14.0 | 18.8 | 18.4 | 11.8 | 8.7 |
| Samsung1 | 4.0 | 9.0 | 11.0 | 7.0 | 4.0 | 1.0 |
| Samsung2 | 7.0 | 11.0 | 12.0 | 11.0 | 9.0 | 10.0 |
| Sony | 9.5 | 11.4 | 16.2 | 19.0 | 23.3 | 24.4 |
| Ericsson | 9.5 | 11.4 | 16.2 | 19.0 | 23.3 | 24.4 |
| Sony2 | 5.8 | 11.6 | 16.6 | 16.0 | 14.6 | 13.6 |
| Eircsson2 | 5.8 | 11.6 | 16.6 | 16.0 | 14.6 | 13.6 |
| vivo - plastic | 6.1 | 11.3 | 11.9 | 10.3 | 8.9 | 7.7 |
| vivo -metal | 7.0 | 9.5 | 9.8 | 9.3 | 9.3 | 8.8 |
| OPPO | 6.8 | 13.5 | 18.1 | 14.0 | 8.6 | 5.9 |
| Huawei, HiSilicon | 13.9 | 18.3 | 19.7 | 17.7 | 8.6 | 1.5 |
| Huawei, HiSilicon2 | 10.7 | 16.5 | 18.0 | 15.5 | 8.6 | 6.3 |
| Nokia, Nokia Shanghai Bell | 12.0 | 31.0 | 35.0 | 35.0 | 30.0 |  |
| Average | 8.3 | 13.8 | 16.9 | 16.1 | 13.6 | 10.5 |

| Arithmetic mean | Panels in opposite faces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 60 | 90 | 120 | 150 | 180 |
| Qualcomm Incorporated | 30 | 60 | 90 | 120 | 150 | 180 |
| Apple |  | 3.5 | 11 | 23.9 | 37.6 |  |
| LG Electronics | 0.0 | 0.0 | 5.5 | 23.4 | 37.7 | 48.7 |
| Samsung2 | 0.0 | 2.0 | 10.0 | 17.0 | 23.0 | 29.0 |
| Sony | 0.0 | 2.0 | 11.0 | 21.0 | 27.0 | 44.0 |
| Ericsson | 0.7 | 3.8 | 12.7 | 24.8 | 33.0 | 38.1 |
| Sony2 | 0.7 | 3.8 | 12.7 | 24.8 | 33.0 | 38.1 |
| vivo -metal | 1.9 | 5.0 | 12.0 | 18.6 | 24.3 | 28.3 |
| OPPO | 10.7 | 12.4 | 10.9 | 9.5 | 12.9 | 17.2 |
| Huawei, HiSilicon | 7.1 | 6.7 | 10.3 | 19.9 | 32.7 | 39.3 |
| Huawei, HiSilicon2 | 4.0 | 9.6 | 17.2 | 25.1 | 25.3 | 22.6 |
| Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |
| Average | 3.2 | 6.3 | 13.3 | 22.6 | 30.6 | 33.9 |

| Arithmetic mean | Panels in same faces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 60 | 90 | 120 | 150 | 180 |
| Qualcomm Incorporated |  | 10.3 | 18.8 | 15.7 | 8.7 |  |
| Samsung1 | 17.0 | 12.0 | 5.0 | 1.0 | 0.0 | 0.0 |
| Samsung2 | 20.0 | 15.0 | 9.0 | 4.0 | 3.0 | 2.0 |
| Sony | 26.9 | 21.6 | 17.6 | 16.0 | 17.6 | 18.6 |
| Ericsson | 26.9 | 21.6 | 17.6 | 16.0 | 17.6 | 18.6 |
| vivo - plastic | 21.9 | 14.5 | 12.8 | 10.5 | 9.2 | 10.7 |
| vivo -metal | 14.5 | 9.3 | 10.0 | 12.2 | 11.2 | 10.0 |
| OPPO | 28.9 | 23.0 | 16.3 | 10.8 | 9.3 | 11.6 |
| Huawei, HiSilicon | 12.4 | 15.8 | 17.8 | 15.5 | 8.3 | 4.3 |
| Nokia, Nokia Shanghai Bell | 10.0 | 28.0 | 37.0 | 28.0 | 30.0 |  |
| Average | 19.8 | 17.1 | 16.2 | 13.0 | 11.5 | 9.5 |

## <<End of Change>>

- References

[1] R4-2314668	WF on FR2_multiRx_UERF	vivo， RAN4#108

[2]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_108bis/Inbox/Drafts/%5B108bis%5D%5B127%5D%20FR2_multiRx_UERF_part1/draftTopic127_simresults_v3.xlsx