3GPP TSG-RAN WG4 Meeting # 109	R4-2318986

Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	Discussion on FR2 multi-Rx RF test method

Agenda Item:	8.2.2

Document for:	Approval

- Introduction

In the last meeting, several new issues for UE orientation were raised [1]:

(New) Issue 1-3-1: UE orientations/alignment options for multi-Rx RF testing:

- Agreement

  - Further discussions are needed in next meeting:

    - On the definition of UE orientation/alignment options for multi-RX RF testing.

      - Based on the decisions in RF core requirements session, the 9 orientations in J.3.X of TS 38.101-2 need to be considered as starting orientation for testing.

    - Further study whether the re-positioning concept must be adapted for multi-RX

    - The position of AoA1 with respect to the coordinate system in the starting orientation (theta=0, phi=0) can be further studied

We further provide our analysis on re-positioning procedure.

- Discussion

In the legacy single AoA test, the re-positioning procedure is used to avoid the blockage issue. The re-positioning can be used because in single AoA test, the test system can traverse all AoAs. The repositioning procedure just split the AoA into to 2 hemispheres and test each of them sequentially. Considering in multi-Rx test system, the zenith angles θ∈[0,360], so the only way to cut the sphere is θ∈[0, 180) and θ∈(180, 360], as shown in Figure 1

Figure 1 Two hemispheres for multi-Rx (left) and single AoA (right)

Observation 1: The hemisphere division for multi-Rx is different from single AoA test.

For 2 AoA test system, due to lack of full degree of freedom, only part of AoA pair can be tested and the AoA pairs are closely related to the relative position between antenna modules of UE and probes in test system. To further show whether re-positioning procedure still works in multi-Rx system, one of the simplest ways to do is to compare the performance when the initial UE orientation is before re-positioning and after re-positioning. The simulation results are shown in below:

Table I UE performance comparison between before/after re-positioning

| AoA offset | Overall probability | Initial UE orientation | Overall probability | Initial UE orientation |
| --- | --- | --- | --- | --- |
| 30 | 7.2% |  | 5.7% |  |
| 60 | 9.3% |  | 8.3% |  |
| 90 | 7.5% |  | 8.1% |  |
| 120 | 7.3% |  | 6.7% |  |
| 150 | 8.8% |  | 8.1% |  |
| AoA offset | Overall probability | Initial UE orientation | Overall probability | Initial UE orientation |
| 30 | 9.8% |  | 9.6% |  |
| 60 | 18.0% |  | 21.0% |  |
| 90 | 20.6% |  | 25.1% |  |
| 120 | 17.7% |  | 20.8% |  |
| 150 | 10.3% |  | 11.7% |  |

| AoA offset | Overall probability | Initial UE orientation | Overall probability | Initial UE orientation |
| --- | --- | --- | --- | --- |
| 30 | 11.1% |  | 10.7% |  |
| 60 | 20.2% |  | 18.5% |  |
| 90 | 23.0% |  | 21.7% |  |
| 120 | 19.7% |  | 18.0% |  |
| 150 | 15.5% |  | 14.1% |  |

The results show that UE performances will change after re-positioning, so for multi-Rx test, the current re-positioning will bring unexpected performance degradation and cannot be applied directly.

Observation 2: current re-positioning of single AoA may bring unexpected performance degradation in multi-Rx system.

Since the multi-Rx requirement is verified by a pass/fail test, the either of the probes obscured by the pedal will lead to related test point fail the test, so we think the re-positioning is still needed but need to be redesigned for multi-Rx system.

Proposal: The re-positioning procedure to avoid blockage need to be redesigned for multi-Rx test system.

- Conclusion

Observation 1: The hemisphere division for multi-Rx is different from single AoA test.

Observation 2: Current re-positioning of single AoA may bring unexpected performance degradation in multi-Rx system.

Proposal: The re-positioning procedure to avoid blockage need to be redesigned for multi-Rx test system.

- References

- R4-2317002	WF on FS_NR_FR2_OTA_en,	Qualcomm, RAN4#108bis