3GPP TSG-RAN WG4 Meeting # 109 	R4-2318987

Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	Preliminary MU assessment for multi-Rx RF testing

Agenda Item:	8.2.5

Document for:	Approval

- Introduction

In the last meeting, the following framework for MU analysis is agreed as starting point [1]:

Table 1.2.1-1: Uncertainty assessment for wanted DL signal absolute power in 2AoA coverage measurement with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | [0.00] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 7) | 0.6 | Actual | 1.00 | [0.6] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | [1.45] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Multiple measurement antenna uncertainty (NOTE 6) | 0.15 | Actual | 1.00 | [0.15] |
| 14 | DUT repositioning | 0.00 | Rectangular | 1.73 | [0.00] |
| 15 | Influence of spherical coverage grid (NOTE 4) | 0.12 | Actual | 1 | [0.12] |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 19 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | [0.75] |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 23 | Quality of quiet zone for calibration process (NOTE 7) | 0.4 | Actual | 1.00 | [0.4] |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Measurement uncertainty |  |  |  | Value |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.86] |

Table 1.2.1-2: Total uncertainty assessment for 2AoA coverage measurement with IFF

| Measurement uncertainty | Value |
| --- | --- |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [%] | X% |
| Uncertainty related to measurement grid | Y% |
| Total Measurement uncertainty | Value |
| [2AoA spherical coverage] expanded uncertainty (1.96σ - confidence interval of 95 %) [%] | X+Y% |
| NOTE 1: X% is derived based on the simulations with different DL power vs percentage of 2AoA metric. NOTE 2: Y% is derived based on the simulations with measurement step size vs percentage of 2AoA metric. |  |

In this contribution, we further provide our analysis for X% and Y%.

- Discussion

  - The value of X%

In the RF session, 2 types of method to construct the requirement was agreed, which related whether the AoA offset can be declared by UE [2].

Three types of reference UE implementation (two panels on the same side, two panels on the adjacent side and two panels on the opposite side) will be used to determine the core requirement:

- If the AoA offset would be declared by UE

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 |
| --- | --- | --- | --- | --- | --- |
| Reference UE | same | same | adjacent | opposite | opposite |

- If the AoA offset would be specified in the standard.

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 |
| --- | --- | --- | --- | --- | --- |
| Reference UE | Min (same, adjacent, opposite) | Min (same, adjacent, opposite) | Min (same, adjacent, opposite) | Min (same, adjacent, opposite) | Min (same, adjacent, opposite) |

In the following MU assessment, the rules above are analyzed respectively. In the simulation, we assume DL power change is △P which satisfy normal distribution N (0, 2.482). The △P is applied to each probe independently, which mean the △P can be different for a pair of AoA. The measurement grid is 15° and the best initial orientation is guaranteed for each case. Due to the time constraints, 1000 times simulations were performed for each case.

One UE declared AoA offset. Both the results of arithmetic mean combining and OR combing are provided.

- AoA offset is declared by UE

The simulation results are show in following figures and summarized in Table I

probability without △P = 0.216

probability without △P = 0.331

Figure 1 Results for AoA offset = 30° when panels in same side

probability without △P = 0.259

probability without △P = 0.142

Figure 2 Results for AoA offset = 60° when panels in same side

probability without △P = 0.11

probability without △P = 0.22

Figure 3 Results for AoA offset = 90° when panels in adjacent side

probability without △P = 0.345

probability without △P = 0.182

Figure 4 Results for AoA offset = 120° when panels in opposite side

probability without △P = 0.238

probability without △P = 0.380

Figure 5 Results for AoA offset = 150° when panels in opposite side

Table I X% for 95% confidence interval when AoA offset is decalred

| AoA offset | 30° | 60° | 90° | 120° | 150° |
| --- | --- | --- | --- | --- | --- |
| OR | 5.0% | 5.3% | 4.1% | 4.9% | 4.5% |
| Arithmetic mean | 3.3% | 2.9% | 2.2% | 3.1% | 3.3% |

- AoA offset is specified (90°) in the spec

The simulation results are show in following figures and summarized in Table II

probability without △P = 0.134

probability without △P = 0.251

Figure 6 Results for AoA offset = 90° when panels in same side

probability without △P = 0.125

probability without △P = 0.235

Figure 7 Results for AoA offset = 90° when panels in opposite side

probability without △P = 0.11

probability without △P = 0.22

Figure 8 Results for AoA offset = 90° when panels in adjacent side

Table II X% for 95% confidence interval when AoA offset is specified

|  | AoA offset | 90° |
| --- | --- | --- |
| Panels in same side | OR | 4.5% |
|  | Arithmetic mean | 2.3% |
| Panels in oppsite side | OR | 5.4% |
|  | Arithmetic mean | 3.3% |
| Panels in adjacent side | OR | 4.0% |
|  | Arithmetic mean | 2.2% |

Based on the simulation results, we have following observations:

Observation 1: The X% is similar across different AoA offset when same combining method is used.

Observation 2: The rules of RF requirement construction have small impact on the value of X%.

So, the X% mainly depends on the combing method. Considering leave some margin, we propose:

Proposal 1: For OR combing, take the X% = 6% as starting point, and for arithmetic mean combing, take X% = 4% as starting point.

  - The value of Y%

In previous meeting, the MU of measurement grid is analyzed by several companies and summarized in following tables:

Table III Impact of measurement grid for OR combining

| Adjacent modules |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung | 10deg | 0.00% | -0.08% | -0.27% | 0.32% | 0.17% | 0.16% |
| Samsung | 15deg | 0.21% | 0.28% | 0.23% | 0.56% | -0.08% | -0.56% |
| QC | 10deg |  | 0.10% | 0.20% | 0.80% | 0.60% | 0.00% |
| QC | 15deg |  | 1.60% | 0.30% | 2.60% | 2.60% | 1.50% |
| OPPO | 10deg | 0.21% | 0.54% | 0.27% | 0.82% | 0.84% | 0.39% |
| OPPO | 15deg | 0.97% | 0.26% | 1.47% | 0.38% | 0.20% | 0.53% |
| Back-to-back modules |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung | 10deg | 0.04% | -0.11% | 0.09% | 0.25% | 0.10% | 0.09% |
| Samsung | 15deg | -0.15% | -0.43% | -0.72% | -0.35% | -0.43% | -0.41% |
| QC | 10deg |  | 1.20% | 0.10% | -1.10% | -0.70% | -1.30% |
| QC | 15deg |  | 1.50% | 0.40% | -3.20% | -2.80% | -2.20% |
| OPPO | 10deg | 2.93% | 3.48% | 0.16% | 2.74% | 0.81% | 1.29% |
| OPPO | 15deg | 3.65% | 4.56% | 0.41% | 0.77% | 1.22% | 1.03% |
| Same side modules |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung | 10deg | -0.02% | 0.10% | 0.28% | 0.25% | 0.31% | 0.04% |
| Samsung | 15deg | -0.16% | -0.35% | -0.08% | -0.23% | 0.07% | -0.30% |
| OPPO | 10deg | 1.25% | 0.24% | 0.50% | 0.23% | 2.66% | 1.96% |
| OPPO | 15deg | 0.24% | 1.74% | 2.08% | 0.25% | 1.35% | 1.10% |

Table IV Impact of measurement grid for arithmetic mean combining

| Adjacent modules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30 |  | 60 |  | 90 |  | 120 |  | 150 |  | 180 |  |  |
| QC |  | 10deg |  |  | -0.10% |  | 0.10% |  | 0.40% |  | 0.20% |  | 0.00% |  |  |
| QC |  | 15deg |  |  | 1.20% |  | 0.10% |  | 1.30% |  | 1.90% |  | 1.50% |  |  |
| vivo |  | 10deg | -0.30% |  | -0.50% |  | 0.10% |  | 0.20% |  | 0.10% |  |  |  |  |
| vivo |  | 15deg | -0.20% |  | -1.00% |  | -0.70% |  | -1.10% |  | -0.10% |  |  |  |  |
| Back-to-back modules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 60 |  | 90 |  | 120 |  | 150 |  | 180 |  |  |
| QC |  | 10deg |  |  | 0.60% |  | 0.10% |  | -0.20% |  | -0.50% |  | -1.30% |  |  |
| QC |  | 15deg |  |  | 0.70% |  | 0.30% |  | -0.90% |  | -1.90% |  | -2.20% |  |  |
| vivo |  | 10deg | 0.20% |  | -0.50% |  | 0.10% |  | -0.20% |  | -1.50% |  |  |  |  |
| vivo |  | 15deg | 0.60% |  | 1.30% |  | 0.50% |  | 0.20% |  | 1.20% |  |  |  |  |
| Same side modules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 60 |  | 90 |  | 120 |  | 150 |  | 180 |  |
| vivo | 10deg |  |  | -1.20% |  | 0.50% |  | 0.10% |  | 1.00% |  | 0.50% |  |  |  |
| vivo | 15deg |  |  | -0.40% |  | -0.40% |  | -0.20% |  | 0.40% |  | -0.40% |  |  |  |

Using same rules mentioned in previous part, we pick out the maximum performance change across all companies:

Table V maximum Y% when AoA offset is decalred

|  | AoA offset | 30° | 60° | 90° | 120° | 150° |
| --- | --- | --- | --- | --- | --- | --- |
| 10 deg | OR | 1.25% | 0.24% | -0.27% | 2.74% | 0.8% |
|  | Arithmetic mean | -1.20% | 0.5% | 1.47% | 0.4% | -1.5% |
| 15 deg | OR | 0.24% | 1.74% | 0.10% | -3.2% | -2.8% |
|  | Arithmetic mean | -0.40% | -0.40% | -0.70& | 1.3% | -1.9% |

Table VI maximum Y% when when AoA offset is specified

|  | AoA offset | 90°(10 deg) | 90°(15 deg) |
| --- | --- | --- | --- |
| Panels in same side | OR | 0.5% | 2.08% |
|  | Arithmetic mean | 0.1% | -0.2% |
| Panels in oppsite side | OR | 0.16% | -0.72% |
|  | Arithmetic mean | 0.1% | 0.5% |
| Panels in adjacent side | OR | -0.27% | 1.4% |
|  | Arithmetic mean | 0.1% | -0.7% |

Based on the results, we observe that:

Observation 3: The difference of overall probability between 10° and 15° is not significant and the performance accuracy of 10° is not always better than 15°.

Observation 4: There is also no obvious trend for performance change when AoA offset increases.

To easily accommodate different cases, we propose use the maximum performance change as the Y%

Proposal 2: Take the Y% = 3.5% as the starting point.

- Conclusion

In this contribution, we provide our views on the value of X% and Y% in MU assessment.

Observation 1: The X% is similar across different AoA offset when same combining method is used.

Observation 2: The rules of RF requirement construction have small impact on the value of X%.

Observation 3: The difference of overall probability between 10° and 15° is not significant and the performance accuracy of 10° is not always better than 15°.

Observation 4: There is also no obvious trend for performance change when AoA offset increases.

Proposal 1: For OR combing, take the X% = 6% as starting point, and for arithmetic mean combing, take X% = 4% as starting point.

Proposal 2: Take the Y% = 3.5% as the starting point.

- References

- R4-2309811	WF for FR2 OTA test SI	Qualcomm Incorporated, RAN4#108bis