3GPP TSG-RAN WG4 Meeting # 109										 R4-2318988

# Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	Discussion on the restriction of R17 DC location signaling

Agenda Item:	5.2.1

Document for:	Approval

- Introduction

The new R17 DC location reporting signaling was introduced in RAN2 and after checking the spec, it seems some unexpected restriction exist. In this contribution, we provide our view on the current signaling design.

- Discussion

In current TS 38. 331, the R17 DC location signaling is described as follows:

It is noted that under current description, only single value can be reported if the frequency component is set to configuredCarrier or configuredBWP. In our understanding, the single offsetValue is introduce to save overhead when DC location is not changed for different CC configurations, but the current description may put unnecessary restrictions on UE implementation, as shown in Figure 1:

Figure 1 DC location in different UE implementation

For UE implementation 1, the current offsetValue can save signaling overhead successfully, but the description in the spec will preclude UE implementation 2 which is not preferred. The analysis in [2] also prove removing such restriction is beneficial for UE. Similar restrictions may also happen to the frequency component is activeCarrier or activeBWP, so in our view, it is better to inform RAN2 to remove such restriction and allow UE to use its preferred signaling method. However, to minimize the impact for commercial product, this change can be applied from R18 rather than R17.

Observation: Current description for R17 DC location signaling in RAN2 spec will restrict UE implementation in some cases.

Proposal: Send LS to RAN2 to remove the restriction from R18 and let the UE choose whether a single offset value or offset list is needed.

- Conclusion

In this contribution, we identify a possible restriction on UE implementation for DC location signaling.

Observation: Current description for R17 DC location signaling in RAN2 spec will restrict UE implementation in some cases.

Proposal: Send LS to RAN2 to remove the restriction from R18 and let the UE choose whether a single offset value or offset list is needed.

- References

- TS 38.331

- R4-2312762, R17 DC location reporting clarification, OPPO, RAN4#108