# 3GPP TSG-RAN WG4 Meeting #109	R4-2318995

# Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	Maintenance TP to TR 38.786

Agenda Item:	8.30.1

Document for:	Approval

# Introduction

In this text proposal, we provided revisions based on the latest version of TR 38.786[1].

# Reference

[1] TR 38.786 V1.1.0

3	Definitions of terms, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces, irrespective of TDM mode and FDM mode, while operation is agnostic of the service used on each interface.

Inter-band con-current operation: Operation of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL CA operation: Aggregation of two or more NR Sidelink component carriers in order to support wider transmission bandwidths.

Intra-band SL CA UE: UE that supports NR SL CA operation in a single band

NR SL inter-band con-current operating Band：Band combinations of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL-U UE: UE that supports NR Sidelink operation in unlicensed bands (e.g. n46, n96, n102).

PC5: The interface for sidelink transmission.

PC3/PC5 UE: UE that supports Power class 3 or Power class 5

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

<Unchanged Parts Omitted>

5.1.2	Operating band combinations for inter-band con-current operation

It has been agreed to use  n78@licensed band + n46@unlicensed band as the example band combination for Uu@Licensed and SL@Un-licensed.

The below inter-band con-current operation has been agreed not to be supported in Rel-18.

  - Uu@Un-licensed and SL@Un-licensed

  - Uu@Un-licensed and SL@licensed

  - Intra-band SL@Un-licensed

NR SL operation is designed to operate concurrently with NR uplink/downlink on the operating band combinations listed in Table 5.1.2-1.

- Table 5.1.2-1 Band combinations of inter-band con-current operating bands

| NR SL inter-band con-current operating Band | NR Band | Interface |
| --- | --- | --- |
| SL_n78-n46 | n78 | Uu |
|  | n46 | PC5 |

5.1.3	Operating band combinations for NR SL CA operation

NR SL CA operation is designed to operate in the operating bands in FR1 defined in Table 5.1.3-1.

Table 5.1.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

5.2	Channel bandwidth

5.2.1	Channel bandwidth for single carrier operation in unlicensed band

Channel bandwidths defined for NR SL-U operating bands are in Table 5.2.1-1.

Table 5.2.1-1 NR SL-U channel bandwidth

| SL-U band /channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n46 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| n96 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| n102 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |

Support for 10MHz channel bandwidth is not defined for NR SL-U single carrier operation in bands n46, n96 and n102.

5.2.2	Channel bandwidth for inter-band con-current operation

It has been agreed to apply each channel bandwidth that is supported in a band for Uu and a band for SL-U.

For NR SL inter-band con-current operation in FR1, the NR channel bandwidths and SL channel bandwidths for each operating band are specified in Table 5.2.2-1.

Table 5.2.2-1: Inter-band con-current operation configurations

| NR SL inter-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SL_n78A-n46A | n78 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n46 | PC5 | 20, 40, 60, 80, 1002 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 in TS38.101-1. NOTE 2: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |

5.2.3	Channel bandwidth for NR SL CA operation

For NR SL CA operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.3-1.

Table 5.2.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | 20 | 20,30 |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

5.3	Channel arrangement

### 5.3.1 Channel spacing

The general requirements in clause 5.4.1 of TS 38.101-1 are applied for NR SL-U.

### 5.3.2	Channel raster

The general requirements in clause 5.4.2 of TS 38.101-1 are applied for SL-U.

NR-ARFCN and channel raster requirements in 5.4.2.1 of TS 38.101-1 are applied for NR SL-U with following exception:

- 7.5kHz frequency raster shift, which can be used in NR V2X in band n47 is not defined for NR SL-U operation in bands n46, n96, n102.

- Channel raster entries for each operating band requirements in clause 5.4.2.3 of TS 38.101 are applied for NR SL-U with following exception: Channel raster points for 10MHz CBW in band n46 as defined in Table 5.4.2.3-2 are not applicable for NR SL-U.

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in Table 5.3.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both Tx and Rx for SL. The mapping must apply to at least one numerology supported by the UE.

Table 5.3.2-1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

### 5.3.3	Synchronization raster

There is no synchronization raster definition for NR sidelink unlicensed bands n46, n96, n102.

6	Transmitter characteristics for NR SL evolution

6.1	Tx requirements for NR SL single carrier operation in unlicensed band

6.1.1	Maximum output power for NR SL-U

The SL-U UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth in unlicensed bands. The period of measurement shall be at least one sub frame (1ms).

Table 6.1.1-1: SL-U UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The SL-U UE shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 in TS38.101-1 when NS is signalled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.1.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80, 100 | 5925 – 7125 | -1 |
|  | NS_60 | 20, 40, 60, 80, 100 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
| n102 | NS_58 | 20, 40, 60, 80, 100 | 5945 – 6425 | 10 |

6.1.2	UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

- Basic RF parameters for MPR of single carrier SL-U UE

  - Waveforms: CP-OFDM for SL-U

  - Supported CBW: 20/40/60/80/100MHz.

  - Modulation: QPSK/16-QAM/64-QAM/256-QAM

  - SCS: 15/30kHz

  - C-IMD: 45dBc or 60dBc

  - EVM and impairments

| Modulation | SystemEVM | PA only | Image |
| --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 34dB |
| 16QAM | 12.5% | 8% | 34dB |
| 64QAM | 8% | 4% | 34dB |
| 256QAM | 3.5% | 1.8% | 34dB |

  - IBE and impairment exceptions are same as NR-U

- Reuse IBE mask from TS38.101-1 Table 6.4F.2.3-1

- Image/carrier exceptions position and specific test according to interlaces.

  - ACLR: 27dBc for both PC3/PC5 UE with NR MBW

  - SEM according to 38.101-1 clause 6.5F.2.2 as well as Table 6.1.2-4

  - PA calibration & PA configuration:

- Power class 5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

  - MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission if any issue is not seen..

  - The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

  - Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, and on the repeated RB allocation for S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-2: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is to follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB-set, RAN4 consider interlacing RBs for PSFCH For multiple RB-sets, RAN4 considers both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of interlaced transmission should meet at least 80% of channel bandwidth in a single RB-set [Ngap = RBend – RBstart ]. - RB allocation method as NR SL legacy RB allocation is also considered. |

For S-SSB transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-3: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of repeated S-SSB transmission should meet at least 80% of channel bandwidth.in a single RB-set [Ngap = RBend – RBstart]. |

For MPR and A-MPR for SL-U wideband operation, RAN4 assumed in-carrier SEM as follows.

In the case of non-transmitted 20 MHz channel(s) on the edges of an assigned channel bandwidth the spectrum emission mask for operation with shared spectrum channel access, specified in Table 6.5F.2.2.0-1, is applied by using the total bandwidth of the remaining transmitted channels. The spectrum emission mask for non-transmitted channels is floored at -28dBr inside the assigned channel bandwidth and continues at same level until the channel bandwidth mask is intercepted, i.e. until ΔfOOB, as defined in 38.101-1 Clause 6.5F.2.2.0, equals BWChannel / 2. When ΔfOOB > BWChannel / 2 requirements in clause 6.5F.2.2.0 apply.

The In-Carrier SEM in gap between transmitted channels is defined in Table 6.1.2-4 and some examples are shown in Figure 6.1.2-1.

The relative power of any UE emission shall not exceed the most stringent levels given by the spectrum emission mask for operation with shared spectrum channel access with full channel bandwidth and the spectrum emission mask for non-transmitted channels with the channel bandwidth of the transmitted channels in the case of non-transmitted channels at the edge of an assigned channel bandwidth or spectrum emission mask defined for gaps between the transmitted channels.

An exception to the spectrum emission mask for non-transmitted channels allows a single 2 MHz bandwidth to extend to -28 dBc relative to total transmit power, or -20 dBm, whichever is the greatest.

<Unchanged Parts Omitted>

6.1.2.1.1.3	Huawei’s simulation results (R4-2315226)

Referring to the simulation cases listed in Table 6.1.2.1.1.3-1, the MPR value are shown in Table 6.1.2.1.1.3-2 and Table 6.1.2.1.1.3-3 for below.

Table 6.1.2.1.1.3-1 Simulation cases for SL-U

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 8 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 9 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 10 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 11 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation | 13 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 14 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 15 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 16 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 17 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 18 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 21 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 22 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 23 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 6.1.2.1.1.3-2  Maximum power reduction (MPR) for SL-U with Full Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 3.8 | 4.1 | 4.0 | 4.0 | 4.1 |
| 16 QAM | 4.1 | 3.9 | 4.1 | 4.1 | 3.8 | 4.2 |
| 64 QAM | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| 256 QAM | 7.5 | 7.7 | 7.6 | 7.8 | 7.5 | 7.9 |

Table 6.1.2.1.1.3-3  Maximum power reduction (MPR) for SL-U with Interlaced Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 4.1 | 4.0 | 4.1 | 4.1 | 4.1 |
| 16 QAM | 4.1 | 4.2 | 4.2 | 4.1 | 4.2 | 4.2 |
| 64 QAM | 5.8 | 5.9 | 5.7 | 5.6 | 5.9 | 6.1 |
| 256 QAM | 7.0 | 7.3 | 7.1 | 7.4 | 7.7 | 7.4 |

Table 6.1.2.1.1.3-4  Maximum power reduction (MPR) for SL-U with Wide band operation

| Modulation | Case: Bandwidth/Bitmap pattern |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40MHz | 60MHz |  | 80MHz |  |  | 100MHz |  |  |  |  |  |
|  | 10 | 100 | 110 | 1000 | 1110 | 0110 | 10000 | 11110 | 01000 | 01100 | 01110 | 00100 |
| QPSK | 4.0 | 3.7 | 3.9 | 3.8 | 4.0 | 3.8 | 3.9 | 4.1 | 3.0 | 3.2 | 3.9 | 3.4 |
| 16 QAM | 4.0 | 3.9 | 4.1 | 3.9 | 4.1 | 3.9 | 3.9 | 4.2 | 3.7 | 3.8 | 4.0 | 3.7 |
| 64 QAM | 5.1 | 5.1 | 5.5 | 5.2 | 5.7 | 5.7 | 5.1 | 5.8 | 5.4 | 6.0 | 5.9 | 5.5 |
| 256 QAM | 6.4 | 6.8 | 6.9 | 6.3 | 7.0 | 7.1 | 6.5 | 7.3 | 6.7 | 7.4 | 7.3 | 7.0 |

Table 6.1.2.1.1.3-5: Proposed MPR for SL-U with single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.1 | ≤ 4.1 |
|  | 16 QAM | ≤ 4.2 | ≤ 4.2 |
|  | 64 QAM | ≤ 6.1 | ≤ 6.1 |
|  | 256 QAM | ≤ 8.0 | ≤ 7.7 |

<Unchanged Parts Omitted>

6.2	Tx requirements for inter-band con-current operation

It has been agreed to consider the functionality and requirements of single carrier SL-U operation as a basis of SL-U  and the functionality and requirements of single carrier Uu operation as a basis of Uu in con-current operation.

6.2.1	Maximum output power for inter-band con-current operation

It has been agreed to target the following power classes.

  - Power class 3 Uu@Licensed + Power class 5 SL@Un-licensed

  - Power class 3 Uu@Licensed + Power class 3 SL@Un-licensed can be considered as 2nd priority if Power class 3 SL is agreed in a single carrier at unlicensed band

For Power class 2 for Uu@Licensed for inter-band concurrent operation, it is not considered in Rel-18.

RAN4 agreed to define maximum output power for each operating band and there is no total power class for SL-U con-current band combination.

Table 6.2.1: NR UE Power Class for inter band SL-U con-current combination

| NR SL con-current operating band Configuration | NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) | Class 5(dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL_n78-n46 | n78 |  |  |  |  | 23 | +2/-3 |  |  |  |  |
|  | n46 |  |  |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: For the con-current band combinations, the simultaneous transmission and reception of sidelink and Uu interfaces can be supported while operation is agnostic of the service used on each interface. NOTE 2: PPowerClass is the maximum output power specified without taking into account the tolerance for each operating band. NOTE 3: For inter-band con-current operation, the aggregation power apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Parts Omitted>

7	Receiver characteristics for NR SL evolution

7.1	Rx requirements for NR SL single carrier operation in unlicensed bands

7.1.1	Reference sensitivity power level for NR SL-U

Background information for NR V2X reference sensitivity requirement has been captured in the TR 38.886 as captured below and can be reused for NR SL-U

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: 2.5 dB is assumed. When the number of RB size equal to or less than 24RBs, 0.5dB additional relaxation is allowed.

-	Target SNR: -0.5 dB

-	Diversity gain: 3dB

For NR-U, the following equations in table below are used to calculate the REFSENS requirement and it can be further re-used in NR SL-U in bands n46, n96 and n102, while taking account that for NR-U target SNR of -1dB was used and therefore the REFSENS values in table below are 0.5dB higher than with NR SL-U target SNR of 0.5dB.

Table 7.1.1-1 REFSENS for NR-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.7 + 10log10(NRB/106) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -90.1 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -89.2 + 10log10(NRB/106) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for SL-U REFSENS as shown in table 7.1.1-1. |  |  |  |  |

For the NR-U REFESENS requirement, the RB configuration for larger than 20MHz is considered as wideband operation and hence all intra-cell guard bands between sub-bands should be filled. In this case, the full RB allocation of NR is used to define the reference sensitivity requirement. However, for V2X, specific RB allocation using the sub-channels based on the RAN2 specification TS 38.331 is used as captured below:

Following the TS 38.331 definitions Table 7.1.1-2 shows the RB configuration for NR SL-U reference sensitivity requirement.

Table 7.1.1-2 NR SL-U RB configurations for REFSENS

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | 135 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | 135 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | 135 |  |

With the agreed SNR and RB configuration as above, the REFNSENS requirement for NR SL-U is  shown in Table 7.1.1-3.

Table 7.1.1-3 REFSENS for SL-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

<Unchanged Parts Omitted>

8	Co-channel coexistence between LTE Sidelink and NR Sidelink

8.1.	Configured transmitted power for V2X UE supporting co-channel coexistence with LTE SL

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied. When NR V2X UE is configured to co-channel coexistence operation with LTE V2X and NR SCS is configured to 30kHz, the evaluation period for PUMAX,c for NR V2X sidelink is the first slot of NR SL slots overlapping with an LTE SL subframe.

8.2.	Relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL

Relative slot power tolerance is defined for NR V2X supporting co-channel coexistence to ensure the RX performance of the LTE V2X UE operating within the same channel. This requirement applies only to transmissions with 30kHz SCS.

The relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL is the ability of the NR V2X UE operating with 30kHz SCS to control the output power of transmitted slots during PSCCH/PSSCH transmission consisting of two slots overlapping with an LTE SL subframe (500us). The reference slot is the 1st slot overlapping with LTE SL subframe and target slot is the subsequent NR SL slot overlapping with the LTE SL subframe. The measurement period is one NR SL slot with guard symbol omitted.