3GPP TSG-RAN WG4 Meeting # 109		R4-2319006

Chicago, US, Nov 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Draft BigCR on SCell activation enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bells |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh3-Core |  |  |  |  |  | Date: |  |  | 2023-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The draft BigCR R4-2317350 was endorsed in RAN4#108bis meeting. We identify some ambiguity on the texts: - 9.2.4.X: The SCell activation triggered report is defined in RAN2 as a new reportType separate from event-triggered. The last paragraph is not needed. And the valid definition does not fully reflect the RAN4#107 agreements below. RAN4#107 Agreement: - The report of L3 measurement result after SCell activation command needs to be valid. - The report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) and accuracy requirements in TS 38.133 Clause 10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removing the last paragraph. - Adding texts to reflect the agreements above. - Some errors correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for reporting delay requirement for SCell activation triggered reporting is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.4.X; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 9.2.4.X	SCell activation Triggered Reporting

Reported RSRP, RSRQ, and RS-SINR measurements contained in SCell activation triggered reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2).

The measurement reporting delay is defined as the time between the SCell activation command that will trigger a measurement report and the point when the UE starts to transmit the valid measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The SCell activation triggered measurement reporting delay shall be no larger than THARQ + 7ms, where THARQ is the timing between DL data transmission and acknowledgement in ms as specified in TS 38.213 [3]. The SCell activation triggered report is considered as valid if it fulfils the measurement requirements for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2). When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

2µ

End of Change 1