3GPP TSG-RAN WG4 Meeting #109                                   R4-2319015

Chicago, USA, November 13 – November 17, 2023

Source: 	MediaTek Inc.

Title: 	Discussion on FR2-1 UL 256QAM

Agenda Item:	8.6.2

Document for:	Discussion

- Introduction

In RAN4 #108bis, a WF for UL 256QAM has been approved [1] for single CC MPR for PC1/2/5. But there is still an open issue that needs to be resolved regarding intra-band contiguous and non-contiguous CA MPR, as illustrated below.

Issue 1-1-4: MPR requirements for intra-band CA - Proposals - Option 1: Intra-band CA MPRs for both, contig. and NC, and for PC1/2/5 in 256QAM operation are increased from their respective 64QAM values by 3 dB. - Option2: Others.

In this contribution, we provide our perspective on intra-band contiguous and non-contiguous CA MPR.

- 2	Discussion

2.1 MPR for CA Operation

In the last RAN4 meeting, for the completion of the UL 256 QAM feature, issues related to MPRs for intra-band contiguous and non-contiguous CA were raised. Single CC MPRs for Power Classes 1/2/5 at 28GHz were approved, based on existing MPR values that are 3dB larger than the MPR values for 64QAM. In addition, MPRs for Power Classes 1/2/5 at 39GHz have a 1dB margin compared to those at 28GHz. Therefore, we could apply the same rule to MPRs for intra-band contiguous and non-contiguous CA for UL 256QAM.

Observation 1: Single CC MPRs for Power Classes 1/2/5 at 28GHz were approved, based on existing MPR values that are 3dB larger than the MPR values for 64QAM. In addition, MPRs for Power Classes 1/2/5 at 39GHz have a 1dB margin compared to those at 28GHz.

Proposal 1: Based on the above analysis, we propose the FR2-1 UL 256QAM MPR values for intra-band contiguous and non-contiguous CA for Power Classes 1/2/5 at 28GHz, as shown in Tables 1-4.

Table 1 Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 7.7 | 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.5 | 8.7 | 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 | 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | 10.7 | 11.2 | ≤ 11.7 |
|  | 256 QAM | ≤ 12.0 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 | 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | 10.7 | 11.2 | ≤ 11.7 |
|  | 256 QAM | ≤ 12.0 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |

Table 2 MPRNC_CA for UE power class 1 in FR2-1

| Waveform Type |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM | ≤ 12.0 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |
| CP-OFDM | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM | ≤ 12.0 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |

Table 3 Maximum power reduction (MPRC_CA) for UE power classes 2 and 5 in FR2-1

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | 256 QAM | ≤ 12 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |
| CP-OFDM | 256 QAM | ≤ 12 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |

Table 4 MPRNC_CA for UE power classes 2 and 5 in FR2-1

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | 256 QAM | ≤ 12 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |
| CP-OFDM | 256 QAM | ≤ 12 | ≤ 13.7 | ≤ 14.2 | ≤ 14.7 |

- 3	Conclusion

Observation 1: Single CC MPRs for Power Classes 1/2/5 at 28GHz were approved, based on existing MPR values that are 3dB larger than the MPR values for 64QAM. In addition, MPRs for Power Classes 1/2/5 at 39GHz have a 1dB margin compared to those at 28GHz.

Proposal 1: Based on the above analysis, we propose the FR2-1 UL 256QAM MPR values for intra-band contiguous and non-contiguous CA for Power Classes 1/2/5 at 28GHz, as shown in Tables 1-4.

- 4	Reference

- R4-2317596, “WF on FR2-1 UL 256QAM requirements”, Xiaomi.