3GPP TSG-RAN WG4 Meeting #109	R4-2319024

Chicago, USA, November 13 – November 17, 2023

Agenda Item:	8.19.2.2.3

Source:	MediaTek Inc.

Title: 	Maintenance TP to TR 38.858 on Feasibility of FR1 UE aspects

Document for:	Approval

- Introduction

In RAN4#108bis, TPs from different companies addressing the feasibility of UE aspects were consolidated and endorsed in [1]. In this paper, we propose some changes based on the endorsed TP [1].

## 9.6. FR1 Feasibility of UE aspects ### 9.6.1 Interference analysis #### 9.6.1.1 UE-UE co-channel inter-subband CLI modeling Editor's note: This section captures the CLI modeling. #### 9.6.1.2 UE-UE adjacent channel CLI modeling Editor's note: This section captures the CLI modeling. ### 9.6.2 Summary Editor's note: This section captures the conclusion of feasibility.

- Text Proposal

<<Start of Change for TR 38.858>>

## 9.6 FR1 Feasibility of UE aspects

### 9.6.1	Interference analysis

#### 9.6.1.1 General

In the objective of this study item, half duplex operation at UE side is assumed. In this part of feasibility of UE aspects, FR1 is considered.

In the UE feasibility study, existing UE RF requirements in TS 38.101-1 can be applied as default assumption for study phase conclusion if no issues identified by co-existence study. Detailed UE RF requirements if any should be discussed during WID phase.

In the UE feasibility study in FR1, the co-channel inter-subband UE-UE CLI model and adjacent channel UE-UE CLI model are mainly discussed. Co-channel/adjacent channel interference models at the UE side are summarized in Table 9.6.1.1-1. For co-channel interference models, the UE IBE requirement can be used for the interference modelling at Tx side; for receiver in-channel adjacent subband selectivity performance, no rejection/attenuation due to RF/BB filtering is assumed. For adjacent channel models, UE ACLR and adjacent channel selectivity requirements can be used for the interference modeling at Tx and Rx side, respectively.

Table 9.6.1.1-1. Existing UE interference models based on RF requirements in RAN4

| Co-channel RF interference models |  | Adjacent channel RF interference models |  |
| --- | --- | --- | --- |
| Tx side | Rx side | Tx side | Rx side |
| UE IBE for Tx | In-channel adjacent subband selectivity (Note 1) | Power dependent ACLR as described in Section 9.6.1.3.2 | Adjacent channel selectivity |
| Note 1. For legacy UE, there is no existing UE RF requirement for in-channel adjacent subband selectivity. It is only used in SBFD feasibility study purpose and no indication that this requirement shall or shall not be defined in the normative phase. |  |  |  |

#### 9.6.1.2	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

##### 9.6.1.2.1 Overview and analysis framework

The objectives of UE-UE co-channel inter sub-band CLI modeling is to analyze the impact of interference that occurs between two UEs in close proximity, operating on adjacent sub-bands within the same channel. This interference occurs when UL transmission of an aggressor UE in the channel interferes with the DL reception of a victim UE in the same channel.

For this SI RAN4 has decided to use typical UE parameters in the analysis, as different from worst-case parameters that are commonly used to define minimum performance requirements.  The evaluation of gNB performance improvements is underway in RAN1, taking into account a population of typical UEs rather than worst-case UEs.

In-channel adjacent subband selectivity is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. The value of in-channel adjacent subband selectivity is the ratio of the receiver attenuation on the assigned downlink subband to the receiver attenuation on the adjacent uplink subband. In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency errors and is not perfectly time-synchronized with the DL sub-band. The analysis indicates that the IBE interference could be more significant than the in-channel adjacent subband selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradations can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. The measurement data submitted by one company shows the achievable in-channel adjacent subband selectivity of FR1 UE can be 33 dB. Nonetheless, the typical interference will normally not be any worse than this value. For this reason, the 33 dB was agreed for modeling the in-channel adjacent subband selectivity.

For legacy UE, no sub-band filtering is considered.

To model the NF for co-channel CLI in a system level simulation, a fixed value noise figure of 9 dB is used. AGC is not modeled if a fixed NF model is used.

Apart from the in-channel adjacent subband selectivity, it is important to mention that degradation can also be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel cases, the leakage was agreed to be modelled based on IBE based model with details provided in clause 9.6.1.2.2.

##### 9.6.1.2.2 UE co-channel Tx model

In-Band Emissions

For UE co-channel Tx model, RAN4 has decided to use the IBE minimum requirements from 38.101-1 clause 6.4.2.3 as shown in Table 9.6.1.2.2-1 in the feasibility study. This IBE-based model consists of three parts: General, IQ image, and Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation. The granularity of this model is 1RB and it is not pursued to simplify this model to a frequency flat model. It is understood that these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters. Since typical values for the UE parameters were not determined, the formulation from the current specification is being used.

Table 9.6.1.2.2-1: Requirements for in-band emissions in TS 38.101-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

##### 9.6.1.2.3 UE co-channel Rx model

For UE co-channel Rx model, currently there is no corresponding RF requirement for this model. In the feasibility of UE co-channel Rx model, the definition of in-channel adjacent subband selectivity is introduced for SBFD feasibility study purpose:

- In-channel adjacent subband selectivity is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. The value of in-channel adjacent subband selectivity is the ratio of the receiver attenuation on the assigned downlink subband to the receiver attenuation on the adjacent uplink subband.

For legacy UE, no sub-band filtering is assumed in the feasibility study, and accordingly no rejection/attenuation due to RF/BB filtering is assumed and only the selectivity performance of the FFT operation is studied. 33 dB was agreed for FR1 in RAN4 for in-channel adjacent subband selectivity considering FFT operation.

Thermal self-noise performance

RAN4 decided on using a simplified fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was regarded to be sufficient for the purpose of system studies for SBFD operation. RAN4 decided on a noise figure value of 9 dB for FR1 UE.

Effect of jammer – non-thermal self-noise aspect

There are a few factors to consider in determining the in- channel subband interference in the presence of a co-channel jammer. With an in-channel adjacent-subband interferer the 3rd order distortion, reciprocal mixing, residual sideband, quantization noise, phase noise, ADC distortion, and analog filtering should be considered.

In RAN4 study, measurements were made of a UE receiver for various signal levels, interferer levels, interferer offsets, sub-band bandwidths, and interferer bandwidths. The measurements have included the impact of the entire receiver, which includes everything through the FFT operation, in which 120 various conditions were measured.

FFT leakage and selectivity

In the SBFD system an aggressor UE (UE2) operating in the UL sub-band may interfere with a UE (UE1) receiving in the adjacent DL sub-band. The UL signal from UE2 may arrive at UE1 misaligned in time or frequency, which can potentially cause UE1 DL SINR degradation in the FFT operation.

The simulation involved the conversion of the OFDM waveform to a spectrum by using FFT, along with the introduction of timing and frequency errors. A 5 RB guard band was assumed. The analysis revealed that time-misalignment was the main cause of the spectral leakage, and even minor timing errors resulted in leakage.

It appears reasonable to consider the leakage as a single average value, and the data shows about 33 dB down from the jammer level would be appropriate. The leakage effect should be taken into consideration and compared to the interference caused by the aggressor IBE.  It has been observed that the interference in the victim sub-band can be modelled as 33 dB below the input jammer power level, and the interference is approximately frequency flat across the victim subband.

Figure 9.6.1.2.3-1: FFT leakage with time and frequency misaligned blocker (5 RB guard band)

#### 9.6.1.3 UE-UE adjacent channel CLI modelling

Editor's note: This section captures the CLI modeling.

##### 9.6.1.3.1 Overview

The UE-UE adjacent channel CLI occurs when the UL transmission of the aggressor UE in a channel interferes with the DL reception of the victim UE in an adjacent channel. Unlike the case of co-channel interference, there is no need to consider any FFT selectivity in the adjacent channel scenario. Apart from the selectivity, it is necessary to account for the leakage from the UL sub-band into the DL sub-band caused by the transmitter. It was decided to assume the power-dependent ACLR of the aggressor UE and selectivity of the victim UE when modeling adjacent channel interference.

To model the NF for adjacent channel CLI in a system-level simulation, a fixed value noise figure of 9dB is used. The effect of AGC is not modeled when a fixed noise figure model is used. Additionally, UE ACLR should be modeled as 30 dB at max power, improving 1 dB/dB with back-off up to a maximum of 10 dB of improvement. Therefore, when the back-off is 10 dB, the ACLR is 40 dB. When the victim and aggressor UEs are close, between 1 and 50m, and close to the cell edge, with low desired signal level, and potentially high interference level, it cannot be guaranteed that the UE receiver will operate in the linear region.

##### 9.6.1.3.2 UE adjacent channel Tx model

UE adjacent channel leakage ratio is used in the feasibility study for adjacent channel UE-UE CLI Tx model. Only ACLR in 1st adjacent channel was considered in the study item and ACLR in 2nd adjacent channel was precluded. In the UE Tx model, only power class 3 was assumed.  UE ACLR is modelled as 30 dB at max power that improves 1dB/dB with backoff up to a maximum 10 dB of improvement. This means at 10 dB backoff the ACLR is 40 dB. Partially allocated UL subband was not considered in the system simulation. This ACLR model can be seen as frequency flat model, and the distortion is modelled as a flat power spectral density across the frequency range of the distortion.

##### 9.6.1.3.3 UE adjacent channel Rx model

UE adjacent channel selectivity (33dB for FR1) is used as adjacent channel UE-UE CLI model under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) for UE. If the blocker is higher than -25dBm, it is assumed that it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss).

### 9.6.2	Summary

Editor's note: This section captures the conclusion of feasibility.

In the FR1 UE feasibility study, existing UE RF requirements in TS 38.101-1 can be applied as default assumption for study phase conclusion if no issues identified by co-existence study. Detailed UE RF requirements if any should be discussed during WID phase.

For co-channel interference case, RAN4 concluded that the RF effect could be dominant, and the frequency offset and time offset are not significant factors influencing UE-UE interference. Furthermore, the leakage can be modelled by using the in-band emission (IBE) requirement based model.

As for the adjacent channel case, RAN4 concluded to assume power-dependent ACLR of the aggressor UE and adjacent channel selectivity of the victim UE when modeling adjacent channel interference.

For legacy UE, no sub-band filtering is considered.

A fixed value noise figure of 9 dB has been used to model the AGC and NF modeling for co-channel and adjacent channel CLI in a system level simulation.

<<End of Change>>

- References

- R4-2316936, TP to TR 38.858 on Feasibility of FR1 UE aspects, MediaTek, RAN4#108bis