3GPP TSG-RAN WG4 Meeting # 109	R4-2319034

# Chicago, US, November 13 – 17, 2023

Source:	China Telecom

Title:	Discussion on network B requirements for MUSIM gaps

Agenda item:	8.25.2.4

Document for:	Discussion

1	Introduction

In this contribution, we provide our views on the open issues for network B requirements in the WF [1].

2	Discussion

Issue 4-1-1: Network B requirements conditions - Proposals - P1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only (Ericsson vivo CMCC China Telecom Huawei oppo Apple) - P1-1: The inactive state requirement should be the same as NW B’s Idle state (Ericsson vivo China Telecom Nokia) - P2: RAN4 only one set of requirements for NW-B requirements when UE is allocated with MUSIM gaps. Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer. Continue discussion other conditions during or once NW B requirements are agreed. (Nokia) - P3: Postpone the discussion of additional conditions for defining Network B requirements until there is agreement on the framework for defining the requirements (issue 4-1-2). (Qualcomm) Recommendations:

For Issue 4-1-1, we support P1 and P1-1. Since the behaviors of UE in inactive and idle mode are similar, and the cell re-selection requirements for RRC_INACTIVE state is the same as that for RRC_IDLE state. So we think it is reasonable to define NW B measurement/cell reselection requirements in IDLE/inactive mode, and the inactive state requirement should be the same as NW B’s Idle state.

Proposal 1: Agree to define NW B measurement/cell reselection requirements in IDLE/inactive mode, and the inactive state requirement should be the same as NW B’s Idle state.

Issue 4-1-2: Network B requirements framework - Proposals - P1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (xiaomi Qualcomm oppo) - P2: The NW-B’s requirement should decouple with MUSIM gaps(mgrp) requested by UE; RAN4 to introduce a relaxed NW-B’s IDLE mode requirement as follow (Ericsson CMCC China Telecom) - P2-1: N = 4 (Qualcomm Huawei Apple) - P3: The measurement/cell reselection requirements in IDLE/inactive mode for NW B will reuse the existing idle/inactive requirements as the baseline. In requirements for a particular DRX cycle, it is replaced by N*DRX, where N is FFS and 1≤N≤16. N could use the format of P1 or P2 (vivo) - P4: Replace DRX cycle by max(DRX cycle, MGRP_max) and introduce a scaling factor of 2. (MTK) - P5: The UE measurement requirements for measurement/cell reselection requirements in IDLE/inactive mode for NW B, need to be based on a reasonable balance of the allocated MUSIM gap. The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements. current DRX cycle is replaced with Max(DRX cycle, Min(MUSIM gap MGRP)). Remove the M1 scaling factor. (Nokia) Recommendations: Online session outcome: - New Option 1: the network B requirement is related to MGRP - Issues for further discussion: the impact given that network B does not know the MGRP configured by network A - New Option 2: the network B requirements is not related to MGRP, and with a fixed scaling factor N based on the DRX cycle. Further discuss the N - Option 2A: N = 4, and other values are not precluded. - Issue for further discussion: UE behavior or network impact for the case that NxDRX cycle is shorter than MGRP.

Since MUSIM gaps are configured by NW A and NW B in inactive/idle mode can’t obtain the information of MUSIM gaps, new option 1 is not feasible. Based on this, we support new option 2 that NW-B’s requirement is not related to MGRP, and with a fixed scaling factor N based on the DRX cycle. Furthermore, since the NW-B’s requirement is fixed and decoupled with MGRP, UE should request MUSIM gaps based on the defined NW B requirements.

Considering the MGRP of MUSIM gaps can be from 20ms to 5.12s, while existing cell-reselection requirements are based on DRX cycle from 320ms to 2.56s. We think the relaxation factor can be from 2 (5.12/2.56) to 16 (5.12/0.32). However, a larger value may affect the performance of Network B, while a smaller value may require UE to request gaps with smaller MGRP, resulting in a decrease in NW A performance. To achieve a trade-off between NW B performance and NW A performance, we can accept N = 4. As for the case that NxDRX cycle is shorter than MGRP, we think it is rare and can be avoided by requesting appropriate gap configuration,

Proposal 2: Support new option 2 that NW-B’s requirement is not related to MGRP, and with a fixed scaling factor N based on the DRX cycle. And UE should request MUSIM gaps based on the defined NW B requirements. We can accept N=4.

Issue 4-1-3: Requirement when MGRP = 5.12s - Proposals - P1: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s could be defined. (CMCC vivo Huawei Apple Nokia) - P1-1: The new requirements for 5.12s could reuse corresponding requirements (number of DRX cycles) when DRX = 2.56s. (vivo Huawei Apple) - P2: RAN4 not need to discuss the requirement for MGRP=5.12s if the NW-B’s requirement is only related to NW-B’s DRX. (Ericsson) - P3: NW B requirements does not apply when MUSIM gap’s MGRP=5.12s (MTK) - P4: For MUSIM gap with 5.12s MGPR, whether to define related requirements depending on the discussion of 4-1-2 (vivo Qualcomm China Telecom) Recommendations:

If new option 2 in Issue 4-1-2 is agreed, there is no need to define additional NW B requirements when MGRP=5.12s. The relaxed requirements based on DRX cycle is enough.

Proposal 3: If new option 2 in Issue 4-1-2 is agreed, there is no need to define additional NW B requirements when MGRP=5.12s.

Issue 4-1-4: NW B inter-frequency and inter-RAT measurement - P1: Do not define inter-RAT measurement/evaluation/detection requirements of NW B. (Ericsson vivo Qualcomm China Telecom oppo Apple Nokia) - P2: Define NW B inter-frequency requirements (Ericsson) - P3: Do not define NW B inter-frequency requirements (vivo China Telecom Apple) - P4: Clarify the need for performing inter-frequency and inter-RAT measurement in NW-B; Clarify the need to for RAN4 to define UE requirements for NW-B inter-frequency and inter-RAT measurements. (Nokia) Agreement: NW B Inter-RAT requirements will not be defined. Recommendations: Continue discussion on whether NW B inter-frequency requirements need be defined or not

Consider that UE is in connected in NW A and in idle/inactive in NW B, measurement requirements of NW B will affect the configuration of MUSIM gaps. Performing intra-frequency measurement in NW B can already support most situations. If inter-frequency measurement requirements are to be further defined, it may exacerbate the impact on the performance of NW A. We support to not define inter-frequency measurement requirements for NW B.

Proposal 4: Support to not define inter-frequency measurement requirements for NW B.

3	Conclusion

In this paper we provided our views on NW B requirements for Rel-17 MUSIM gaps:

Proposal 1: Agree to define NW B measurement/cell reselection requirements in IDLE/inactive mode, and the inactive state requirement should be the same as NW B’s Idle state.

Proposal 2: Support new option 2 that NW-B’s requirement is not related to MGRP, and with a fixed scaling factor N based on the DRX cycle. And UE should request MUSIM gaps based on the defined NW B requirements. We can accept N=4.

Proposal 3: If new option 2 in Issue 4-1-2 is agreed, there is no need to define additional NW B requirements when MGRP=5.12s.

Proposal 4: Support to not define inter-frequency measurement requirements for NW B.

Reference

- R4-2317425, WF on R18 MUSIM, vivo