3GPP TSG RAN WG4 Meeting #109	R4-2319038

Chicago, US, November 13 – November 17, 2023

Agenda item:			8.10.2

Source:	vivo

Title:	On RRM requirements for BWP operation without restriction

Document for:	Discussion

- Introduction

In the RAN4#108bis meeting, there were further discussions on L1 and L3 measurements for options supporting BWP operation without restriction. Following agreements on option B-1-1, C and option B-1-2 were made and captured in the WF [1].

#### Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP for option A - It is RAN4 common understanding that timing requirements apply when the SSB is within the channel BW of the UE. #### Issue 3-3: Clarification on existing timing requirements for Option B-1-1 - It is RAN4 common understanding that timing requirements apply when the SSB is within the channel BW of the UE. #### Issue 3-4: Clarification on reporting of NeedForGap/NeedForGapNCSG for UE supporting option B-1-1 - For UE capable of supporting Option B-1-1 capability and additionally supporting NeedForGap or NeedForGapNCSG or NeedForInterruption” - UE shall report no gap and no interruption/no NCSG for intra-frequency measurement. #### Issue 4-2: Definition of intra-frequency handover and inter-frequency handover - Definition of intra-frequency handover is introduced for non-RedCap and it is to reuse the definition for handover in RedCap. #### Issue 4-3: Scenarios of handover for Option C <Online agreement> - Intra frequency handover from NCD-SSB to NCD-SSB - Inter frequency handover form NCD-SSB to CD-SSB - Inter frequency handover from CD-SSB to NCD-SSB - Inter frequency handover from NCD-SSB to NCD-SSB #### Issue 4-3-1: Requirements for applicable scenarios for handover for Option C - For UE supporting option C, the handover requirements are applicable for the scenarios - Handover to the first active DL BWP of the target cell associated with CD-SSB - Handover to the first active DL BWP of the target cell associated with NCD-SSB. #### Issue 4-6: CA requirements for UE supporting option C - The following scenario is supported from RAN4 requirement perspective: - NCD-SSB + CA, NCD-SSB only in PCell - Inform RAN1/2 the above agreement

In this contribution, we further provide our views on RRM requirements for option A, C, B-1-1 and B-1-2 for BWP operation without restriction.

- Discussion on Option C

- 3.1 Handover requirements

Good progress was made in the last meeting on handover requirements for option C. Following agreements were made.

#### Issue 4-2: Definition of intra-frequency handover and inter-frequency handover - Definition of intra-frequency handover is introduced for non-RedCap and it is to reuse the definition for handover in RedCap. #### Issue 4-3: Scenarios of handover for Option C <Online agreement> - Intra frequency handover from NCD-SSB to NCD-SSB - Inter frequency handover form NCD-SSB to CD-SSB - Inter frequency handover from CD-SSB to NCD-SSB - Inter frequency handover from NCD-SSB to NCD-SSB #### Issue 4-3-1: Requirements for applicable scenarios for handover for Option C - For UE supporting option C, the handover requirements are applicable for the scenarios - Handover to the first active DL BWP of the target cell associated with CD-SSB - Handover to the first active DL BWP of the target cell associated with NCD-SSB. #### Issue 4-6: CA requirements for UE supporting option C - The following scenario is supported from RAN4 requirement perspective: - NCD-SSB + CA, NCD-SSB only in PCell

Regarding how to specifying handover requirements for option C, the handover requirements for RedCap UE with 2 Rx antennas are reused as much as possible according to the revised WID.

#### Issue 4-1: Principle for specifying handover requirements for Option C - FFS - Option 1: - Existing handover requirements for RedCap UE with 2Rx are reused as baseline. Necessary changes can be made. - Option 2: - Existing handover requirements and handover scenarios of RedCap UE with 2Rx can be applied to option C. - Option 3: - The existing handover requirements defined in clause 6.1.1 (NR Handover) of TS 38.133 are also applicable for the HO when the NCD-SSB is in the active DL BWP of the source cell and the NCD-SSB is in the active DL BWP of the target cell.

The requirements for option C should be captured in the clause for legacy handover requirements rather than in the clauses for RedCap requirements.

Proposal 1: The handover requirements for option C should be captured in the clause for legacy handover requirements. Existing handover requirements for RedCap UE with 2Rx are reused as baseline as much as possible and necessary changes can be made.

#### Issue 4-4: Tsearch in handover requirements for known inter-frequency cell - FFS - Option 1: - For UE supporting option C, the handover requirements for known inter-frequency cell are specified as follows. - if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms; - if the measured SSB and the target SSB for handover belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms: - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover - NCD-SSB in DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover - Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs - Option 2: - For known inter-frequency target cell, three cases are included and Tsearch is N*Trs ms. - The measured SSB is a CD-SSB and the target SSB for HO is an NCD-SSB - The measured SSB is an NCD-SSB and the target SSB for HO is a CD-SSB - The measured SSB is an NCD-SSB and the target SSB for HO is another NCD-SSB #### Issue 4-5: Tsearch in handover requirements for other cases than known inter-frequency cell - FFS - Option 1: - Reuse the requirements for RedCap UE with 2Rx.

There are requirements for UE operating with 2Rx antennas being specified for RedCap.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell which depends on whether the target cell is already known when the handover command is received by the UE. - If the target cell is a known intra-frequency cell, Tsearch = 0ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna. - If the target cell is a known inter-frequency cell, then if the measured SSB is the target SSB for HO of the target cell, Tsearch = 0ms; if the measured SSB and the target SSB for HO belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms: - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in RedCap specific DL BWP is the target SSB for HO - NCD-SSB in RedCap specific DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for HO - Both measured SSB and the target SSB for HO are NCD-SSBs within different RedCap specific DL BWPs Otherwise, the target cell is an unknown inter-frequency cell. In this case, if the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms if the UE is operating with 2 Rx antennas; Tsearch = 5*Trs ms if UE is operating with only 1 Rx antenna.

Thus, the handover requirements for 2Rx antennas RedCap UE can be directly reused for non-RedCap UE.

The comparison of Redcap handover requirements and legacy handover requirements is summarized in the Table 1 below.

Table 1. Comparison of handover requirements for RedCap and non-RedCap

|  |  | RedCap requirements |  | Legacy requirements |
| --- | --- | --- | --- | --- |
|  |  | known intra-frequency cell |  | target cell is known (intra) |
|  |  | Tsearch = 0ms |  | Tsearch = 0ms |
|  |  | unknown intra-frequency cell |  | unknown intra-frequency cell |
|  |  | Tsearch = 1*Trs target cell Es/Iot≥-2 dB if UE is capable of 2 Rx antennas |  | Tsearch = 1*Trs target cell Es/Iot≥-2 dB |
|  |  | known inter-frequency cell |  | target cell is known (inter) |
| measured SSB | target SSB for HO of the target cell |  |  |  |
| same |  | Tsearch = 0ms |  | Tsearch = 0ms |
| CD-SSB | NCD-SSB | Tsearch = 1*Trs |  |  |
| NCD-SSB | CD-SSB |  |  |  |
| NCD-SSB | NCD-SSB |  |  |  |
|  |  | unknown inter-frequency cell |  | unknown inter-frequency cell |
|  |  | Tsearch = 3*Trs target cell Es/Iot≥-2 dB if UE is capable of 2 Rx antennas |  | Tsearch = 3*Trs target cell Es/Iot≥-2 dB |

It can be seen that missing requirements for non-RedCap UE supporting option C are the requirements for the known inter-frequency cases which are highlighted in yellow in the RedCap requirements and in Table 1.

If the SSB associated with first active DL BWP of the target cell is different from the measured SSB, then the UE needs one SSB for fine timing tracking even if it is known inter-frequency cell regardless of 2Rx UE or 1Rx UE. The same principle should be applied for option C.

For RedCap, the requirements are same for the three cases where measured SSB is different from target cell’s SSB associated with first active BWP regardless of NCD-SSB or CD-SSB. For non-RedCap UE supporting option C, all of three cases are supported and same requirements are defined.

Proposal 2: For UE supporting option C, the handover requirements for known inter-frequency cell are specified as follows.

if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms;

if the measured SSB and the target SSB for handover belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms:

-	CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover

-	NCD-SSB in a DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover

-	Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs

Proposal 3: For UE supporting option C, the handover requirements in terms of Tsearch for other cases than known inter-frequency cell are to reuse the requirements for RedCap UE with 2Rx.

- Discussion on Option B-1-2

The RF tuning time for option B-1-2 has not been decided.

#### Issue 5-2a: Interruption length (in ms) for option B-1-2 - FFS - Option 1: - Interruption length is 0.5ms for FR1 and 0.25ms for FR2. - Option 2: - RAN4 shall define the interruption length requirements the same as these defined for NCSG in Rel-17 (i.e., VIL=1 ms in FR1 and VIL=0.75 ms in FR2).

It was already agreed in the RAN4#107 meeting that interruption length is [0.5ms] for FR1 and [0.25ms] for FR2. When UE is performing RLM/BFD/BM(L1-RSRP) measurements based on SSB outside active BWP, it may cause interruptions due to RF tuning between larger BW and active BWP. It is reasonable to reuse RF retuning time for measurement gap. Option 2 has doubled RF retuning time and it would lead much longer interruptions than option 1, which would make option B-1-2 less useful.

Proposal 4: RF retuning time is 0.5ms for FR1 and 0.25ms for FR2 for option B-1-2.

Interruption length requirements will be based on RF retuning time. For sync case, it is rounded up to slot granularity. For async case, one additional slot is added.

Proposal 5: Requirements for interruption length is defined in slot granularity by rounding up RF retuning time and 1 additional slot is added for async case.

#### Issue 5-2c: Interruption ratio for option B-1-2 - FFS - Option 1: - For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. - Option 1a: - The interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to 0.5%. - The maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined: - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %. - The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %. - The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %. - Option 2: - For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP/L1-SINR) measurements based on SSB outside active BWP. - Option 2a: - The probability of missed ACK/NACK for a UE supporting Option B-1-2, due to interruptions caused by UE performing BM/RLM/BFD measurements based on SSB outside the active BWP, shall not exceed [1.0]%. - Option 3: - X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account. - Option 4: - RAN4 shall define interruption length for Option B-1-2 and RAN shall not define interruption ration for the same option.

Depending on UE implementation, it may cause interruptions on each L1 measurements. However, this may have big impact on system throughput. To achieve balance between system performance and UE power consumption, it is proposed that allowed probability of missed ACK/NACK for ALL RLM/BFD/BM(L1-RSRP) measurements based on SSB outside active BWP is 1%.

Proposal 6: For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP/L1-SINR) measurements based on SSB outside active BWP.

- Summary

In this contribution, we further provided our views on RRM requirements for all options for BWP operation without restriction. Based on above analysis, following proposals are present.

Proposal 1: The handover requirements for option C should be captured in the clause for legacy handover requirements. Existing handover requirements for RedCap UE with 2Rx are reused as baseline as much as possible and necessary changes can be made.

Proposal 2: For UE supporting option C, the handover requirements for known inter-frequency cell are specified as follows.

if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms;

if the measured SSB and the target SSB for handover belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms:

-	CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover

-	NCD-SSB in a DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover

-	Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs

Proposal 3: For UE supporting option C, the handover requirements in terms of Tsearch for other cases than known inter-frequency cell are to reuse the requirements for RedCap UE with 2Rx.

Proposal 4: RF retuning time is 0.5ms for FR1 and 0.25ms for FR2 for option B-1-2.

Proposal 5: Requirements for interruption length is defined in slot granularity by rounding up RF retuning time and 1 additional slot is added for async case.

Proposal 6: For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP/L1-SINR) measurements based on SSB outside active BWP.

- References

- R4-2317337	WF on support for bandwidth part operation without restriction, vivo