3GPP TSG RAN WG4 Meeting #109	R4-2319043

Chicago, US, November 13 – November 17, 2023

Agenda item:			8.7.2.3

Source:	vivo

Title:	Remaining issues on RLM and BFD/CBD requirements for FR2 multi-Rx

Document for:	Discussion

- Introduction

In the RAN4#108bis meeting, there were continued discussions on the RRM enhancement for RLM and BFD/CBD for FR2 multi-Rx chain DL reception. The agreements and open issues are captured in the WF [1]. Following agreements were made.

#### Issue 2-2-1: Updated conditions for TRP specific BFD requirements enhancement for multi-Rx - Both CSI-RSs are not in any CSI-RS resource set with repetition ON - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection [and both PDSCHs] are overlapped on the same OFDM symbol. - [The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDSCH] - Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report. - [FFS how to capture UE is activated with multi-Rx operation]

In this contribution, we further provide our views on remaining open issues for RLM and BFD/CBD requirements for FR2 multi-Rx chain DL reception.

- Discussion

- 2.1 Cell specific RLM and BFD/CBD

Issue 2-1-5: Other aspects of RLM and cell specific BFD/CBD requirements for multi-Rx

- FFS - Option 1a: - For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, without any clarification on multi-TRP operation. - Option 1b: - For mDCI scenario of intra-cell mTRP, no need to give any clarification or assumption on whether the RLM-RS originated from two TRPs or not. - Option 1c: - For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133. No need of additional clarification that RLM-RSes for evaluating out-of-sync and in-sync may originate from two different TRPs as RAN4 agreed to specify measurement restrictions. - Option 2: - Capture in the RLM requirements that in m-DCI scenario, RLM RS during the evaluation period for out-of-sync and in-sync may originate from one or two TRPs

Firstly, TRP specific procedure is only supported by link recovery procedure, which can be seen by the procedures defined in TS 38.213 as below.

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRS-List and candidateBeamRS-List2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1.

However, TRP specific procedure is not supported in radio link monitoring in TS 38.213. It is not reasonable to just define TRP specific requirements in TS 38.133 by clarifying that RLM-RSs for OOS and IS may come from two different TRPs. Moreover, UE behavior should be the same with or without the clarification.

Proposal 1: For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, without any clarification on multi-TRP operation.

- 2.2 TRP specific BFD/CBD

Issue 2-2-1: Updated conditions for TRP specific BFD requirements enhancement for multi-Rx

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection [and both PDSCHs] are overlapped on the same OFDM symbol. - [The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDSCH] - Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report. - [FFS how to capture UE is activated with multi-Rx operation]

The condition is for whether CSI-RS based TRP specific BFD measurement can be performed simultaneously for two TRPs. In essence, it would be almost the same as simultaneous measurement of different L1 measurements. Therefore, it is suggested to use similar conditions as for measurement restriction, which was discussed in [2].

The conditions for TRP specific BFD requirements enhancement for multi-Rx is proposed to be revised as below.

Proposal 2: PTRP=1 for CSI-RS based TRP specific BFD requirements for multi-Rx operation under following conditions,

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON

- The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection and both of the PDSCHs are on the same OFDM symbol(s), or either one of the CSI-RS resources in the two sets q ̅_0,0 or q ̅_0,1 and one of the PDSCHs with different QCL typeD are on the same OFDM symbol(s) when partially overlapping PDSCHs are scheduled.

- The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDSCH.

- Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

- Summary

In this contribution, we further provided views on remaining open issues for RLM and BFD/CBD requirements for FR2 multi-Rx chain DL reception. Following proposals and observations are present.

Proposal 1: For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, without any clarification on multi-TRP operation.

Proposal 2: PTRP=1 for CSI-RS based TRP specific BFD requirements for multi-Rx operation under following conditions,

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON

- The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection and both of the PDSCHs are on the same OFDM symbol(s), or either one of the CSI-RS resources in the two sets q ̅_0,0 or q ̅_0,1 and one of the PDSCHs with different QCL typeD are on the same OFDM symbol(s) when partially overlapping PDSCHs are scheduled.

- The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDSCH.

- Resources of the active TCI states for the PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

- References

- R4-2317332	WF on FR2 multi-RX operation RRM requirements (part 1), vivo

- R4-2319044 Remaining issues on scheduling and measurement restrictions for multi-Rx, vivo