3GPP TSG RAN WG4 Meeting #109	R4-2319062

Chicago, US, November 13th – 17th, 2023

Agenda item:	8.26.6.1

Source: 	vivo

Title: 	General discussion on NTN RRM requirements in above 10 GHz bands

Document for:	Discussion

- Introduction

According to the outcome in RAN plenary [1], RRM requirements for Ka band shall be defined based on the agreed scope. Correspondingly, the objective on defining RRM requirements for Fully electronically-steered beam UEs (Type 1) and Fully mechanically-steered beam UEs (Type 2) were captured in revised WID on NR NTN enhancements [2]. In this paper, we provide our views on general aspects for Ka band in NTN based on the progress reached in the last meeting [3].

- Discussion

2.1 UE uplink timing accuracy requirements

In the last meeting, RAN4 initially discussed the exact value of Te_NTN and values assumed for X and Y’. To move forward, companies would provide the analysis results based on the following criterion in this meeting. Next, we would like to provide our analysis.

### Issue 1-6: Te_NTN for 60kHz and 120kHz Agreement: Companies should provide ‘the exact value of Te_NTN and values assumed for X and Y’ and ‘the analysis result based on the following criterion.’ Otherwise, the values/proposals won’t be captured in the list of options. Tg = 0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm): an effective guard period in CP - Tcp: a length of CP for the given SCS of UL channel/signal - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs) - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error - Tr: TAC resolution error (from TS38.213) - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133) - Tf: an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm - Tm: a margin needed at gNB receiver to accommodate any additional impairments if needed. - If a non-zero value is assumed in the proposal for Tm, the source of the impairments shall be provided too. Technical analysis is required if any number will be provided for each of the components in the next meeting. Whether the same or different values for different channels is contribution driven.

Here we would like to calculate is the maximum total positioning error Tg that gNB which can tolerate with enough accuracy.

If we let the inequality become:

Tg = 0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm) > 0,

The maximum value of Tg shall be:

Tp = 0.5*Tcp – (Td + Tr + Ta + Tf + Tm)

Further, each component is shown above.

| UL=60kHz DL=120kHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Component | Tcp | Td | Tr | Ta | Tf | Tp | Te |
| value | 36Ts | 1.1Ts | 2Ts | 2Ts | 0.5Ts | 12.4Ts | 14Ts |
| Calculation |  | Td/Ts = 0.00347/0.032 = 1.1 | (8/2u)*Ts = 2Ts | 128Tc=2Ts | 16ns =0.5Ts | Tp=0.5*Tcp – (Td + Tp + Tr + Ta + Tf) | Te=Td+Tf+Tp |
|  |  |  |  |  |  | Total pos error = 60 m |  |
| UL=120kHz DL=120kHz |  |  |  |  |  |  |  |
| Component | Tcp | Td | Tr | Ta | Tf | Tp | Te |
| value | 18Ts | 1.1Ts | 1Ts | 0.5Ts | 0.5Ts | 5.9Ts | 7.5Ts |
| Calculation |  | Td/Ts = 0.00347/0.032 = 1.1 | (8/2u)*Ts = 1Ts | 32Tc=0.5Ts | 16ns =0.5Ts | Tp=0.5*Tcp – (Td + Tp + Tr + Ta + Tf) | Te=Td+Tf+Tp |
|  |  |  |  |  |  | Total pos error≈28 m |  |
| UL=60kHz DL=240kHz |  |  |  |  |  |  |  |
| Component | Tcp | Td | Tr | Ta | Tf | Tp | Te |
| value | 36Ts | 0.55Ts | 2Ts | 2Ts | 0.5Ts | 13Ts | 14Ts |
| Calculation |  | Td/Ts = 0.0017/0.032 = 0.55 | (8/2u)*Ts = 2Ts | 128Tc=2Ts | 16ns =0.5Ts | Tp=0.5*Tcp – (Td + Tp + Tr + Ta + Tf) | Te=Td+Tf+Tp |
|  |  |  |  |  |  | Total pos error≈62 m |  |
| UL=120kHz DL=240kHz |  |  |  |  |  |  |  |
| Component | Tcp | Td | Tr | Ta | Tf | Tp | Te |
| value | 18Ts | 0.55Ts | 1Ts | 0.5Ts | 0.5Ts | 6.45Ts | 7.5Ts |
| Calculation |  | Td/Ts = 0.0017/0.032 = 0.55 | (8/2u)*Ts = 1Ts | 32Tc=0.5Ts | 16ns =0.5Ts | Tp=0.5*Tcp – (Td + Tp + Tr + Ta + Tf) | Te=Td+Tf+Tp |
|  |  |  |  |  |  | Total pos error≈30 m |  |

Observation 1: For UL SCS = 60 kHz, a UE with zero implementation margin is allowed the error 12.4 to 13 Ts, which corresponds to a total position error of 60 to 62 m to distribute among satellite and UE. The Te value = 14 Ts.

Observation 2: For UL SCS = 120 kHz, a UE with zero implementation margin is allowed the error 5.9 to 6.4 Ts, which corresponds to a total position error of 28 to 30 m to distribute among satellite and UE. The Te value = 7.5 Ts.

Proposal 1: The exact value of Te_NTN and values assumed for X and Y is based on the following analysis result:

| UL=60kHz DL=120kHz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Component | Tcp | Td | Tr | Ta | Tf | Tp | Te |
| value | 36Ts | 1.1Ts | 2Ts | 2Ts | 0.5Ts | 12.4Ts (total pos error = 60m) | 14Ts |
| UL=120kHz DL=120kHz |  |  |  |  |  |  |  |  |
| value | 18Ts | 1.1Ts | 1Ts | 0.5Ts | 0.5Ts | 5.9Ts (total pos error = 28m) | 7.5Ts |
| UL=60kHz DL=240kHz |  |  |  |  |  |  |  |  |
| value | 36Ts | 0.55Ts | 2Ts | 2Ts | 0.5Ts | 13Ts (total pos error = 62m) | 14Ts |
| UL=120kHz DL=240kHz |  |  |  |  |  |  |  |  |
| value | 18Ts | 0.55Ts | 1Ts | 0.5Ts | 0.5Ts | 6.4Ts (total pos error = 30m) | 7.5Ts |

Furthermore, for requirements on UE transmit timing for Satellite Access in above 10 GHz bands, the NTA,offset and its corresponding value also need to be specified.

As shown in following table, for the requirements on UE transmit timing for Satellite Access in FR1, the NTA,offset is referred to the one in existing requirements on UE transmit timing for TN in Table 7.1.2-2. Similarly, the NTA,offset in FR2 NTN can also reuse the value of NTA,offset for FR2 defined in requirements on UE transmit timing for FR2 TN.

- Existing requirements on UE transmit timing for Satellite Access in FR1: The value of NTA-offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA-offset is defined in Table 7.1.2-2. - Existing requirements on UE transmit timing for TN Table 7.1.2-2: The Value of Frequency range and band of cell used for uplink transmission; (Unit: TC). FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case; 25600 (Note 1). FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case; 0 (Note 1). FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case; 39936 (Note 1). FR2; 13792. Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void.

Proposal 2: For requirements on UE transmit timing for Satellite Access in above 10 GHz bands, the NTA,offset and its corresponding value shall refer to NTA-offset for TN in FR2 defined in Table 7.1.2-2.

2.2 UE feature list

For this part, we would like to provide UE feature for NTN enhancement for further discussion. Specifically, feature#40-3 is to support NTN UE capable of VSAT communicating with mechanical steering antenna.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh parallelMeasurementWithoutRestriction-fr2-r18 | 40-1 | Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. [The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-18).] |  | YES | NO | UE doesn’t support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation of serving cell without scheduling restrictions. | Per Band | FDD only | FR2 only | NO |  | Optional with capability signalling |
| 40. NR_NTN_enh maxNumber-NGSO-SatellitesWithinOneSMTC-fr2-r18 | 40-2 | Parallel measurements on multiple NGSO satellites within a SMTC | Support of simultaneously measurements on target cells [belonging to different NGSO satellites within a SMTC] |  | YES | NO | UE doesn’t support of simultaneously measurements on target cells | Per Band | FDD only | FR2 only | NO |  | Optional with capability signalling |
| 40. NR_NTN_enh UEType-MechanicalSteering-r18 | 40-3 | Support NTN UE capable of VSAT communicating with mechanical steering antenna | Support NTN UE capable of VSAT communicating with mechanical steering antenna |  | YES | NO | UE is not capable of VSAT communicating with mechanical steering antenna | Per UE | FDD only | FR2 only | NO |  | Optional with capability signalling |

- Conclusions

Observation 1: For UL SCS = 60 kHz, a UE with zero implementation margin is allowed the error 12.4 to 13 Ts, which corresponds to a total position error of 60 to 62 m to distribute among satellite and UE. The Te value = 14 Ts.

Observation 2: For UL SCS = 120 kHz, a UE with zero implementation margin is allowed the error 5.9 to 6.4 Ts, which corresponds to a total position error of 28 to 30 m to distribute among satellite and UE. The Te value = 7.5 Ts.

Proposal 1: The exact value of Te_NTN and values assumed for X and Y is based on the following analysis result:

| UL=60kHz DL=120kHz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Component | Tcp | Td | Tr | Ta | Tf | Tp | Te |
| value | 36Ts | 1.1Ts | 2Ts | 2Ts | 0.5Ts | 12.4Ts (total pos error = 60m) | 14Ts |
| UL=120kHz DL=120kHz |  |  |  |  |  |  |  |  |
| value | 18Ts | 1.1Ts | 1Ts | 0.5Ts | 0.5Ts | 5.9Ts (total pos error = 28m) | 7.5Ts |
| UL=60kHz DL=240kHz |  |  |  |  |  |  |  |  |
| value | 36Ts | 0.55Ts | 2Ts | 2Ts | 0.5Ts | 13Ts (total pos error = 62m) | 14Ts |
| UL=120kHz DL=240kHz |  |  |  |  |  |  |  |  |
| value | 18Ts | 0.55Ts | 1Ts | 0.5Ts | 0.5Ts | 6.4Ts (total pos error = 30m) | 7.5Ts |

Proposal 2: For requirements on UE transmit timing for Satellite Access in above 10 GHz bands, the NTA,offset and its corresponding value shall refer to NTA-offset for TN in FR2 defined in Table 7.1.2-2.

- Reference

[1] RP-232694 WF on potential RRM objectives for NR-NTN deployment in above 10 GHz (NR-NTN-enh WID), THALES

[2] RP-232669 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, THALES

[3] RP-2317374 RRM WF and CR work split for NR_NTN_enh, Qualcomm Incorporated