3GPP TSG-RAN WG4 Meeting # 109	R4-2319110

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1876 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-1: Time mask for switching across three or four uplink bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, Hisilicon, CMCC, Xiaomi, China Unicom, vivo, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Rel-18 time mask requirements for switching across three or four uplink bands are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Rel-18 time mask requirements for switching across three or four uplink bands. Resubmit the CR R4-2317608 endorsed in RAN4#108bis, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The time mask requirements of Rel-18 NR_MC_enh-Core WI are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sub-clauses 5.5A.3, 6.3A.3.3.6, 6.3C.3.5, and sub-clause 5.5C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of first change>

5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

5.5A.3.0	General

For the NR inter-band CA configurations in sub-clause 5.5A.3, when the capability [BandCombination-UplinkTxSwitch-r18] is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding inter-band CA requirements with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR inter-band CA configurations, according to the capability [uplinkTxSwitchingOptionForBandPair],

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with uplink assigned to one band on band X or band Y apply;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with uplink CA between the two uplink bands apply. For a UE supporting  [BandOrdering1T1Tto1T1T] for parallel uplink transmission switching for a band combination consisting with four different bands, the UE is allowed to report capability [preferredBandPairs] via band-ordering approach to indicate the UE’s preferred switching band pairs for which it supports dualUL and perform the switching case configured by network.

<End of first change>

<start of second change>

5.5C	Configurations for SUL

The configuration tables for SUL describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5. , where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5 and BCS5 shall not be indicated together with BCS4 for a SUL configuration. For SUL band combinations including FR1 intra-band CA and with BCS4 or BCS5, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5.

For the NR SUL band configurations with inter-band CA in sub-clause 5.5C, when the capability [BandCombination-UplinkTxSwitch-r18] is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding requirements for SUL band configurations with inter-band CA and with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR SUL band configurations with inter-band CA, according to the capability [uplinkTxSwitchingOptionForBandPair],

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink assigned to one band on band X or band Y apply;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink CA between the two uplink bands apply.

For SUL band configuration with inter-band CA, band pair(s) of two non-SUL bands with switchedUL or dualUL by the parameter [uplinkTxSwitchingOption] is supported, and any other band pair(s) including SUL with switchedUL is supported, in release 18.

<end of second change>

<start of third change>

6.3A.3.3.6	Time mask for switching across three or four uplink bands

The switching time mask requirements specified in this sub-clause are applicable for an NR inter-band CA configuration when the capability [BandCombination-UplinkTxSwitch-r18] is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10].

In the NR inter-band CA configuration, the number of NR uplink bands is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability [FeatureSetUplinkPerCC].

The switching time masks in Figure 6.3A.3.3.6-1 and Figure 6.3A.3.3.6-2 are applicable to each of the uplink band pairs in the CA configuration, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter [switchingOptionConfigForBandPair-r18]. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s),

–	if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair;

–	if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair;

–	if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.

For each band pair, the switching periods described in Figure 6.3A.3.3.6-1 and Figure 6.3A.3.3.6-2 are located in either NR band X or band Y as indicated in RRC signalling [uplinkTxSwitchingBandList-r18] [7]. For each band pair, the length of uplink switching period X is indicated by UE capability [switchingPeriodFor1T-r18] when 1Tx-1Tx switching or 1Tx-2Tx switching between the two bands in the band pair is supported and configured, or is indicated with [switchingPeriodFor2T-r18] when 2Tx-2Tx switching between the two bands in the band pair is supported and configured. UE shall be capable to transmit until the beginning of the switching period and after the end of switching period with the exception of transient periods.

Figure 6.3A.3.3.6-1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 6.3A.3.3.6-2: Time mask for switching between band X and band Y, where the switching period is located in band Y

The following applies for the uplink switching cases specified in Figure 6.3A.3.3.6-1 and 6.3A.3.3.6-2 in a band pair with [switchingOptionConfigForBandPair-r18] set to either switchedUL or dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period of X indicated by UE capability [uplinkTxSwitchingPeriodForBandPair-r18] on any of the carriers band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

In addition to the requirements in Figure 6.3A.3.3.6-1 and Figure 6.3A.3.3.6-2, the requirements in Figure 6.3A.3.3.6-3 to 6.3A.3.3.6-5 are applicable when dualUL are supported on certain band pair(s) in the CA configuration.

The switching time masks in Figure 6.3A.3.3.6-3 and Figure 6.3A.3.3.6-4 are applicable when dualUL is supported for at least two uplink band pairs in the CA configuration. The two band pairs supporting dualUL are denoted as band pairs of {band X and band Z} and {band Y and band Z}. When one transmitter is switched between band X and band Y,

–	if the UE indicates the band Z as non-affected band with [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18] for band pair [X, Y], UE shall be capable of uplink transmission on band Z during the switching period that is located on band X, and UE is not required to transmit on band X and Y during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3A.3.3.6-3;

–	if the UE does not indicate [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18], UE is not required to transmit on any of the three bands during the switching period during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3A.3.3.6-4;

–	if the UE does not indicate [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18] and if UE additionally reports the capability  [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] as optional UE behaviour, UE is not required to transmit on any of the three bands during the switching period indicated by UE capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] located on band X, as shown in Figure 6.3A.3.3.6-4.

In Figure 6.3A.3.3.6-3 and Figure 6.3A.3.3.6-4, the uplink transmission on band X, band Y and band Z are all with one transmit antenna connector and one antenna port.

Figure 6.3A.3.3.6-3: Time mask for one transmitter switching between band X and band Y, and UE is capable of uplink transmission on band Z during the switching period

The following applies for the uplink switching cases specified in Figure 6.3A.3.3.6-3 in a band pair with [switchingOptionConfigForBandPair-r18] set to dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period X indicated by UE capability [uplinkTxSwitchingPeriodForBandPair-r18] on any of the carriers in band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

Figure 6.3A.3.3.6-4: Time mask for one transmitter switching between band X and band Y, and UE is not capable of uplink transmission on band Z during the switching period

The switching time mask in Figure 6.3A.3.3.6-5 is applicable when dualUL is supported for at least one uplink band pair including band X and band Y, and two transmit antenna connectors are supported on at least one uplink band of band Z. When one transmitter is switched between band X and band Z, and across the same time, the other transmitter is switched between Y and band Z, the switching time mask in Figure 6.3A.3.3.6-5 is applicable.

–	As baseline UE behaviour, UE is not required to transmit on any of the three bands during time period with the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of band X and band Z, and T3 is the length of switching period for the band pair of band Y and band Z.

–	As optional UE behaviour when UE additionally reports the capability [UplinkTxSwitchingAdditionalPeriodDualUL-r18], UE is not required to transmit on any of the three bands during time period indicated by UE capability [UplinkTxSwitchingAdditionalPeriodDualUL-r18].

In Figure 6.3A.3.3.6-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports. The switching period location is configured according to [TS 38.331], and band Z is with the highest priority according to the RRC configuration.

Figure 6.3A.3.3.6-5: Time mask for one transmitter switching between band X and band Z, and one transmitter switching between band Y and band Z

The following applies for the uplink switching case specified in Figure 6.3A.3.3.6-5 and with uplinkTxSwitchingOptionForBandPair-r18 set to dualUL for at least one band pair

-	if uplink switching on a band pair is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the maximum of the lengths of uplink switching periods indicated by UE capability [uplinkTxSwitchingPeriodForBandPair-r18] on any of the carriers in band X, band Y and band Z before T0 on any switched-to carrier

-	the configuration of the location of the switching period and the priority of bands in the uplinkTxSwitchingBandList are ignored by the UE

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the switched-from carrier(s) before any T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s)

The requirements in this sub-clause apply for the case of synchronized network deployment for the uplink bands.

<end of third change>

<start of forth change>

6.3C.3.5	Time mask for switching across three or four uplink bands

The switching time mask requirements specified in this sub-clause are applicable for an NR SUL band configuration with inter-band CA when the capability [BandCombination-UplinkTxSwitch-r18] is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10].

In the NR SUL band configuration with inter-band CA, the number of uplink bands with different carrier frequencies is three or four.  NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability [FeatureSetUplinkPerCC].

The switching time masks in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are applicable to each of the uplink band pairs in the SUL band configuration with inter-band CA, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter [switchingOptionConfigForBandPair-r18]. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s),

–	if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair;

–	if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair;

–	if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.

For each band pair, the switching periods described in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are located in either NR band X or band Y as indicated in RRC signalling [uplinkTxSwitchingBandList-r18] [7]. For each band pair, the length of uplink switching period X is indicated by UE capability [switchingPeriodFor1T-r18] when 1Tx-1Tx switching or 1Tx-2Tx switching between the two bands in the band pair is supported and configured, or is indicated with [switchingPeriodFor2T-r18] when 2Tx-2Tx switching between the two bands in the band pair is supported and configured. UE shall be capable to transmit until the beginning of the switching period and after the end of switching period with the exception of transient periods.

Figure 6.3C.3.5-1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 6.3C.3.5-2: Time mask for switching between band X and band Y, where the switching period is located in band Y

The following applies for the uplink switching cases specified in Figure 6.3C.3.5-1 and 6.3C.3.5-2 in a band pair with [switchingOptionConfigForBandPair-r18] set to either switchedUL or dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period of X indicated by UE capability [uplinkTxSwitchingPeriodForBandPair-r18] on any of the carriers band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

In addition to the requirements in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2, the requirements in Figure 6.3C.3.5-3 to 6.3C.3.5-5 are applicable when dualUL are supported on certain band pair(s) in the SUL band configuration with inter-band CA.

The switching time masks in Figure 6.3C.3.5-3 and Figure 6.3C.3.5-4 are applicable when dualUL is supported for at least two uplink band pairs in the band configuration. The two band pairs supporting dualUL are denoted as band pairs of {band X and band Z} and {band Y and band Z}. When one transmitter is switched between band X and band Y,

–	if the UE indicates the band Z as non-affected band with [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18] for band pair [X, Y], UE shall be capable of uplink transmission on band Z during the switching period that is located on band X, and UE is not required to transmit on band X and Y during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.5-3;

–	if the UE does not indicate [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18], UE is not required to transmit on any of the three bands during the switching period during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.5-4;

–	if the UE does not indicate [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18] and if UE additionally reports the capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] as optional UE behaviour, UE is not required to transmit on any of the three bands during the switching period indicated by UE capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] located on band X, as shown in Figure 6.3C.3.5-4.

In Figure 6.3C.3.5-3 and Figure 6.3C.3.5-4, the uplink transmission on band X, band Y and band Z are all with one transmit antenna connector and one antenna port.

Figure 6.3C.3.5-3: Time mask for one transmitter switching between band X and band Y, and UE is capable of uplink transmission on band Z during the switching period

The following applies for the uplink switching cases specified in Figure 6.3C.3.5-3 in a band pair with [switchingOptionConfigForBandPair-r18] set to dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period X indicated by UE capability  [uplinkTxSwitchingPeriodForBandPair-r18] on any of the carriers in band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

Figure 6.3C.3.5-4: Time mask for one transmitter switching between band X and band Y, and UE is not capable of uplink transmission on band Z during the switching period

The switching time mask in Figure 6.3C.3.5-5 is applicable when dualUL is supported for at least one uplink band pair including band X and band Y, and two transmit antenna connectors are supported on at least one uplink band of band Z. When one transmitter is switched between band X and band Z, and across the same time, the other transmitter is switched between Y and band Z, the switching time mask in Figure 6.3C.3.5-5 is applicable.

–	As baseline UE behaviour, UE is not required to transmit on any of the three bands during time period with the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of band X and band Z, and T3 is the length of switching period for the band pair of band Y and band Z.

–	As optional UE behaviour when UE additionally reports the capability [UplinkTxSwitchingAdditionalPeriodDualUL-r18], UE is not required to transmit on any of the three bands during time period indicated by UE capability [UplinkTxSwitchingAdditionalPeriodDualUL-r18].

In Figure 6.3C.3.5-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports. The switching period location is configured according to [TS 38.331], and band Z is with the highest priority according to the RRC configuration.

Figure 6.3C.3.5-5: Time mask for one transmitter switching between band X and band Z, and one transmitter switching between band Y and band Z

The following applies for the uplink switching case specified in Figure 6.3C.3.5-5 and with uplinkTxSwitchingOptionForBandPair-r18 set to dualUL for at least one band pair

-	if uplink switching on a band pair is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the maximum of the lengths of uplink switching periods indicated by UE capability [uplinkTxSwitchingPeriodForBandPair-r18] on any of the carriers in band X, band Y and band Z before T0 on any switched-to carrier

-	the configuration of the location of the switching period and the priority of bands in the uplinkTxSwitchingBandList are ignored by the UE

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the switched-from carrier(s) before any T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s)

The requirements in this sub-clause apply for the case of synchronized network deployment for the uplink bands.

<end of forth change>