3GPP TSG-RAN WG4 Meeting #109		R4- 2319134

Chicago, US, 13th – 17th November 2023

Agenda item:	8.34.3.1

Source:	Intel Corporation

Title:	NR network energy saving RRM aspects – other aspects

Document for:	Discussion

# Introduction

Following the SI of NR network energy saving, a WI is approved in RANP#98-e in Dec. 2022. The WID can be seen in [1]. The objectives are copied in the below box.

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

In this paper we provide our views on RRM aspects that are related to the discussions within WI scope.

# Discussion

## Cell DTX/DRX

### RAN2 discussions

This part of the objectives is handled mainly in RAN2. The below box shows the agreements in RAN2 in the last few WG meetings.

Agreements from RAN2#121 meeting There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX). No further enhancements for CBRA and CFRA will be pursued. Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell. FFS whether we have DTX UE specific inactivity timer. FFS on configuration signaling and stage 3. Confirm study item agreement that we can have separate DTX and DRX configuration. We will focus on designing DTX/DRX for at least single configuration. FFS whether multiple configuration of cell DTX or DRX will be supported

Agreements from RAN2#121bis meeting A periodic cell DTX/DRX configuration is explicitly signalled to the UEs. A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling. The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration. As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released. From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details. Ask about feasibility and reliability of using L1 signaling. Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration. Extract a few key benefits of dynamic signaling from email discussion and online discussions As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH. For the FFS case there may be some exceptions. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions

Agreements from RAN2#122 meeting UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy. UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy. Working assumption: When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX. Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX). The behavior is captured in stage 2 spec When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy). This includes DL HARQ feedback.

Agreements from RAN2#123 meeting Activation/deactivation is per serving cell. FFS if the configuration is per cell or per MAC entity RAN2 will reuse the start timer formula of the onDurationTimer from UE C-DRX (including SlotOffset) to specify the start of cellDTX-onDurationTimer (and cellDRX-onDurationTimer) in 38.321. The gNB should ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration. It is up to network implementation to ensure the alignment. We will capture this in stage 2 specification. Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other. FFS if we anything needs to be specified in stage 3 (i.e. in IE description) As a baseline legacy C-DRX reconfiguration is used to change UE C-DRX configuration once Cell DTX/DRX is activated/deactivated. RAN2 specifies cellDTX-onDurationTimer (and cellDRX-onDurationTimer) to have the same value range as UE C-DRX on-duration timer. RAN2 specifies cellDTX-Cycle (and cellDRX-Cycle) to have the same value range as UE C-DRX Long cycle. Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX) On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured. FFS if we have different start offset configuration for cell DTX and cell DRX RAN2 will not introduce a MAC CE for cell DTX/DRX (de)activation. Confirm working assumption, when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX. We focus on the case where DTX in RRC can only be configured when C-DRX is configured. We will not optimize for the case where C-DRX is not configured.

Agreements from RAN2#123bis meeting Cell DTX/DRX configuration is provided per Serving Cell with the following restrictions: - A maximum of two cell DTX/DRX patterns can be configured per MAC entity - The two configured patterns are aligned, - The start and slot offset are common for the two patterns. - one periodicity is an integer multiple of the other. Working assumption: UE triggers RACH upon determining that an emergency call is initiated during the cell DTX/DRX non active period. We rely on the UE implementation to determine whether an emergency call is initiated. We will take time to check until next meeting to confirm the WA. Introduce explicit activation/deactivation in RRC once DTX/DRX is configured (i.e. not for dynamic activation/deactivation). This reverses previous agreement on implicit activation. Start offset and slot offset configuration is also common between Cell DTX and Cell DRX when both are configured Standalone cell DRX configuration is possible to configure Multiple configurations of Cell DTX/DRX are not pursued in Rel-18 for serving cell. The case that Cell DRX activation is received between delivering a configured grant to the HARQ entity and HARQ processing for the CGO will not be addressed by RAN2, as it is not valid for the MAC model.

### RAN1 discussions

RAN1 had also discussed cell DTX/DRX for a few meetings. The below box captures all the available RAN1 agreements so far.

Agreement - RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX - Further discussions on other aspects are not precluded Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Agreement Study L1 signalling for enhancing cell DTX/DRX including activation/deactivation for a single configuration which will have the following characteristics: - PDCCH based signaling - FFS: Whether enhancing legacy DCI or introducing new DCI - FFS: DCI content - FFS: Whether L1 signaling is UE specific DCI or group common DCI - FFS: Timer or validity duration based activation/deactivation of cell DTX/DRX - FFS: whether to specify a reference time for activation/deactivation of cell DTX/DRX - FFS: If multiple Cell DTX/DRX patterns are to be supported - FFS on detailed UE behavior upon reception of L1 signaling at least including application delay - FFS how to guarantee reliability of the L1 signaling - FFS whether the L1 signal can be monitored in non-active periods. Agreement From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - FFS: SRS for positioning - FFS: - HARQ feedback for SPS PDSCH - FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX - FFS Whether the listed signals/channels can be configurable by gNB - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Further study the following in RAN1: - Handling of HARQ-ACK codebook generation when configured with cell DTX/DRX - Handling of PUCCH deferral operation during non-active periods of cell DRX - Handling of overlapping channels where a least a channel overlaps with non-active periods of cell DTX/DRX - Handling of signals/channels that can be received/transmitted repeatedly during non-active periods of cell DTX/DRX - Handling of PUCCH switching during non-active period to an active cell - Other enhancements are not precluded. Agreement For PDDCH monitoring, further work on Rel-18 NES in RAN1 is to follow the RAN2 agreement below: 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). Working Assumption - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective. - This does not imply that L1 activation/deactivation is supported in Rel-18\ Note: Reliability, overhead, and benefits are FFS Agreement DCI format 2_X, for activation and deactivation of cell DTX and DRX configuration, - at least includes following fields, - N information block field(s), - Spare/reserved padding bits to match the size configured for DCI 2_X (if needed) - payload size is configurable and within the bounds set by existing RAN1 specification - an information block field contains signaling of activation or deactivation of ‘a configuration of cell DTX and/or DRX’ of ‘a serving cell’ - for serving cell configured with SUL, the same bit is applicable for both NUL and SUL Above applies at least for sTRP case. Agreement For at least the case where one cell DTX/DRX pattern is configured, an information block field of DCI format 2_X for activation and deactivation of cell DTX and DRX configuration supports the following: - Separate (activation/deactivation) signaling for cell DTX and cell DRX, i.e. one activation/deactivation signaling sub-field for cell DTX configuration and one activation/deactivation signaling sub-field for cell DRX configuration - Separate 1 bit indication for each of activation/deactivation for one cell DTX and one cell DRX Above does not imply that multiple DTX/DRX patterns is not supported. Agreement Support new RNTI (e.g. nes-RNTI) which is configured by higher layer, for scrambling of DCI format 2_X Agreement From RAN1 point of view, DCI format 2_X supports activation/deactivation of cell DTX/DRX configuration of multiple serving cells and support activation/deactivation per cell - UE monitor DCI format 2_X in one serving cell Agreement Delay that is applied after DCI Format 2_X reception that activate/deactivate cell DTX/DRX configuration is introduced in Rel-18. Agreement DCI format 2_X is monitored in the common search space Note: Search space set configuration for DCI format 2_X is separately provided by higher layers Agreement The following high layer signaling are to be included to the RRC parameter list for new DCI format 2_X for activation and deactivation of cell DTX/DRX - search space set configuration with new DCI format 2_X - DCI size for new DCI format 2_X Agreement - An information block field of DCI format 2_X is variable size either 1 or 2 bits. - Higher layer signaling configures whether the activation/deactivation of cell DTX and/or cell DRX is indicated in DCI format 2_X for a serving cell. - If both cell DTX and cell DRX are configured for a serving cell, - 1st bit corresponds to activation/deactivation of cell DTX configuration, and - 2nd bit corresponds to activation/deactivation of cell DRX configuration, - otherwise, the 1 bit corresponds to the configured cell DTX or cell DRX configuration. - Note: this does not imply there may be separate higher layer signaling to enable L1 signaling based activation/deactivation for a cell DTX and/or cell DRX configuration. Signaling design is up to RAN2. Agreement For each serving cell configured with L1 signaling based activation/deactivation of cell DTX and/or cell DRX configuration, starting bit position of an information block of DCI format 2_X is provided by UE specific higher layer signaling. Agreement - UE is expected to apply cell DTX or DRX activation/deactivation change at beginning of the slot X where the SCS of slot X is with respect to the active DL or UL BWP of the serving cell, respectively. - Slot X is the first slot whose beginning is no earlier than (i.e., same or after) beginning of slot n + D, where D is the delay and n is the slot containing the PDCCH of DCI format 2_X based on SCS of PDCCH. SCS of PDCCH (kHz); Value of D (in unit of slot). 15; 3. 30; 6. 60; 12. 120; 24. 480; 96. 960; 192. Agreement Rel-18 UE supporting cell DTX is not required to monitor the following signals/channels from the gNB, during non-active periods of cell DTX - PDCCHs associated with DCI format 2_0 – DCI Format 2_5 Conclusion: - HARQ-ACK of SPS PDSCH transmitted is not impacted by non-active period of cell DRX. Agreement For the FFS from agreement from RAN1 #112bis - SRS for positioning is not impacted by cell DRX operation. Conclusion - The following channels are not impacted by non-active period of cell DRX - HARQ-ACK of a DCI format without scheduling a PDSCH Agreement Send LS to RAN2 to ask to consider the following RAN1 agreements and ask RAN2 to capture them in RAN2 specification appropriately. - Agreement (from RAN1 #114) - Rel-18 UE supporting cell DTX is not required to monitor the following signals/channels from the gNB, during non-active periods of cell DTX - PDCCHs associated with DCI format 2_0 – DCI Format 2_5 - Conclusion: - HARQ-ACK of SPS PDSCH transmitted is not impacted by non-active period of cell DRX. - Conclusion - The following channels are not impacted by non-active period of cell DRX - HARQ-ACK of a DCI format without scheduling a PDSCH - Part of the Agreement (from RAN1 #112-bis-e) - From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. - Periodic/Semi-persistent CSI report Include a note saying that for the conclusions, RAN1 does not expect any specification impact. Final LS is endorsed in R1-2310476. Agreement For CSI report associated with P/SP CSI-RS resource and configured with reportQuantity including RI, when cell DTX is configured - the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each P/SP CSI-RS resource for channel measurement and/or interference measurement for the CSI report in cell DTX active period no later than CSI reference resource and drops the report otherwise. Agreement Cell DTX/DRX operation is only supported for sTRP.

### RAN4 impact

The idea of part of the discussions so far in RAN1 and RAN2 is to try to align the cell DTX/DRX to existing UE DRX configurations so that all the individual UE-s will follow the cell DTX/DRX cycle in order to generally save energy at the network side. The detailed impact to RRM specification is not clear at the moment. We recommend to further wait until further design is agreed.

RAN1 has not reached consensus on any muting for PRS or CSI-RS yet so it is not time yet we should discuss about any other impact brought on positioning or CSI-RS measurement requirements.

## Connected mode mobility

### RAN2 discussions

This part of the objectives is also handled mainly in RAN2. The below box shows the agreements in RAN2 in the last few WG meetings.

Agreements from RAN2#121bis meeting RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”. FFS further details For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode” As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration. FFS what trigger is used for execution of CHO

Agreements from RAN2#122 meeting We will have a CHO solution that considers NES mode of at least source cell. We can have a specific NES CHO execution condition based on source cell NES mode. FFS how the UE determines is in NES mode. FFS on how this is achieved in RRC We will not introduce new L1 signalling for the purpose of CHO Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario. We will study the time based mechanism

Agreements from RAN2#123 meeting We will support the CHO triggers for the use case of turning off the cell (At least for cell DTX/DRX) Time-based CHO is not to be considered in NES. Do not consider using an indication in SIB1 for triggering NES CHO execution condition

Agreements from RAN2#123bis meeting Group common DCI format 2-X is reused to notify the UE that source cell is entering NES mode. - add one bit of DCI 2-X to trigger both use cases of Cell DTX/DRX activation and cell turning off. RAN2 send LS to RAN1 to request this signaling change. RAN2 will not specify anything related to target cell NES mode for CHO.

### RAN4 impact

The idea of part of the discussions so far in RAN2 is to try to introduce a new CHO triggering mechanism so that handover can be performed correctly according to the NES status of at least the source cell. The detailed impact to RRM specification is not clear at the moment. We recommend to further wait until further design is agreed.

One of the observations we have about RRM impacts is that the exact condition to trigger the handover may need to be reflected in the RAN4 spec and corresponding test cases are to be introduced accordingly.

Proposal 1: RAN4 is to discuss new triggering of CHO in core requirements and test cases according to further RAN2 conclusions.

# Conclusion

In this paper we provide our initial views on RRM aspects that are related to the discussions within WI scope.

Proposal 1: RAN4 is to discuss new triggering of CHO in core requirements and test cases according to further RAN2 conclusions.

# References

- RP-223540, New WID: Network energy savings for NR, Huawei